| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0267 |
| --- | --- |

| Title: | AHG9/AHG12: On mixed NAL unit types |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He Muhammed Coban Vadim Seregin Adarsh Krishnan Ramasubramonian Marta Karczewicz | Email: | YongHe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The contribution proposes updates to high-level syntax for mixed NAL unit types. It includes:

- NAL unit type and picture type definition update

- Options to handle mixing RASL_NUT with a different NAL unit type

- Reference picture list constraint on IDR picture

- SPS flag to indicate the mixed NAL unit types in the CLVS

# Proposal

The mixed NAL unit types feature enables the region-based random access using sub-pictures. An example use case is viewport dependent streaming of 360° video, basically from a BEAMer scenario. Several syntax elements are used in VVC draft 8 to support mixed NAL unit types as follows:

- sps_idr_rpl_present_flag (SPS)

- mixed_nalu_types_in_pic_flag (PPS)

- no_mixed_nalu_types_in_pic_constraint_flag (PTL)

When mixed_nalu_types_in_pic_flag is equal to 1, a picture contains two types of NAL units, and the combination is shown in Table 1. The picture type of mixed NAL unit types may be either a leading picture or a trailing picture.

Table 1 mixed NAL unit types

|  | IDR_W_RADL | IDR_N_LP | CRA_NUT | RADL_NUT | RASL_NUT | STSA_NUT |
| --- | --- | --- | --- | --- | --- | --- |
| TRAIL_NUT | trailing | trailing | trailing | trailing | trailing | trailing |
| RADL_NUT | leading | leading | leading |  | leading | trailing |
| RASL_NUT | leading | leading | leading | leading |  | trailing |

Figure 1 shows an example of mixed NAL unit types. The IRAP interval of sub-picture #A is 8 and the IRAP interval of subpicture #B and #C IRAP is 4. Subpicture #A is merged with subpicture #B first, and then merged with subpicture #C. A picture with different NAL unit type merged with TRAIL_NUT or STSA_NUT is a trailing picture, and IRAP NAL unit type or RASL_NUT, RADL_NUT merged with RASL_NUT or RADL_NUT is a leading picture.

Figure 1 example of mixed NAL unit types

## NAL unit type and picture unit type

The VCL NAL unit type codes and classes defined in VVC (Table 2) should be modified to accommodate mixed NAL unit types.

Table 2 NAL unit type codes and classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded slice of a trailing picture slice_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture slice_layer_rbsp( ) | VCL |
| 2 | RADL_NUT | Coded slice of a RADL picture slice_layer_rbsp( ) | VCL |
| 3 | RASL_NUT | Coded slice of a RASL picture slice_layer_rbsp( ) | VCL |
| 4..6 | RSV_VCL_4.. RSV_VCL_6 | Reserved non-IRAP VCL NAL unit types | VCL |
| 7 8 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture slice_layer_rbsp( ) | VCL |
| 9 | CRA_NUT | Coded slice of a CRA picture silce_layer_rbsp( ) | VCL |
| 10 | GDR_NUT | Coded slice of a GDR picture slice_layer_rbsp( ) | VCL |
| 11 12 | RSV_IRAP_11 RSV_IRAP_12 | Reserved IRAP VCL NAL unit types | VCL |

The definition of various types of picture is as follows:

clean random access (CRA) picture: An IRAP picture for which each VCL NAL unit has nal_unit_type equal to CRA_NUT.

gradual decoding refresh (GDR) picture: A picture for which each VCL NAL unit has nal_unit_type equal to GDR_NUT.

instantaneous decoding refresh (IDR) picture: An IRAP picture for which each VCL NAL unit has nal_unit_type equal to IDR_W_RADL or IDR_N_LP.

leading picture: A picture that is in the same layer as the associated IRAP picture and precedes the associated IRAP picture in output order.

random access decodable leading (RADL) picture: A coded picture for which each VCL NAL unit has nal_unit_type equal to RADL_NUT.

random access skipped leading (RASL) picture: A coded picture for which each VCL NAL unit has nal_unit_type equal to RASL_NUT.

step-wise temporal sublayer access (STSA) picture: A coded picture for which each VCL NAL unit has nal_unit_type equal to STSA_NUT.

trailing picture: A non-IRAP picture that follows the associated IRAP picture in output order and is not an STSA picture.

All picture types except leading and trailing picture are defined based on the unique NAL unit type. With mixed NAL unit types, each VCL NAL unit type except GDR_NUT and STSA_NUT may be available in one or more of the three basic classes of pictures: IRAP picture, leading picture and trailing picture. Either picture or NAL unit type definition shall be modified to support mixed NAL unit types.

All pictures may be consolidated into 5 picture types: IRAP, GDR, leading, trailing and STSA picture.

The NAL unit type definition is proposed to be updated as highlighted in Table 3.

Table 3 Proposed NAL unit type codes and classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | TRAIL_NUT | Coded slice of a trailing picture slice_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture or a trailing picture slice_layer_rbsp( ) | VCL |
| 2 | RADL_NUT | Coded slice of a RADL leading or a trailing picture slice_layer_rbsp( ) | VCL |
| 3 | RASL_NUT | Coded slice of a RASL leading or a trailing picture slice_layer_rbsp( ) | VCL |
| 4..6 | RSV_VCL_4.. RSV_VCL_6 | Reserved non-IRAP VCL NAL unit types | VCL |
| 7 8 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture, a leading or a trailing picture slice_layer_rbsp( ) | VCL |
| 9 | CRA_NUT | Coded slice of a CRA picture, a leading or a trailing picture silce_layer_rbsp( ) | VCL |
| 10 | GDR_NUT | Coded slice of a GDR picture slice_layer_rbsp( ) | VCL |
| 11 12 | RSV_IRAP_11 RSV_IRAP_12 | Reserved IRAP VCL NAL unit types | VCL |

The editorial name changes of RADL and RASL may be needed as they are not only available in leading pictures, but also in trailing pictures.

## Mixed RASL_NUT with other NAL unit types

RASL picture is defined as a coded picture for which each VCL NAL unit has nal_unit_type equal to RASL_NUT. A RASL picture is a leading picture that may not be decodable when random access is performed from the associated IRAP picture. The constraints on RASL picture are as follows:

- Every picture depends on a RASL picture must also be a RASL picture. RADL and RASL pictures may be mixed in decoding order but RASL pictures must precede RADL pictures in output order

- All RASL pictures are leading pictures of an associated CRA picture. When the associated CRA picture has NoOutputBeforeRecoveryFlag equal to 1, the RASL picture is not output and may not be correctly decodable, as the RASL picture may contain references to pictures that are not present in the bitstream. RASL pictures are not used as reference pictures for the decoding process of non-RASL pictures. When field_seq_flag is equal to 0, all RASL pictures, when present, precede, in decoding order, all non-leading pictures of the same associated CRA picture

With mixed NAL unit types, RASL_NUT may be included in any leading picture or trailing picture, the constraints on RASL picture may be extended to other picture types as well. VVC draft 8 does not specify the picture containing RASL_NUT but not a RASL picture shall be output or not.

RASL_NUT may be available in a leading picture or trailing picture, and the output may be problematic as the RASL slice may not be decoded correctly even though other NAL units may be output.

### Option I

One straightforward option is to prohibit mixing RASL_NUT with a different NAL unit types as it may degrade the viewing experience. The text modification is as follows:

For VCL NAL units of any particular picture, the following applies:

–	If mixed_nalu_types_in_pic_flag is equal to 0, the value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU.

–	Otherwise (mixed_nalu_types_in_pic_flag is equal to 1), the VCL NAL units of one or more subpictures of the picture all have a particular value of nal_unit_type equal to STSA_NUT, RADL_NUT, RASL_NUT, IDR_W_RADL, IDR_N_LP, or CRA_NUT, while the other VCL NAL units in the picture all have a different particular value of nal_unit_type equal to TRAIL_NUT, RADL_NUT, or RASL_NUT.

### Option II

To mixed RASL_NUT with a different NAL unit type in a leading or trailing picture, an indication flag is proposed to be signaled in either PPS or PH to indicate that the associated picture may not be output when the subpicture based bitstream switching occurs as shown in Figure 2.

Figure 2 Picture with RASL subpicture example

Table 4 shows the flag, pps_skipped_subpic_present_flag, in PPS, it indicates one or more subpictures of a picture referring to the PPS may not be correctly decodable due to the random access performed from the associated IRAP subpicture. As a result,

Table 4 Proposed PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| mixed_nalu_types_in_pic_flag | u(1) |
| if( mixed_nalu_types_in_pic_flag ) |  |
| pps_skipped_subpic_present_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| … | u(1) |
| } |  |

pps_skipped_subpic_present_flag equal to 1 specifies that the picture referring to the PPS has one or more VCL NAL units with a nal_unit_type value equal to RASL_NUT. pps_skipped_subpic_present_flag equal to 0 specifies that the picture referring to the PPS has no VCL NAL unit with a nal_unit_type value equal to RASL_NUT.

The general decoding process is updated (highlighted) as follows:

For each picture in the bitstream, the following applies:

- If the picture is the first picture of a layer in the bitstream in decoding order, an IDR picture, or the first picture of a layer that follows an EOS NAL unit of the layer in decoding order, the variable NoOutputBeforeRecoveryFlag for the picture is set equal to 1.

- Otherwise, when the VCL NAL unit type is equal to CRA_NUT, the following applies:

- If some external means not specified in this Specification is available to set the variable HandleCraAsCvsStartFlag[ i ] for the i-th sub-picture to a value, HandleCraAsCvsStartFlag[ i ] for the i-th sub-picture is set equal to the value provided by the external means.

- Otherwise, HandleCraAsCvsStartFlag[ i ] is set equal to 0.

- NoOutputBeforeRecoveryFlag is set to 1 if at least one HandleCraAsCvsStartFlag[ i ], where i ranges from 0 to sps_num_subpics_minus1, is equal to 1. Otherwise, NoOutputBeforeRecoveryFlag is set to 0

PictureOutputFlag is set equal to 0 when the current picture is a RASL picture or a picture referrring to a PPS with the value of pps_skipped_subpic_present_flag equal to 1 and NoOutputBeforeRecoveryFlag of the associated IRAP picture is equal to 1.

For a single-layer bitstream, the following constraint is updated (highlighted)

- Any RASL picture or a leading picture referring to a PPS with pps_skipped_subpic_present_flag equals to 1 associated with a CRA picture shall precede any RADL picture associated with the CRA picture in output order.

For 8.3.2 Decoding process for the reference picture lists construction, the following update is proposed:

It is a requirement of bitstream conformance that the following constraints apply:

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following: [Ed. (YK): Further study for the case of a picture with mixed RASL and RADL NAL unit types.]

  - A RASL picture or a picture containing a RASL_NUT VCL NAL unit

  - A picture that was generated by the decoding process for generating unavailable reference pictures

  - A picture that precedes the associated IRAP picture in decoding order

## Reference picture list construction constraint

There is requirement of bitstream conformance for CRA picture for reference picture list construction as follows:

- When the current picture is a CRA picture, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture in decoding order (when present).

Since IDR picture may carry reference picture list when sps_idr_rpl_present_flag equal to 1, similar constraint may apply to IDR picture as follows:

When the current picture is an IDR picture and sps_idr_rpl_present_flag is equal to 1, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture in decoding order (when present).

## Mixed NAL unit type indication

Current VVC signals mixed_nalu_types_in_pic_flag in PPS, the decoder has to parse PH and trace the referred PPS for every picture to evaluate the mixed type property of current picture. The syntax element no_mixed_nalu_types_in_pic_constraint_flag is signalled in the profile_tier_level( ) syntax strucutre which specifies when no mixed NAL unit types are applied to an output layer set, such general constraint flag is mainly used for subprofile purpose.

For common case when there is no mixed types, a higher level indicator may help to simplify the evaluation process. A SPS level flag is proposed to indicate whether the mixed NAL unit types are present in CLVS referring to the SPS or not. The proposed syntax is shown in Table 5.

Table 5 Proposed SPS mixed NAL unit types syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ mixed_nalu_types_flag | u(1) |
| … | u(1) |
| } |  |

sps_mixed_nalu_types_flag equal to 1 specifies that each picture within a CLVS referring to the SPS has one or more VCL NAL units and when there are more than one VCL NAL unit in the picture, the VCL NAL units may or may not have the same value of nal_unit_type. sps_mixed_nalu_types_flag equal to 0 specifies that each picture within a CLVS referring to the SPS has one or more VCL NAL units and the VCL NAL units of each picture within a CLVS referring to the SPS have the same value of nal_unit_type.

When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of sps_mixed_nalu_types_flag shall be equal to 0.

mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and the VCL NAL units do not have the same value of nal_unit_type. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type.

When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of mixed_nalu_types_in_pic_flag shall be equal to 0.

When sps_mixed_nalu_types_flag is equal to 0, the value of mixed_nalu_types_in_pic_flag shall be equal to 0.

Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).