| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0269-v2 |
| --- | --- |

| Title: | Fix on minimum QT size value range |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, XiangLi, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a fix on the issue reported in ticket #925, to set value range of min QT size in intra slices with consideration of whether dual tree implicit QT split is applied.

# Problem Statement

In current VVC draft, when a minimum luma QT size (MinQTSizeY) is set to 128, and maximum luma multi-type tree depth (maxMTTDepthY) is greater than zero, the maximum luma binary tree node size (MaxBTSizeY) or maximum luma tenary tree node size (MaxTTSizeY) are not allowed to be set to 64, because they have to be larger than or equal to minimum luma QT size (MinQTSizeY).

But due to the fact that when intra dual tree is used, the dual tree implicit splitting is applied to a luma coding block with size of 128. This is effectively resulting the actual minimum luma QT size is up to  64 luma samples. In VVC draft 8, luma BT and TT sizes must be equal to or larger than minimum luma QT size. If minimum luma QT is set to128, luma BT and TT sizes must be 128 even though minimum luma QT size is effectively 64, which is not only redundant but also confusing.

# Proposed Method

It is proposed to constrain the range of syntax elements on min QT size with the consideration of CTU size and whether dual tree is used.

## Value range of luma min QT size for intra slice

As in current VVC draft, dual tree luma block with size of 128 luma samples must be implicitly split into QT node with size of 64. In such case, the semantic of syntax element sps_log2_diff_min_qt_min_cb_intra_slice_luma may be changed as following:

sps_log2_diff_min_qt_min_cb_intra_slice_luma specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_min_qt_min_cb_luma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY − (ctbLog2SizeY > 6 && qtbtt_dual_tree_intra_flag ? 1 : 0), inclusive. The base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU is derived as follows:

- MinQtLog2SizeIntraY = sps_log2_diff_min_qt_min_cb_intra_slice_luma + MinCbLog2SizeY

…

Similarly, the semantic of ph_log2_diff_min_qt_min_cb_intra_slice_luma in picture header shall also be changed as follows:

ph_log2_diff_min_qt_min_cb_intra_slice_luma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in the slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY− (ctbLog2SizeY > 6 && qtbtt_dual_tree_intra_flag ? 1 : 0), inclusive. When not present, the value of ph_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_luma.

…

## Value range of chroma min QT size for intra slice

Dual tree chroma block with size of 128 luma samples must be implicitly split into QT node with size of 64 in luma samples. In such case, the semantic of syntax element sps_log2_diff_min_qt_min_cb_intra_slice_chroma may be changed as following:

sps_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by ph_log2_diff_min_qt_min_cb_chroma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY− (ctbLog2SizeY > 6 ? 1 : 0), inclusive. When not present, the value of sps_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to 0. The base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a CTU with treeType equal to DUAL_TREE_CHROMA is derived as follows:

- MinQtLog2SizeIntraC = sps_log2_diff_min_qt_min_cb_intra_slice_chroma + MinCbLog2SizeY

…

Similarly, the semantic of ph_log2_diff_min_qt_min_cb_intra_slice_chroma in picture header shall also be changed as follows:

ph_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) associated with the PH. The value of ph_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY− (ctbLog2SizeY > 6 ? 1 : 0), inclusive. When not present, the value of ph_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_chroma.

…

# Conclusion

The proposed method provides a fix on the value range of min QT size in intra slices, with the consideration of whether the implicit QT split in intra dual tree is enabled. It is proposed to adopt the proposed changes to VVC.

# Reference

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, ISO/IEC JTC1/SC29/WG11 JVET-Q2001, Jan. 2020.

- https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/925

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).