| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0270 |
| --- | --- |

| Title: | AHG9: On mixing of RASL and RADL NAL unit types |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seunghwan Kim Robert Skupin Yago Sanchez Karsten Suehring | Email: | {dr.hendry,seunghwan3.kim}@lge.com {firstname.lastname}@hhi.fraunhofer.de |
| Source: | LG Electronics and Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution asserted that there are currently still some missing details to ensure that the decision of allowing mix of RASL and RADL NAL unit types that was agreed in the 17th JVET meeting works and does not cause conflicts in the current VVC working draft.

Some updates are proposed to address the asserted missing details.

# Problem description

In the 17th JVET meeting, it was agreed to allow the mix of RASL and RADL NAL unit types within a picture. We assert that there are still some technical details about how to ensure such mix of NAL unit types works without conflicts in the current VVC working draft still missing.

We assert the following may be a non-exhaustive list of missing details that needs to be clarified regarding the mix of RASL and RADL NAL unit types:

- What is the picture type of a picture with a mixed of RASL and RADL NAL unit types? Clearly, such picture cannot be considered as a trailing picture, as pictures with other types of mixed NAL unit types may be considered, because the picture with mixed RASL and RADL NAL unit is still associated with the same IRAP picture before the merging process that produce the mix.

- How should this picture (i.e., the one with mixed RASL and RADL NAL unit types) be treated in decoding process? For example, should it be output when its associated IRAP picture is the first picture in decoding process?

- Is there any effect to conformance test?

- Do current constraints on reference picture lists adequately prohibit abuse of the use of pictures with mixed RASL and RADL NAL unit types?

We propose updates to the current VVC working draft to address the above asserted missing details related to mix of RASL and RADL NAL units.

# Proposal

To address the asserted missing details on the mix of RASL and RADL NAL unit types, we propose the following:

- When a picture contains more than one slice and the slices are a mix of slices with NAL unit types equal to RASL_NUT and RADL_NUT, the picture is considered as a RASL picture and treated as such during decoding process. That means, regardless whether a RASL picture contains slices with nal_unit_type equal to RADL_NUT, RASL picture is not output when the decoding process starts from the associated IRAP picture.

- When the associated IRAP picture of a RASL picture is the first IRAP picture for decoding process, only RASL picture with mixed_nalu_types_in_pic_flag equal to 0 may be omitted from decoding process in conformance testing. That means, RASL picture that contains no slices with nal_unit_type equal to RADL_NUT associated with the first IRAP picture may be removed from the bitstream to be decoded when alternative timing is used in such cases.

- A RADL picture (i.e., a picture that contains one or more slices with nal_unit_type equal to RADL_NUT) may reference a RASL picture with mixed_nalu_types_in_pic_flag equal to 1. When a slice of such a RADL picture refer to a RASL picture as an active reference picture, the following applies:

  - The current subpicture (i.e., the subpicture that contains the slice) and the referenced subpicture of the RASL picture shall have subpic_treated_as_pic_flag[ i ] equal to 1.

  - The referenced subpicture of the RASL picture shall only contain slices with nal_unit_type equal to RADL_NUT.

# Proposed spec text changes

In subclause 3 (Definition)

…

random access skipped leading (RASL) picture: A coded picture for which each VCL NAL unit has nal_unit_type equal to RASL_NUT or a coded picture for which one or more slice has nal_unit_type equal to RASL_NUT and one or more slice has nal_unit_type equal to RADL_NUT.

NOTE – All RASL pictures are leading pictures of an associated CRA picture. When the associated CRA picture has NoOutputBeforeRecoveryFlag equal to 1, the RASL picture is not output and may not be correctly decodable, as the RASL picture may contain references to pictures that are not present in the bitstream. When field_seq_flag is equal to 0, all RASL pictures, when present, precede, in decoding order, all non-leading pictures of the same associated CRA picture.

…

In subclause 8.3.2 (Decoding process for reference picture lists construction)

...

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture with the value of mixed_nalu_types_in_pic_flag equal to 0

  - A picture that was generated by the decoding process for generating unavailable reference pictures

  - A picture that precedes the associated IRAP picture in decoding order

- When the current slice has nal_unit_type equal to RADL_NUT and belongs to subpicture currSubpicA, refpicA is a RASL picture with mixed_nalu_types_in_pic_flag equal to 1, and refpicA is referred to by one active entry in RefPicList[ 0 ] or RefPicList[ 1 ], all of the following applies:

  - currSubpicA and the referenced subpicture in refpicA shall be subpictures with subpic_treated_as_pic_flag[ i ] equal to 1.

  - The referenced subpicture in refpicA shall only contain slices with nal_unit_type equal to RADL_NUT.

…

In subclause C.1 (Hypothetical reference decoder)

...

- When the BP SEI message associated with AU 0 has cpb_alt_timing_info_present_flag equal to 1, either of the following applies for selection of the initial CPB removal delay and delay offset:

–	If NalHrdModeFlag is equal to 1, the default initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and nal_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message are selected. Otherwise, the default initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	If NalHrdModeFlag is equal to 1, the alternative initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and nal_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message and cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and cpb_alt_initial_removal_offset_delta[ Htid ][ ScIdx ], respectively, in the PT SEI message associated with the AU following AU 0 in decoding order are selected. Otherwise, the alternative initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message and cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and cpb_alt_initial_removal_offset_delta[ Htid ][ ScIdx ], respectively, in the PT SEI message associated with the AU following AU 0 in decoding order are selected. The variable DefaultInitCpbParamsFlag is set equal to 0, and one of the following applies:

–	The RASL AUs pictures with mixed_nalu_types_in_pic_flag equal to 0 associated with AU 0 are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

–	All AUs following AU 0 in decoding order up to an AU associated with a DRAP indication SEI message are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

Each conformance test consists of a combination of one option in each of the above steps. When there is more than one option for a step, for any particular conformance test only one option is chosen. All possible combinations of all the steps form the entire set of conformance tests. For each operation point under test, the number of bitstream conformance tests to be performed is equal to n0 * n1 * ( 2 * n2 + n3 + n4 + n5), where the values of n0, n1, n2, n3, n4, and n5 are specified as follows:

–	n0 is set equal to 2.

–	n1 is equal to hrd_cpb_cnt_minus1 + 1.

–	n2 is the number of AUs in BitstreamToDecode that each is associated with a BP SEI message applicable to TargetOlsIdx and for which all of the following conditions are true:

–	nal_unit_type is equal to CRA_NUT.

–	The associated BP SEI message has bp_alt_cpb_params_present_flag equal to 1.

–	There is at least one RASL picture with mixed_nalu_types_in_pic_flag equal to 0 asscociated to the AU.

–	n3 is the number of IRAP AUs without associated RASL pictures in BitstreamToDecode that each is associated with a BP SEI message applicable to TargetOlsIdx.

–	n4 is the number of AUs in BitstreamToDecode that each is associated with a DRAP indication SEI message applicable to TargetOlsIdx and for each of which the associated PT SEI message has cpb_alt_timing_info_present_flag equal to 1.

–	n5 is derived as follows:

–	If general_decoding_unit_hrd_params_present_flag in the selected general_hrd_parameters( ) syntax structure is equal to 0, n5 is equal to 1.

–	Otherwise, n5 is equal to 2.

# Patent rights declaration(s)

LG Electronics and Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).