| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0273 |
| --- | --- |

| Title: | AHG9: APS signalled in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Muhammed Coban Yong He Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to add a mode of APS signalling inside a PH NAL in a similar manner as picture header signalled inside a VCL NAL. This new mode is indicated by a flag in a picture header followed by the number of APSs included into a PH NAL.

# Proposal

In VVC draft 8, there is mode when PH structure can be signalled inside a VCL NAL for the purpose of saving NAL unit header overhead and byte alignment at the end of a NAL.

For the same purpose and benefit, it is proposed to signal APSs inside a PH NAL. This new mode of APS signalling is indicated by a flag in PH followed by a syntax element indicating the number of included APSs.

The changes relative to VVC draft 8 are highlighted in yellow below.

#### 7.3.2.5 Adaptation parameter set RBSP syntax

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_structure( ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| adaptation_parameter_set_structure( ) adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| picture_header_structure( ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.2.7 Picture header structure syntax

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| aps_in_picture_header_flag | u(1) |
| if( aps_in_picture_header_flag ) { |  |
| ph_num_aps_minus1 | ue(v) |
| for( i = 0; i <= ph_num_aps_minus1; i++ ) |  |
| adaptation_parameter_set_structure( ) |  |
| } |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| non_reference_picture_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ph_pic_order_cnt_lsb | u(v) |
| if( gdr_or_irap_pic_flag ) |  |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| … |  |

aps_in_picture_header_flag equal to 1 specifies that the APS syntax structure is present in the picture header. aps_in_picture_header_flag equal to 0 specifies that the APS syntax structure is not present in the picture header.

ph_num_aps_minus1 specifies the number of APS structures minus 1 present in the picture header. The value of ph_num_aps_minus1 shall be in the range of 0 to 19, inclusive.

The proposed APS signalling was implemented in VTM-8.0 and tested under common test condition. In the current CTC, APS is not signalled frequently, however if it is signalled per picture then the bit saving should be comparable to the results reported in JVET-Q0255 and may be even better if more than one APS is included into PH NAL and signalled per picture.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | -0.01% | 99% | 100% |
| Class B | -0.02% | -0.02% | -0.02% | 100% | 102% |
| Class C | -0.05% | -0.05% | -0.05% | 101% | 101% |
| Class E | -0.06% | -0.06% | -0.06% | 100% | 102% |
| Overall | -0.03% | -0.03% | -0.03% | 100% | 101% |
| Class D | -0.13% | -0.12% | -0.13% | 100% | 100% |
| Class F | -0.04% | -0.04% | -0.04% | 101% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Class A2 | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Class B | -0.03% | -0.04% | -0.04% | 100% | 101% |
| Class C | -0.04% | -0.04% | -0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.03% | -0.03% | 100% | 100% |
| Class D | -0.09% | -0.09% | -0.09% | 99% | 100% |
| Class F | -0.04% | -0.04% | -0.04% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.05% | -0.05% | 100% | 101% |
| Class C | -0.05% | -0.05% | -0.05% | 100% | 101% |
| Class E | -0.08% | -0.08% | -0.08% | 99% | 98% |
| Overall | -0.05% | -0.06% | -0.06% | 99% | 100% |
| Class D | -0.05% | -0.05% | -0.05% | 99% | 101% |
| Class F | -0.13% | -0.13% | -0.13% | 99% | 100% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).