| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0275 |
| --- | --- |

| Title: | AHG8: On PTL, HRD, and DPB structures signalling in VPS and SPS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Muhammed Coban Yong He Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes fixing a problem of referencing not signaled dpb_parameters() or ols_hrd_parameters() structures, when more than one independent layer is included into OLS. It is proposed to:

- Condition the signaling of the number of DPB parameters vps_num_dpb_params based on !each_layer_is_an_ols_flag instead of !vps_all_independent_layers_flag.

- Add a constraint that vps_general_hrd_params_present_flag shall be equal to 1 when more than one layer is included into any OLS.

- Add a constraint that sps_ptl_dpb_hrd_params_present_flag shall be equal to 1 when there is at least one OLS which has only one layer or VPS is not present.

- For design consistency, it is proposed to unify dpb_parameters() and ols_hrd_parameters() signalling using the number of parameters minus 1 with a gating flag.

- Infer the value of ols_ptl_idx[ i ] or ols_dpb_params_idx[ i ] when the number of PTL or DPB structure is equal to the number of OLSs

# Problem Statement

In VVC draft 8, there are two distinct cases when OLS contains more than one layer or only one layer. In the first case, the DPB and HRD structures are signalled and accessed from VPS, while in the second case DPB and HRD strucutres are signalled and accessed from SPS.

For example, in VPS semantics:

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

ols_hrd_idx[ i ] specifies the index, to the list of ols_hrd_parameters( ) syntax structures in the VPS, of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

If the value of num_ols_hrd_param_minus1 + 1 is equal to TotalNumOlss, the value of ols_hrd_idx[ i ] is inferred to be equal to i. Otherwise, when NumLayersInOls[ i ] is greater than 1 and num_ols_hrd_params_minus1 is equal to 0, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0.

Signalling of the number of dpb_parameters() is conditioned based on whether all layers are not independent. When not present, the value of vps_num_dpb_params is inferred to be equal to 0.

| if( !vps_all_independent_layers_flag ) |  |  |
| --- | --- | --- |
| vps_num_dpb_params | ue(v) |

If more than one independent layer is included into OLS and all layers are independent, the number of DPB parameters is not signaled and is inferred to be 0. As a result, no dpb_parameters() structure is signaled in VPS, while later those parameters are accessed by ols_dpb_params_idx.

Similar problem exists for ols_hrd_parameters() signalling, as there is no requirement to signal vps_general_hrd_params_present_flag being equal to 1 when more then one layer is included into any OLS in VPS.

In general, dpb_parameters() and ols_hrd_parameters() parameters should be signalled under the similar conditions and using the same syntax structure, which is not the case in the current VVC draft.

# Proposal

To solve the problem of referencing of the not signalled dpb_parameters() mentioned earlier, it is proposed to signal vps_num_dpb_params being condition on !each_layer_is_an_ols_flag instead of !vps_all_independent_layers_flag.

To solve the problem of referencing of the not signalled ols_hrd_parameters(), it is proposed to add a constraint that the value of vps_general_hrd_params_present_flag shall be equal to 1 when more than one layer is included into any OLS.

To solve the problem of referencing of the not signalled dpb_parameters() or ols_hrd_parameters() in SPS, it is proposed to add a constraint that the value of sps_ptl_dpb_hrd_params_present_flag shall be equal to 1 when there is at least one OLS which contains only one layer or VPS is not present, i.e. sps_video_parameter_set_id is equal to 0.

For design consistency, it is proposed to unify dpb_parameters() and ols_hrd_parameters() signalling using the number of parameters minus 1 with a gating flag.

The proposed changes are reflected in the specification modification below marked in yellow.

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) { |  |  |
| for( j = 0; j < i; j++ ) |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| max_tid_ref_present_flag[ i ] | u(1) |  |
| if( max_tid_ref_present_flag[ i ] ) |  |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| each_layer_is_an_ols_flag | u(1) |  |
| if( !each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| ols_mode_idc | u(2) |  |
| if( ols_mode_idc = = 2 ) { |  |  |
| num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_dpb_hrd_params_present_flag | u(1) |  |
| if( vps_dpb_hrd_params_present_flag ) { |  |  |
| vps_num_dpb_params_minus1 | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i <= vps_num_dpb_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( vps_num_dpb_params_minus1 > 01 ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_dpb_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

vps_num_dpb_params_minus1 specifies the number minus 1of dpb_parameters( ) syntax strutcures in the VPS. The value of vps_num_dpb_params_minus1 shall be in the range of 0 to 156, inclusive. When not present, the value of vps_num_dpb_params is inferred to be equal to 0.

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params_minus1 − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

vps_general_dpb_hrd_params_present_flag equal to 1 specifies that the syntax structures dpb_parameters(), general_hrd_parameters( ) and other HRD parameters are present in the VPS RBSP syntax structure. vps_general_dpb_hrd_params_present_flag equal to 0 specifies that the syntax structures dpb_parameters(), general_hrd_parameters( ) and other HRD parameters are not present in the VPS RBSP syntax structure. When not present, the value of vps_general_dpb_hrd_params_present_flag is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters() and general_hrd_parameters( ) syntax structures that applyies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

vps_dpb_hrd_params_present_flag shall be equal to 1 if more than one layer is included into any OLS in VPS.

In SPS semantics, the inference to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is removed as it is not sufficient since in case of more than one layer in OLS dpb_parameters() and ols_hrd_parameters() are derived from VPS and shall be signaled there.

sps_ptl_dpb_hrd_params_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS, and a general_hrd_parameters( ) syntax structure and an ols_hrd_parameters( ) syntax structure may also be present in the SPS. sps_ptl_dpb_hrd_params_present_flag equal to 0 specifies that none of these four syntax structures is present in the SPS. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to 1 when sps_video_parameter_set_id is equal to 0 or only one layer is included into any OLS of the referred VPS. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ].

# PTL

PTL, DPB, and HRD structures are signaled under one common gating flag in SPS, while PTL is always signaled in VPS (vps_num_ptls_minus1). It is understood that PTL signalled in VPS can be used for session negotiation, however there is no currently a mechanism to disable PTL signaling in VPS even for the case having only a single layer in an OLS.

It is proposed to include PTL under the common gating flag along with DPB and HRD inVPS, the proposed changes are reflected in the specification modification below marked in yellow.

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) { |  |  |
| for( j = 0; j < i; j++ ) |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| max_tid_ref_present_flag[ i ] | u(1) |  |
| if( max_tid_ref_present_flag[ i ] ) |  |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| each_layer_is_an_ols_flag | u(1) |  |
| if( !each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| ols_mode_idc | u(2) |  |
| if( ols_mode_idc = = 2 ) { |  |  |
| num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_ptl_dpb_hrd_params_present_flag | u(1) |  |
| if( vps_ptl_dpb_hrd_params_present_flag ) { |  |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params_minus1 | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i <= vps_num_dpb_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( vps_num_dpb_params_minus1 > 01 ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

vps_num_dpb_params_minus1 specifies the number minus 1of dpb_parameters( ) syntax strutcures in the VPS. The value of vps_num_dpb_params shall be in the range of 0 to 156, inclusive. When not present, the value of vps_num_dpb_params is inferred to be equal to 0.

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params_minus1 − 1, inclusive. When ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the dpb_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

vps_general_ptl_dpb_hrd_params_present_flag equal to 1 specifies that the syntax structures profile_tier_level( ) dpb_parameters( ), general_hrd_parameters( ) and other HRD parameters are present in the VPS RBSP syntax structure. vps_general_ptl_dpb_hrd_params_present_flag equal to 0 specifies that these syntax structures general_hrd_parameters( ) and other HRD parameters are not present in the VPS RBSP syntax structure. When not present, the value of vps_general_ptl_dpb_hrd_params_present_flag is inferred to be equal to 0.

When NumLayersInOls[ i ] is equal to 1, the general_hrd_parameters( ) and dpb_parameters( )syntax structures that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

vps_ptl_dpb_hrd_params_present_flag shall be equal to 1 if more than one layer is included into any OLS in VPS.

In SPS semantics, the inference to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is removed as it is not sufficient since in case of more than one layer in OLS dpb_parameters() and ols_hrd_parameters() are derived from VPS and shall be signaled there.

sps_ptl_dpb_hrd_params_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS, and a general_hrd_parameters( ) syntax structure and an ols_hrd_parameters( ) syntax structure may also be present in the SPS. sps_ptl_dpb_hrd_params_present_flag equal to 0 specifies that none of these four syntax structures is present in the SPS. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to 1 when sps_video_parameter_set_id is equal to 0 or only one layer is included into any OLS of the referred VPS. The value of sps_ptl_dpb_hrd_params_present_flag shall be equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ].

# Inference of ols_ptl_idx[ i ] and ols_dpb_params_idx[ i ]

In VVC draft 8, ols_ptl_idx[ i ] is explicitly signaling in VPS as an index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. ols_dpb_params_idx[ i ] is also explicitly signaling in VPS as an index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1.

When the number of PTL structures is equal to the total number of OLSs, the value of ols_ptl_idx[ i ] may be inferred to be equal to i.

When the number of DPB parameters structures is equal to the total number of OLSs, the value of ols_dpb_params_idx[ i ] may be inferred to be equal to i.

The proposed conditions on signaling syntax elements, ols_ptl_idx[ i ] and ols_dpb_params_idx[ i ], and the semantics update are highlighted in below.

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 > 0 && vps_num_ptls_minus1 + 1 != TotalNumOlss ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( vps_num_dpb_params > 1 && vps_num_dpb_params != TotalNumOlss ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls_minus1, inclusive. If the value of vps_num_ptls_minus1 + 1 is equal to TotalNumOlss, the value of ols_ptl_idx[ i ] is inferred to be equal to i. Otherwise, when vps_num_ptls_minus1 is equal to 0, the value of ols_ptl_idx[ i ] is inferred to be equal to 0.

ols_dpb_params_idx[ i ] specifies the index, to the list of dpb_parameters( ) syntax structures in the VPS, of the dpb_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_dpb_params_idx[ i ] shall be in the range of 0 to vps_num_dpb_params − 1, inclusive. If the value of vps_num_dpb_params is equal to TotalNumOlss, the value of ols_dpb_params_idx[ i ] is inferred to be equal to i. Otherwise, when ols_dpb_params_idx[ i ] is not present, the value of ols_dpb_params_idx[ i ] is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).