| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0276 |
| --- | --- |

| Title: | AHG9: On IRAP NAL constraint for reordered sub-pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Yong He Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to modify the constraint to require IRAP NAL if a sub-picture in the previously decoded picture has a different ID from the current sub-picture ID, instead it is proposed to disallow using such pictures as active reference pictures.

# Proposal

In VVC draft 8, sub-picture can be reordered, i.e. a sub-picture ID in the current picture is different from the corresponding sub-picture in the previously decoded picture. In this case such sub-picture requires to have an IRAP NAL.

When the current picture is not the first picture of the CLVS, for each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, if the value of SubpicIdVal[ i ] is not equal to the value of SubpicIdVal[ i ] of the previous picture in decoding order in the same layer, the nal_unit_type for all coded slice NAL units of the subpicture in the current picture with subpicture index i shall be equal to a particular value in the range of IDR_W_RADL to CRA_NUT, inclusive.

The problem in the current constraint is that IRAP slices are required even if the previously decoded picture has reordered sub-pictures but is not used as a reference picture, i.e. current sub-picture cannot use inter prediction.

It is proposed to modify the constraint as if the corresponding sub-picture in a reference picture is reordered, i.e. has different sub-picture ID, such reference picture shall not be used for predicting the slices of the corresponding sub-picture in the current picture.

For each value of i in the range of 0 to sps_num_subpics_minus1, inclusive, if the value of SubpicIdVal[ i ] is not equal to the value of SubpicIdVal[ i ] of a reference picture, such reference picture shall not be used for predicting the slice NAL units of the subpicture in the current picture with subpicture index i.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).