| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0277 |
| --- | --- |

| Title: | AHG9: On reference picture list signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Muhammed Coban Yong He Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to modify reference picture list signalling as follows:

- Remove reference index override flag signalling for I-slices

- When not present, infer slice_collocated_from_l0_flag to be equal to 1 for P-slices

- Repurpose rpl1_idx_present_flag to indicate the presence RefPicList1 related syntaxe elements and not only the presence of rpl_sps_flag[1] and rpl_idx[1].

- Add a constraint for reference pictures to be the same for all slices in a picture

# RPL override flag in SH

In the VVC draft 8, num_ref_idx_active_override_flag is signaled for I-slices when it is not needed.

It is proposed to not signal override flag for I-slices, since RPL is not needed for I-slices decoding and there is no reference index signaling in I-slices, so there is no extra overhead issue.

| slice_header( ) { | Descriptor |
| --- | --- |
| … | u(2) |
| if( !rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| ref_pic_lists( ) |  |
| if( ( rpl_info_in_ph_flag ¦ ¦ ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) && ( slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| … |  |
| } |  |

# Inference of slice_collocated_from_l0_flag

In the VVC draft 8, a reference picture list from which collocated picture is derived can be signalled in PH using ph_collocated_from_l0_flag. There is a problem when this flag ph_collocated_from_l0_flag is equal to 0, i.e. collocated picture is derived from RefPicList1 and there are P and B slices present in the same picture. In such case, collocated picture cannot be derived for a P-slice since there may be no RefPicList1 for P slices.

It is proposed to infer the value of slice_collocated_from_l0_flag to be equal to 1 for P-slices regardless of the ph_collocated_from_l0_flag value.

slice_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. slice_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

When slice_type is equal to B or P, ph_temporal_mvp_enabled_flag is equal to 1, and slice_collocated_from_l0_flag is not present, the following applies:

- If slice_type is equal to P, the value of slice_collocated_from_l0_flag is inferred to be equal to 1.

- If Otherwise, if rpl_info_in_ph_flag is equal to 1, slice_collocated_from_l0_flag is inferred to be equal to ph_collocated_from_l0_flag.

- Otherwise (rpl_info_in_ph_flag is equal to 0 and slice_type is equal to P), the value of slice_collocated_from_l0_flag is inferred to be equal to 1.

# Repurpose rpl1_idx_present_flag

In the VVC draft 8, there is a flag rpl1_idx_present_flag in PPS to indicate the presence of rpl_sps_flag[1] and rpl_idx[1]. This flag does not control any other RefPicList1 related syntax elements.

rpl1_idx_present_flag equal to 0 specifies that rpl_sps_flag[ 1 ] and rpl_idx[ 1 ] are not present in the PH syntax structures or the slice headers for pictures referring to the PPS. rpl1_idx_present_flag equal to 1 specifies that rpl_sps_flag[ 1 ] and rpl_idx[ 1 ] may be present in the PH syntax structures or the slice headers for pictures referring to the PPS.

In this contribution, it is proposed to extend this flag to control all RefPicList1 related syntax elements as follows.

rpl1_present_flag equal to 0 specifies that RefPicList1 rellaed syntax elements are not present in the PH syntax structures or the slice headers for pictures referring to the PPS. rpl1_present_flag equal to 1 specifies that RefPicList1 related syntax elements may be present in the PH syntax structures or the slice headers for pictures referring to the PPS.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| ph_temporal_mvp_enabled_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| if( rpl1_present_flag ) |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| if( rpl1_present_flag ) |  |
| mvd_l1_zero_flag | u(1) |
| … |  |
| } |  |

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When not present, ph_collocated_from_l0_flag is inferred be equal ot 1.

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( wp_info_in_ph_flag ) |  |
| num_l0_weights | ue(v) |
| for( i = 0; i < NumWeightsL0; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL0; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i < NumWeightsL0; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| if( pps_weighted_bipred_flag && wp_info_in_ph_flag && rpl1_present_flag ) |  |
| num_l1_weights | ue(v) |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i < NumWeightsL1; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

num_l1_weights specifies the number of weights signalled for entries in reference picture list 1 when pps_weighted_bipred_flag and wp_info_in_ph_flag are both equal to 1. The value of num_l1_weights shall be in the range of 0 to num_ref_entries[ 1 ][ RplsIdx[ 1 ] ], inclusive.

The variable NumWeightsL1 is derived as follows:

if( !pps_weighted_bipred_flag  | |  !rpl1_present_flag )
	NumWeightsL1 = 0
else if( wp_info_in_ph_flag )		(148)
	NumWeightsL1 = num_l1_weights
else
	NumWeightsL1 = NumRefIdxActive[ 1 ]

| ref_pic_lists( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 2 rpl1_present_flag ? 2 : 1; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| rpl_sps_flag[ i ] | u(1) |
| if( rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

The condition checks in ref_pic_list signaling are simplified.

Slice type is constrained in a way that when rpl1_present_flag is equal to 0, a slice may have only I-slice or P-slice types.

slice_type specifies the coding type of the slice according to Table 9.

Table 9 – Name association to slice_type

| slice_type | Name of slice_type |
| --- | --- |
| 0 | B (B slice) |
| 1 | P (P slice) |
| 2 | I (I slice) |

When not present, the value of slice_type is inferred to be equal to 2.

When ph_intra_slice_allowed_flag is equal to 0, the value of slice_type shall be equal to 0 or 1. When nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, slice_type shall be equal to 2.

When rpl1_present_flag is equal to 0 the value of slice_type shall be equal to 1 or 2.

# Constraint on reference picture lists of a picture

In HEVC specification, when multiple slices are used in a picture, reference picture set signalled by all slices in the picture shall be the same. In VVC specification, it is not clear whether such a constraint exists for RefPicList[ 0 ] and RefPicList[ 1 ] signalled or referred to by multiples slices in a picture.

It is proposed to add a constraint in the specification to clarify the intended behavior. Proposed clarification is to add a constraint:

Set of reference pictures consisting of RefPicList[ 0 ] and RefPicList[ 1 ] entries shall be the same for all VCL NALs of a picture.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).