| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0278 |
| --- | --- |

| Title: | AHG8: On SPS sharing and slice type constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes enabling SPS sharing between independent and dependent layers. It allows to avoid signalling of SPS different only in enabled inter-layer prediction and to save SPS ids as the total number of SPSs can be up to 16.

It is proposed to remove a constraint that inter-layer prediction flag shall be equal to 0 for independent layers and add inter-layer reference pictures only for dependent layers during RPL derivation proses.

In the second part of the contribution, it is proposed to add a constraint for ph_inter_slice_allowed_flag to be equal to 0 for IDR and CRA pictures of independent layers.

# Problem Statement

In VVC draft 8, there is constraint for inter-layer prediction flag inter_layer_ref_pics_present_flag that when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pics_present_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.

It prohibits the SPS sharing between independent and dependent layers. For example, if there are two layers, the first layer is independent and the second layer is dependent then two SPSs, one for each layer, shall be signaled even the difference is only in the value of inter_layer_ref_pics_present_flag.

There is another constraint on slice type for IDR and CRA NAL units of independent layers as follows.

When nal_unit_type is in the range of IDR_W_RADL to CRA_NUT, inclusive, and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, slice_type shall be equal to 2.

However, there is no similar constraint for ph_inter_slice_allowed_flag for independent layers.

# SPS sharing proposal

It is proposed to allow SPS sharing between dependent and independent layers by removing the above mentioned constraint for inter_layer_ref_pics_present_flag and add inter-layer reference pictures only for dependent layers during RPL derivation. The changes relative to VVC draft 8 are highlighted in yellow.

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pics_present_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ], the reference picture scaling ratios RefPicScale[ i ][ j ][ 0 ] and RefPicScale[ i ][ j ][ 1 ], and the reference picture scaled flags RprConstraintsActive[ 0 ][ j ] and RprConstraintsActive[ 1 ][ j ] are derived as follows:

for( i = 0; i < 2; i++ ) {
	for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
		if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
			if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				RefPicPocList[ i ][ j ] = pocBase − DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ]
				if( there is a reference picture picA in the DPB with the same nuh_layer_id as the current picture
							and PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"	(203)
				pocBase = RefPicPocList[ i ][ j ]
			} else {
				if( !delta_poc_msb_cycle_lt[ i ][ k ] ) {
					if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
									PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) equal to PocLsbLt[ i ][ k ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
					RefPicLtPocList[ i ][ j ] = PocLsbLt[ i ][ k ]
				} else {
					if( there is a reference picA in the DPB with the same nuh_layer_id as the current picture and
								PicOrderCntVal equal to 	FullPocLt[ i ][ k ] )
						RefPicList[ i ][ j ] = picA
					else
						RefPicList[ i ][ j ] = "no reference picture"
					RefPicLtPocList[ i ][ j ] = FullPocLt[ i ][ k ]
				}
				k++
			}
		} else if( !vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] ) {
			layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ]
			refPicLayerId = vps_layer_id[ layerIdx ]
			if( there is a reference picture picA in the DPB with nuh_layer_id equal to refPicLayerId and
						the same PicOrderCntVal as the current picture )
				RefPicList[ i ][ j ] = picA
			else
				RefPicList[ i ][ j ] = "no reference picture"
		} 
		fRefWidth is set equal to PicOutputWidthL of the reference picture RefPicList[ i ][ j ]
		fRefHeight is set equal to PicOutputHeightL of the reference picture RefPicList[ i ][ j ]

		refPicWidth, refPicHeight, refScalingWinLeftOffset, refScalingWinRightOffset, refScalingWinTopOffset,
			and refScalingWinBottomOffset, are set equal to the values of pic_width_in_luma_samples,
			pic_height_in_luma_samples, scaling_win_left_offset, scaling_win_right_offset,
			scaling_win_top_offset, and scaling_win_bottom_offset, respectively, of the reference picture
			RefPicList[ i ][ j ]

		RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL  >>  1 ) ) / PicOutputWidthL
		RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL  >>  1 ) ) / PicOutputHeightL
		RprConstraintsActive[ i ][ j ] = (	 pic_width_in_luma_samples  !=  refPicWidth  | |
				pic_height_in_luma_samples  !=  refPicHeight  | |
				scaling_win_left_offset  !=  refScalingWinLeftOffset  | |
				scaling_win_right_offset  !=  refScalingWinRightOffset  | |
				scaling_win_top_offset  !=  refScalingWinTopOffset  | |
				scaling_win_bottom_offset  !=  refScalingWinBottomOffset )
	}
}

Additional argument interLayerRefPicsPresentFlag is added to ref_pic_list_struct to indicate whether the reference picture list structure contains inter-layer reference pictures.

| ref_pic_list_struct( listIdx, rplsIdx, interLayerRefPicsPresentFlag ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( long_term_ref_pics_flag ) |  |
| ltrp_in_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( interLayerRefPicsPresentFlag inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| ilrp_idx[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

When ref_pic_list_struct is signaled in SPS, the argument is set equal to inter_layer_ref_pics_present_flag.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(4) |
| inter_layer_ref_pics_present_flag | u(1) |
| sps_idr_rpl_present_flag | u(1) |
| rpl1_same_as_rpl0_flag | u(1) |
| for( i = 0; i < !rpl1_same_as_rpl0_flag ? 2 : 1; i++ ) { |  |
| num_ref_pic_lists_in_sps[ i ] | ue(v) |
| for( j = 0; j < num_ref_pic_lists_in_sps[ i ]; j++) |  |
| ref_pic_list_struct( i, j, inter_layer_ref_pics_present_flag ) |  |
| } |  |
| … |  |
| } |  |

When reference picture list ref_pic_lists is signaled in a picture or a slice header, the added argument interLayerRefPicsPresentFlag is set equal to vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ], as shown in below.

| ref_pic_lists( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| rpl_sps_flag[ i ] | u(1) |
| if( rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ], vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| poc_lsb_lt[ i ][ j ] | u(v) |
| delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( delta_poc_msb_present_flag[ i ][ j ] ) |  |
| delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

# Constraint for ph_inter_slice_allowed_flag

It is proposed to add a constraint for IDR and CRA pictures that ph_inter_slice_allowed_flag shall be equal to 0 for independent layers in a similar way it is done for slice type.

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1.

When gdr_or_irap_pic_flag is equal to 1 and gdr_pic_flag is equal to 0 and vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, ph_inter_slice_allowed_flag shall be equal to 0.

As a second option, it also can be expressed as a condition for ph_inter_slice_allowed_flag signalling with added inference that ph_inter_slice_allowed_flag is inferred to be equal to 0 when not present.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| if( gdr_or_irap_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| if( !gdr_or_irap_pic_flag ¦ ¦ gdr_pic_flag ¦ ¦ !vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] ) |  |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| … |  |

ph_inter_slice_allowed_flag equal to 0 specifies that all coded slices of the picture have slice_type equal to 2. ph_inter_slice_allowed_flag equal to 1 specifies that there may or may not be one or more coded slices in the picture that have slice_type equal to 0 or 1. When ph_inter_slice_allowed_flag is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).