| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0279 |
| --- | --- |

| Title: | AHG9: On decoupling luma deblocking parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra1, Frank Bossen2, Jonatan Samuelsson1, Sachin Deshpande1, A. Segall1 | Email: | misrak@sharplabs.com |
| Source: | 1Sharp Labs of America, Inc., 2Sharp Electronics of Canada |  |  |

_____________________________

# Abstract

In the current VVC draft, the deblocking activation and clipping threshold values (beta and tc) are common for the luma long filters and luma strong filters. This contribution proposes that the threshold values for each type of filtering be separate. This is achieved by signaling separate beta offset and tc offset for luma long filters and luma strong filters. The proposed modification enables the decoupling of activation and clipping control thresholds for the two types of filtering. The proposed change is asserted to be a desirable improvement over the signalling in the current VVC draft.

# Introduction

In the current VVC draft [1], the deblocking activation (β) and clipping threshold (tC) values are derived based on:

Q = Clip3( 0, 63, qP + ( slice_beta_offset_div2  <<  1 ) ) 	(1294)

and

Q = Clip3( 0, 65, qP + 2 * ( bS – 1 ) + ( slice_tc_offset_div2  <<  1 ) ) 	(1296)

The quantization parameters Q are used as index for lookup into a table of threshold values. In the current draft, these threshold values are common for luma long filters and luma strong filter decoding processes. This creates an unnecessary coupling in the activation and clipping processes of the two types of filtering. As a result of the coupling, one cannot adjust the threshold to decrease luma long filtering application while maintaining the application for luma strong filters, or vice versa, such as may be desired in regions with texture.

The contribution proposes that the existing offsets in VVC draft be used for luma strong filters only (as in HEVC) and an additional set of offsets be signalled for luma long filters. As a result of the proposed change, the luma long filters deblocking activation (β) and clipping threshold (tC) values are derived based on:

Q = Clip3( 0, 63, qP + ( slice_long_filter_beta_offset_div2  <<  1 ) )	(XXXX)
and

Q = Clip3( 0, 65, qP + 2 * ( bS − 1 ) + ( slice_long_filter_tc_offset_div2  <<  1 ) )	)   (XXXX)

The proposed modification enables decoupling of activation and clipping control thresholds for the two types of filtering and is asserted to be a desirable improvement over the signalling in the current VVC draft.

# Proposed syntax

The syntax modifications for the proposed change are listed below and highlighted in yellow. Other details of modifications to the VVC Draft can be found in attached specification document.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_long_filter_beta_offset_div2 | se(v) |
| pps_long_filter_tc_offset_div2 | se(v) |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |
| … |  |
| } |  |

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_beta_offset_div2 | se(v) |
| ph_tc_offset_div2 | se(v) |
| ph_long_filter_beta_offset_div2 | se(v) |
| ph_long_filter_tc_offset_div2 | se(v) |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && !dbf_info_in_ph_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| slice_long_filter_beta_offset_div2 | se(v) |
| slice_long_filter_tc_offset_div2 | se(v) |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

# Recommendation

It is recommended to adopt the proposed changes in the VVC Draft.

# References

[1] Benjamin Bross, Jianle Chen, Shan Liu, Ye-Kui Wang, “Versatile Video Coding (Draft 8),” JVET-Q2001, 17th Meeting of Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Brussels, BE, 7–17 January 2020

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).