| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0280-v1 |
| --- | --- |

| Title: | AHG16: Cleanup of intra reference sample filter selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Heo Hyeongmun Jang Jangwon Choi Junghak Nam Moonmo Koo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {jin78.heo, hm.jang, jangwon84.choi, junghak.nam moonmo.koo jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In current VVC draft, the intra mode checking process is used to select intra reference sample filters. If the current intra mode is an angular mode with integer slope, a 3-tap [1 2 1]/4 reference sample filter is applied. Since the 4-tap [16 32 16 0]/64 smoothing filter for integer sample position is the same as the 3-tap reference sample filter for an angular mode with integer slop, the 3-tap reference sample filter used in the reference sample generation process can be replaced by the 4-tap smoothing filter used in the interpolation process. Therefore, this contribution proposes to remove the intra mode checking process, which process is redundant, and directly determine intra reference sample filters. The experimental results show that the proposed method provides the coding performance changes of 0.00%, 0.00%, and -0.05% BD-rate in AI, RA, and LD configuration, respectively. Encoding and decoding run-times are not changed.

Proposed method

In VTM-8.0, intra reference sample filters are selected based on the intra mode checking process which indicates whether it is the intra prediction mode with integer slope (one of the {-14, -12, -10, -6, 2, 34, 66, 72, 76, 78, 80}). If the current intra prediction mode is an angular mode with integer slope, a 3-tap [1 2 1]/4 filter is applied to the reference sample and the filtered reference samples are directly used as the predicted samples.

As shown in Table 1, the 4-tap smoothing filter coefficient for integer sample position is [16 32 16 0]/64 which means that it is the same as filtering the reference samples by [1 2 1]/4 filter. Therefore the proposed method employs the 4-tap smoothing filter to generate the predicted samples from the unfiltered reference sample for the intra prediction mode with integer slope. In VTM-8.0, if the intra mode checking process is removed, the reference sample filtering process is switched off and the 4-tap smoothing filter is directly selected as the interpolation filter for the intra prediction mode with integer slope.

Table 1. Specification of interpolation filter coefficients fG

|  | Smoothing filter coefficients (fG) |  |  |  |
| --- | --- | --- | --- | --- |
|  | fG[ p ][ 0 ] | fG[ p ][ 1 ] | fG[ p ][ 2 ] | fG[ p ][ 3 ] |
| Integer sample position P | 16 | 32 | 16 | 0 |

Working Draft Text

The changes in the VVC working draft [2] is highlight below.

- 8.4.5.2.5  General intra sample prediction

Inputs to this process are:

- a sample location ( xTbCmp, yTbCmp ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture,

- a variable predModeIntra specifying the intra prediction mode,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable cIdx specifying the colour component of the current block.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variables refW and refH are derived as follows:

- If IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT or cIdx is not equal to 0, the following applies:

refW = nTbW * 2		(306)

refH = nTbH * 2		(307)

- Otherwise ( IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and cIdx is equal to 0 ), the following applies:

refW = nCbW + nTbW		(308)

refH = nCbH + nTbH		(309)

The variable refIdx specifying the intra prediction reference line index is derived as follows:

refIdx = ( cIdx  = =  0 ) ? IntraLumaRefLineIdx[ xTbCmp ][ yTbCmp ] : 0	(310)

The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

The variable refFilterFlag is derived as follows:

- If predModeIntra is equal to 0, −14, −12, −10, −6, 2, 34, 66, 72, 76, 78, or 80, refFilterFlag is set equal to 1.

- Otherwise, refFilterFlag is set equal to 0.

- 8.4.5.2.12  Specification of INTRA_ANGULAR2..INTRA_ANGULAR66 intra prediction modes

Inputs to this process are:

- the intra prediction mode predModeIntra,

- a variable refIdx specifying the intra prediction reference line index,

- a variable nTbW specifying the transform block width,

- a variable nTbH specifying the transform block height,

- a variable refW specifying the reference samples width,

- a variable refH specifying the reference samples height,

- a variable nCbW specifying the coding block width,

- a variable nCbH specifying the coding block height,

- a variable refFilterFlag specifying the value of reference filter flag,

- a variable cIdx specifying the colour component of the current block,

- the neighbouring samples p[ x ][ y ], with x = −1− refIdx, y = −1− refIdx..refH − 1 and x = −refIdx..refW − 1, y = −1− refIdx.

Outputs of this process are the predicted samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable nTbS is set equal to ( Log2 ( nTbW ) + Log2 ( nTbH ) )  >>  1.

The variable filterFlag is derived as follows:

- If one or more of the following conditions are true, filterFlag is set equal to 0:

- refFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 23.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

Experimental result

The proposed method has been implemented using VTM-8.0 and under the common test conditions [1].

Table 1. Coding performance of the proposed method (AI)

Table 2. Coding performance of the proposed method (RA)

Table 3. Coding performance of the proposed method (LD)

Conclusions

This contribution proposes to remove intra mode checking process for intra reference sample filter decision. Experimental results show that the removal provides no coding performance changes without increasing the complexity of encoding and decoding. It is recommended for the proposed change to be adopted as a part of VVC.

Reference

- F. Bossen, el al., “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.

- B. Bross, et al., “Versatile Video Coding (Draft 8,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-P2001(JVET-P2001-vE), 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).