| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0282-r3 |
| --- | --- |

| Title: | GEO with MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone Seungwook Hong Limin Wang | Tel: Email: | +1 858-674-6883 krit.panusopone@nokia.com seungwook.hong@nokia.com limin.2.wang@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to harmonize GEO with MMVD such that MVD can be applied to derive MV of a GEO partition. The proposed method first determines base MV using current GEO MV calculation, then computes MVD for each base MV following method like MVD calculation for MMVD. Simulation results show the proposed method has BD-rate of approximately -0.25%, -0.36%, -0.49% for RA, and -0.51%, -0.66%, -0.97% for LB, respectively, compared to VTM-8.0 anchor.

# Introduction

In the VVC video coding specification [1], geometric partitioning merge mode (GEO) is an inter coding mode that allows flexible partitioning of CU into 2 unequal size rectangular or 2 non-rectangular partitions. The two partitions in a GEO CU have their own uni-direction MV. These MVs are derived from merge candidate index.

VVC also supports merge mode with MV difference (MMVD). A CU using MMVD derives its MV from a base MV and an MVD. Base MV is determined based on one of the first two candidates in the merge list. MVD is computed using MMVD offset which is selectable from predetermined 32 choices. In current version of VVC [1], MMVD is not allowed for GEO CU.

The proposed method extends GEO to allow MMVD style MV refinement. This extension improves prediction accuracy of GEO mode by allowing more flexible MV for GEO partition. However, flexibility in MV refinement comes at a cost in terms of overhead bits and implementation complexity. This contribution uses approach like that of MMVD for GEO MV refinement to minimize these costs while maximizing coding efficiency, since MMVD in VVC has proven to be an efficient tool with low complexity.

# Proposed Method

The proposed method uses six new syntax elements to indicate MVD for two partitions in a GEO CU. The first two flags; geo_mmvd_idx0_flag and geo_mmvd_idx1_flag indicate whether MVD refinement is needed for first GEO, and second GEO partition, respectively. Additional four new syntax elements; geo_mmvd_distance_idx [0], geo_mmvd_distance_idx [1], geo_mmvd_direction_idx [0]. geo_mmvd_direction_idx [1], are conditionally signaled to provide information for MVD calculation. Table 1 shows the proposed syntax.

Table 1. Proposed changes to merge data syntax table

| } else { |  |
| --- | --- |
| if( sps_ciip_enabled_flag && sps_gpm_enabled_flag && slice_type = = B && cu_skip_flag[ x0 ][ y0 ] = = 0 && cbWidth >= 8 && cbHeight >= 8 && cbWidth < ( 8 * cbHeight ) && cbHeight < ( 8 * cbWidth ) && cbWidth < 128 && cbHeight < 128 ) |  |
| ciip_flag[ x0 ][ y0 ] | ae(v) |
| if( ciip_flag[ x0 ][ y0 ] && MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| if( !ciip_flag[ x0 ][ y0 ] ) { |  |
| merge_gpm_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_gpm_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGpmMergeCand > 2 ) |  |
| merge_gpm_idx1[ x0 ][ y0 ] | ae(v) |
| geo_mmvd_idx0_flag[ x0 ][ y0 ] | ae(v) |
| geo_mmvd_idx1_flag[ x0 ][ y0 ] | ae(v) |
| if ( geo_mmvd_idx0_flag[ x0 ][ y0 ] ) { |  |
| geo_mmvd_distance_idx[ 0 ][ x0 ][ y0 ] | ae(v) |
| geo_mmvd_direction_idx[ 0 ][ x0 ][ y0 ] | ae(v) |
| } |  |
| if ( geo_mmvd_idx1_flag[ x0 ][ y0 ] ) { |  |
| geo_mmvd_distance_idx[ 1 ][ x0 ][ y0 ] | ae(v) |
| geo_mmvd_direction_idx[ 1 ][ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |

MVD is calculated using MMVD-like syntax mechanism; distance index and direction. Specifically, distance index indicates amplitude of MVD and direction indicates sign for horizontal or vertical direction of MVD. Same look up tables for 8 distance indices and 4 directions, as in MMVD process, are used in conversion process to derive MVD for each GEO partition.

# Experimental Results

The proposed method is implemented on top of VTM-8.0 reference software. Experimental results are shown in Table 2 against the VTM-8.0 anchor under RA and LB CTC [2], respectively.

Table 2. Simulation results of the proposed method

|  | Random access |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.26% | -0.43% | 128% | 100% |  |  |  |  |  |
| Class A2 | -0.26% | -0.41% | -0.32% | 128% | 100% |  |  |  |  |  |
| Class B | -0.16% | -0.24% | -0.36% | 129% | 101% | -0.32% | -0.35% | -0.82% | 134% | 100% |
| Class C | -0.42% | -0.53% | -0.83% | 131% | 100% | -0.77% | -1.32% | -1.25% | 135% | 99% |
| Class E |  |  |  |  |  | -0.47% | -0.28% | -0.85% | 127% | 98% |
| Overall | -0.25% | -0.36% | -0.49% | 129% | 100% | -0.51% | -0.66% | -0.97% | 133% | 99% |
| Class D | -0.41% | -0.57% | -0.81% | 128% | 100% | -0.68% | -1.55% | -1.32% | 133% | 98% |
| Class F | -0.07% | -0.18% | -0.24% | 119% | 101% | -0.25% | 0.12% | 0.02% | 122% | 99% |

A version of the proposed method with encoder complexity reduction and separate contexts for the proposed syntax elements are implemented on top of VTM-8.0 reference software. This version reduces encoding run time by searching only first 4 MMVD distance indices and 3 GEO angles during MVD refinement in GEO mode decision. Experimental results are shown in Table 3 against the VTM-8.0 anchor under RA and LB CTC [2], respectively.

Table 3. Simulation results of the proposed method with faster encoding

|  | Random access |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.17% | -0.19% | 106% | 99% |  |  |  |  |  |
| Class A2 | -0.19% | -0.24% | -0.21% | 106% | 100% |  |  |  |  |  |
| Class B | -0.09% | -0.23% | -0.15% | 107% | 98% | -0.17% | -0.25% | -0.14% | 110% | 96% |
| Class C | -0.30% | -0.30% | -0.43% | 107% | 91% | -0.50% | -0.68% | -0.57% | 111% | 94% |
| Class E |  |  |  |  |  | -0.21% | -0.37% | -0.29% | 107% | 95% |
| Overall | -0.17% | -0.24% | -0.24% | 107% | 97% | -0.29% | -0.43% | -0.32% | 110% | 94% |
| Class D | -0.29% | -0.45% | -0.51% | 104% | 92% | -0.45% | -1.05% | -0.78% | 111% | 93% |
| Class F | -0.01% | -0.07% | -0.05% | 105% | 93% | -0.11% | 0.59% | -0.47% | 107% | 95% |

# Conclusions

This contribution proposes to use MMVD-like MVD derivation to conditionally refine MV of each GEO partition. Six new syntax elements; two control flags and four MVD related indices, are proposed to support this extension. With the proposed changes, MV of a GEO partition is a sum of base MV and MVD. Base MV is computed using GEO syntax and MVD is computed using MVD related indices. Simulation results show -0.25%, and -0.51% luma BD-rate gains with 100% and 99% relative decoder run-time for RA and LB CTC, respectively.

# References

- B. Bross, J. Chen, S. Lin, and Y-K. Wang, “Versatile Video Coding”, JVET-Q2001-v13, Jan 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Mar. 2019.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).