| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0284 |
| --- | --- |

| Title: | AhG12/AhG9: On independent subpicture signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Yong He Vadim Seregin Muhammed Coban Marta Karczewicz | Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This proposal fixes the unintended design on independent subpicture signalling when there is only one subpicture. There are two proposed changes:

- sps_independent_subpics_flag conditioned on the number of subpictures

- sps_ref_wraparound_enabled_flag conditioned on (!sps_independent_subpics_flag)

# Problem Statement

The sps_independent_subpics_flag (JVET-Q0271) was adopted in the previous meeting for a signalling efficiency purpose by inferring subpic_treated_as_pic_flag[ i ] to be 1 and loop_filter_across_subpic_enabled_flag[ i ] to be 0 if sps_independent_subpics_flag is equal to 1. In another contribution JVET-Q0816 which was also adopted in the previous meeting, the syntax elements subpic_treated_as_pic_flag[ 0 ] and loop_filter_across_subpic_enabled_flag[ 0 ] are inferred to be 0 and 1 respectively when the subpicture signalling is present but there is only one subpicture. Because there are inferences of subpic_treated_as_pic_flag[ 0 ] and loop_filter_across_subpic_enabled_flag[ 0 ] already for the case of single subpicture, it wastes bits to further signal sps_independent_subpics_flag. Besides, it is also meaningless to siginal sps_independent_subpics_flag to infer the values of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] differently when the original intention is to infer the values to be 0 and 1 respectively. Therefore, this constribution proposes to condition sps_independent_subpics_flag on sps_num_subpics_minus1 to remove the redundant signalling bits and to avoid unintended inferences of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ].

In the current VVC, sps_ref_wraparound_enabled_flag is always signeled in SPS, and pps_ref_wraparound_offset is signeled in PPS if pps_ref_wraparound_enabled_flag is equal to 1. This wraparound functionality in the decoding process of inter prediction is disabled if the value of subpic_treated_as_pic_flag[ i ] for the i-th subpicture is equal to 1. Becuase subpic_treated_as_pic_flag[ i ] is inferred to be 1 for all i when sps_independent_subpics_flag is equal to 1, there is no need to signal sps_ref_wraparound_enabled_flag and pps_ref_wraparound_offset in such case. This proposal disables the signalling of the wraparound functionality when sps_independent_subpics_flag is enabled to further add more signalling efficiency.

# sps_independent_subpics_flag conditioned on the number of subpictures

When subpic_info_present_flag is equal to 0, the value of sps_independent_subpics_flag  is inferred to be equal to 0, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0, and the value of loop_filter_across_subpic_enabled_flag[ i ] is inferred to be equal to 1.

When subpic_info_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, subpic_treated_as_pic_flag[ i ] is inferred to be equal to sps_independent_subpics_flag, and loop_filter_across_subpic_enabled_flag[ i ] is inferred to be equal to (1 - sps_independent_subpics_flag).

When subpic_info_present_flag is equal to 1, sps_num_subpics_minus1 is great than 0 and sps_independent_subpics_flag is equal to 1, subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1, loop_filter_across_subpic_enabled_flag[ i ] is inferred to be equal to 0.

The following table shows the current VVC design:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

The table below shows all possible values of these syntax elements:

| subpic_info_present_flag | 0 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| sps_num_subpics_minus1 | 0 (inferred) | 0 |  | > 0 |  |
| sps_independent_subpics_flag | 0 (inferred) | 1 | 0 | 1 | 0 |
| subpic_treated_as_pic_flag[i] | 0 (inferred) | 1 (inferred) | 0 (inferred) | 1(inferred) | 0/1 (signaled) |
| loop_filter_across_subpic_enabled_flag[i] | 1 (inferred) | 0 (inferred) | 1 (inferred) | 0(inferred) | 0/1 (signaled) |

Because the rationale of sps_independent_subpics_flag is intended to specify the independencies between subpictures, there is no benefit to use one bit to signal sps_independent_subpics_flag when there is only one subpicture. Besides, the original intention in JVET-Q0816 for adoption into VVC is there is no need to further signal the information of the subpicture layout, subpic_treated_as_pic_flag[ 0 ] and loop_filter_across_subpic_enabled_flag[ 0 ] when it is single subpicture. However, there are two inferences for subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ 0 ] based on sps_independent_subpics_flag in such case. It is believed the design of JVET-Q0816 was wrongly integrated into VVC while sps_independent_subpics_flag in JVET-Q0271 was integrated at the same time.

The issue can be fixed by moving the condition sps_num_subpics_minus1 > 0 prior to sps_independent_subpics_flag to condition sps_independent_subpics_flag on sps_num_subpics_minus1. The proposed modification removes the redundant bits and avoids unintended two different inferences for subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] when the number of subpictures is 1. The proposed change on top of the VVC Draft 8 is shown by the yellow marks in the following table:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 ) { |  |
| sps_independent_subpics_flag | u(1) |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |
| if( i > 0 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| if( i > 0 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_width_max_in_luma_samples > CtbSizeY ) |  |
| subpic_width_minus1[ i ] | u(v) |
| if( i < sps_num_subpics_minus1 && pic_height_max_in_luma_samples > CtbSizeY ) |  |
| subpic_height_minus1[ i ] | u(v) |
| if( !sps_independent_subpics_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_id_mapping_explicitly_signalled_flag | u(1) |
| if( subpic_id_mapping_explicitly_signalled_flag ) { |  |
| subpic_id_mapping_in_sps_flag | u(1) |
| if( subpic_id_mapping_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |

The table below shows the proposed change marked with yellow for all possible values of these syntax elements:

| subpic_info_present_flag | 0 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| sps_num_subpics_minus1 | 0 (inferred) | 0 |  | > 0 |  |
| sps_independent_subpics_flag | 0 (inferred) | 1 | 0 | 1 | 0 |
| subpic_treated_as_pic_flag[i] | 0 (inferred) | 1 (inferred) | 0 (inferred) | 1(inferred) | 0/1 (signaled) |
| loop_filter_across_subpic_enabled_flag[i] | 1 (inferred) | 0 (inferred) | 1 (inferred) | 0(inferred) | 0/1 (signaled) |

# sps_ref_wraparound_enabled_flag conditioned on the independent subpicture flag

In the current VVC, sps_ref_wraparound_enabled_flag is always signeled in SPS, and pps_ref_wraparound_offset is signeled in PPS if pps_ref_wraparound_enabled_flag is equal to 1. A bitstream conformance constrains the value of pps_ref_wraparound_enabled_flag to be equal to 0 when sps_ref_wraparound_enabled_flag is equal to 0. The following table shows the current VVC design:

seq_parameter_set_rbsp( ) {; Descriptor. …. sps_independent_subpics_flag; u(1). …. sps_ref_wraparound_enabled_flag; u(1). …. sps_independent_subpics_flag equal to 1 specifies that all subpicture boundaries in the CLVS are treated as picture boundaries and there is no loop filtering across the subpicture boundaries. sps_independent_subpics_flag equal to 0 does not impose such a constraint. When not present, the value of sps_independent_subpics_flag is inferred to be equal to 0. sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. pic_parameter_set_rbsp( ) {; Descriptor. …. pps_ref_wraparound_enabled_flag; u(1). if( pps_ref_wraparound_enabled_flag ). pps_ref_wraparound_offset; ue(v). …. pps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. pps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of CtbSizeY / MinCbSizeY + 1 is greater than pic_width_in_luma_samples / MinCbSizeY − 1, the value of pps_ref_wraparound_enabled_flag shall be equal to 0. When sps_ref_wraparound_enabled_flag is equal to 0, the value of pps_ref_wraparound_enabled_flag shall be equal to 0. pps_ref_wraparound_offset plus ( CtbSizeY / MinCbSizeY ) + 2 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of pps_ref_wraparound_offset shall be in the range of 0 to ( pic_width_in_luma_samples / MinCbSizeY ) − ( CtbSizeY / MinCbSizeY ) − 2, inclusive. The variable PpsRefWraparoundOffset is set equal to pps_ref_wraparound_offset+( CtbSizeY / MinCbSizeY ) + 2.

If the value of subpic_treated_as_pic_flag[ i ] for the i-th subpicture is equal to 1, this wraparound functionality in the inter prediction decoding process is completely disabled. Becuase subpic_treated_as_pic_flag[ i ] is already inferred to be 1 for all i when sps_independent_subpics_flag is enabled to be 1, there is no need to signal sps_ref_wraparound_enabled_flag and pps_ref_wraparound_offset.

Therefore, it is proposed to omit the wraparound signalling by inferring sps_ref_wraparound_enabled_flag to be 0 when sps_independent_subpics_flag is enabled. Compared to the current design which would waste bits to signal sps_ref_wraparound_enabled_flag and pps_ref_wraparound_offset in such case, the proposed design can guarantee to omit the signalling bits of sps_independent_subpics_flag and pps_ref_wraparound_offset. The proposed change on top of the VVC Draft 8 is shown by the yellow marks in the following table:

seq_parameter_set_rbsp( ) {; Descriptor. …. if( !sps_independent_subpics_flag ). sps_ref_wraparound_enabled_flag; u(1). …. sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When not present, the value of sps_ref_wraparound_enabled_flag is inferred to be 0.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).