| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0285 |
| --- | --- |

| Title: | AhG12: On tile information signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Yong He Vadim Seregin Muhammed Coban Marta Karczewicz | Email: | yjchang@qti.qualcomm.com yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution, there are two proposed changes to improve the signalling of tile information:

- Replace num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1 with num_exp_tile_columns and num_exp_tile_rows.

- Change the range of num_exp_tile_columns_minus1 to be 0 to PicWidthInCtbsY − 2, inclusive, and the range of num_exp_tile_rows_minus1 to be 0 to PicHeightInCtbsY − 2, inclusive.

# Signalling for single tile column/row

When there is only one tile column in picture, the current VVC design still needs to signal the tile column width even though it is straightforward the tile column width must be PicWidthInCtbsY in unit of CTU size. Figure 1 shows an example for this case.

Figure 1. An example that there is only one tile column. Suppose PicWidthInCtbsY is equal to 16. When the number of tile columns is 1 only, the width of this tile column must be 16 in unit of CTU size.

It is therefore proposed to replace num_exp_tile_columns_minus1 with num_exp_tile_columns. When num_exp_tile_columns is equal to 0, tile_column_width_minus1[ 0 ] is not explicitly signalled, and is inferred to be PicWidthInCtbsY – 1. The same method can be applied to the signaling of tile rows.

The method is applied to VVC 8, and is shown as follows with yellow` change marks:

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= < num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= < num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |

#### 7.4.3.4 Picture parameter set RBSP semantics

num_exp_tile_columns_minus1 plus 1 specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns_minus1 is inferred to be equal to 0.

num_exp_tile_rows_ minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0.

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1 2, inclusive. tile_column_width_minus1[ num_exp_tile_columns_minus1 – 1 ] is used to derive the width of the tile columns with index greater than or equal to num_exp_tile_columns_minus1 – 1  as specified in clause 6.5.1. The value of tile_column_width_minus1[ i ] shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When not present, the value of tile_column_width_minus1[ 0 ] is inferred to be equal to PicWidthInCtbsY − 1.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1 2, inclusive. tile_row_height_minus1[ num_exp_tile_rows_minus1 – 1  ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 – 1 as specified in clause 6.5.1. The value of tile_row_height_minus1[ i ] shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When not present, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1.

### 6.5.1 CTB raster scanning, tile scanning, and subpicture scanning processes

The variable NumTileColumns, specifying the number of tile columns, and the list colWidth[ i ] for i ranging from 0 to NumTileColumns − 1, inclusive, specifying the width of the i-th tile column in units of CTBs, are derived as follows:

if (num_exp_tile_columns == 0 ) {
	NumTileColumns = 1
	colWidth[ 0 ] = PicWidthInCtbsY
} else {
remainingWidthInCtbsY = PicWidthInCtbsY
for( i = 0; i < num_exp_tile_columns_minus1 – 1; i++ ) {
	colWidth[ i ] = tile_column_width_minus1[ i ] + 1
	remainingWidthInCtbsY  −=  colWidth[ i ]
}
uniformTileColWidth = tile_column_width_minus1[ num_exp_tile_columns_minus1 – 1 ] + 1	(23)
while( remainingWidthInCtbsY  >=  uniformTileColWidth ) {
	colWidth[ i++ ] = uniformTileColWidth
	remainingWidthInCtbsY  −=  uniformTileColWidth
}
if( remainingWidthInCtbsY > 0 )
	colWidth[ i++ ] = remainingWidthInCtbsY
NumTileColumns = i 
}

The variable NumTileRows, specifying the number of tile rows, and the list RowHeight[ j ] for j ranging from 0 to NumTileRows − 1, inclusive, specifying the height of the j-th tile row in units of CTBs, are derived as follows:

if (num_exp_tile_rows == 0) {
	RowHeight[ 0 ] = PicHeightInCtbsY
	NumTileRows = 1
} else {
remainingHeightInCtbsY = PicHeightInCtbsY
for( j = 0; j < num_exp_tile_rows_minus1 – 1; j++ ) {
	RowHeight[ j ] = tile_row_height_minus1[ j ] + 1
	remainingHeightInCtbsY  −=  RowHeight[ j ]
}
uniformTileRowHeight = tile_row_height_minus1[ num_exp_tile_rows_minus1 – 1 ] + 1	(24)
while( remainingHeightInCtbsY  >=  uniformTileRowHeight ) {
	RowHeight[ j++ ] = uniformTileRowHeight
	remainingHeightInCtbsY  −=  uniformTileRowHeight
}
if( remainingHeightInCtbsY > 0 )
	RowHeight[ j++ ] = remainingHeightInCtbsY
NumTileRows = j 
}

# The range of num_exp_tile_columns_minus1 and num_exp_tile_rows_minus1

If proposal 1 is adopted, there is no need to review this proposal 2.

The table below shows the current syntax in VVC 8:

| num_exp_tile_columns_minus1 | ue(v) |
| --- | --- |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |

The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When the value of num_exp_tile_columns_minus1 is equal to the maximum value, i.e., PicWidthInCtbsY – 1, tile_column_width_minus1[ i ] must be equal to 0 and is signalled for all i. Figure 2 shows an example for this case in which all tile column widths are equal to 1 in unit of CTU size. However, the most efficient way to signal the case of one-CTU tile column widths is to simply signal num_exp_tile_columns_minus1 as 0 (1 bit) and signal tile_column_width_minus1[ 0 ] as 0 (1 bit). It is obvious the value of num_exp_tile_columns_minus1 equal to PicWidthInCtbsY – 1 is redundant and no benefit while there is other way with fewer bits for signalling of the case. Therefore, it is proposed to change the value of num_exp_tile_columns_minus1 to be in the range of 0 to PicWidthInCtbsY − 2, inclusive. The same method can be applied to the value of tile row heights, i.e., num_exp_tile_rows_minus1. The semantics change on top of VVC 8 is as follows:

num_exp_tile_columns_minus1 plus 1 specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1 2, inclusive. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns_minus1 is inferred to be equal to 0.

num_exp_tile_rows_minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1 2, inclusive. When no_pic_partition_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0.

Figure 2. An example that all tile column widths are equal to 1 in unit of CTU size. Suppose PicWidthInCtbsY is equal to 4. When the number of tile columns is the maximum value, i.e., 4, the width of each tile column must be 1 in unit of CTU size.

If the value of PicWidthInCtbsY – 1 for num_exp_tile_columns_minus1 is still valid, another alternative method is proposed to save the bits by omitting the signalling of tile_column_width_minus1[ i ] when num_exp_tile_columns_minus1 is equal to PicWidthInCtbsY – 1, and the value of tile_column_width_minus1[ i ] shall be inferred to be equal to 0. The same method can be applied to the signalling of tile_row_height_minus1[ i ] when num_exp_tile_rows_minus1 is equal to PicHeightCtbsY – 1.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).