| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0286 |
| --- | --- |

| Title: | AhG9: On general constraint information syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yao-Jen Chang Vadim Seregin Yong He Muhammed Coban Marta Karczewicz | Email: | yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

There are several proposals to improve the general constraint information syntax:

- Semantics cleanup for general constraint flags. There are two alternative solutions:

  - Solution 1: Remove the bitstream conformance “When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1”.

  - Solution 2: Constrain the chroma-related constraint flags to be equal to 1 when max_chroma_format_constraint_idc is equal to 0 (i.e., when there is no chroma components). Constrain the inter prediction-related constraint flags to be equal to 1 when intra_only_constraint_flag is equal to 1 (i.e., when slice_type are I for all slices).

- Add general constraint flags for the coding tools

- Add general constraint flags for the single layer and single sublayer

- Add general constraint flags for the VPS and PH

# Semantics cleanup for general constraint flags

In current general constraint syntax design, there is a bitstream conformance to constrain one_subpic_per_pic_constraint_flag by one_slice_per_pic_constraint_flag as follows:

When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1.

If there is no such conformance constraint to one_subpic_per_pic_constraint_flag, there is no bug to the decoder from high-level parameter sets to low-level decoding process, and this constraint might be redundant to parameters sets and decoding process. However, there might be beneficial to avoid a product to set up a profile which causes confusion. If the benefit of the bitstream conformance is valid, more bitstream conformance shall be added into the general constraint flags to avoid the similar confusions. Otherwise, such conformance constraint shall be removed for cleanup purpose. Therefore, this proposal provides two alternative solutions to address the issue:

Solution 1: Remove the bitstream conformance “When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1”.

Solution 2: Constrain the chroma-related constraint flags to be equal to 1 when max_chroma_format_constraint_idc is equal to 0. Constrain the inter prediction-related constraint flags to be equal to 1 when intra_only_constraint_flag is equal to 1. The proposed changes on top of VVC 8 are shown by the yellow marks as follows:

no_qtbtt_dual_tree_intra_constraint_flag equal to 1 specifies that qtbtt_dual_tree_intra_flag shall be equal to 0. no_qtbtt_dual_tree_intra_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_qtbtt_dual_tree_intra_constraint_flag shall be equal to 1.

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0. no_ccalf_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_ccalf_constraint_flag shall be equal to 1.

no_joint_cbcr_constraint_flag equal to 1 specifies that sps_joint_cbcr_enabled_flag shall be equal to 0.  no_joint_cbcr_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_joint_cbcr_constraint_flag shall be equal to 1.

no_ref_wraparound_constraint_flag equal to 1 specifies that sps_ref_wraparound_enabled_flag shall be equal to 0. no_ref_wraparound_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_ref_wraparound_constraint_flag shall be equal to 1.

no_temporal_mvp_constraint_flag equal to 1 specifies that sps_temporal_mvp_enabled_flag shall be equal to 0. no_temporal_mvp_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_temporal_mvp_constraint_flag shall be equal to 1.

no_sbtmvp_constraint_flag equal to 1 specifies that sps_sbtmvp_enabled_flag shall be equal to 0. no_sbtmvp_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_sbtmvp_constraint_flag shall be equal to 1.

no_amvr_constraint_flag equal to 1 specifies that sps_amvr_enabled_flag shall be equal to 0. no_amvr_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_amvr_constraint_flag shall be equal to 1.

no_bdof_constraint_flag equal to 1 specifies that sps_bdof_enabled_flag shall be equal to 0. no_bdof_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_bdof_constraint_flag shall be equal to 1.

no_dmvr_constraint_flag equal to 1 specifies that sps_dmvr_enabled_flag shall be equal to 0. no_dmvr_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_dmvr_constraint_flag shall be equal to 1.

no_cclm_constraint_flag equal to 1 specifies that sps_cclm_enabled_flag shall be equal to 0. no_cclm_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_cclm_constraint_flag shall be equal to 1.

no_affine_motion_constraint_flag equal to 1 specifies that sps_affine_enabled_flag  shall be equal to 0. no_affine_motion_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_affine_motion_constraint_flag shall be equal to 1.

no_bcw_constraint_flag equal to 1 specifies that sps_bcw_enabled_flag shall be equal to 0. no_bcw_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_bcw_constraint_flag shall be equal to 1.

no_ciip_constraint_flag equal to 1 specifies that sps_ciip_enabled_flag shall be equal to 0. no_cipp_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_cipp_constraint_flag shall be equal to 1.

no_fpel_mmvd_constraint_flag equal to 1 specifies that sps_fpel_mmvd_enabled_flag shall be equal to 0. no_fpel_mmvd_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_fpel_mmvd_constraint_flag shall be equal to 1.

no_gpm_constraint_flag equal to 1 specifies that sps_gpm_enabled_flag shall be equal to 0. no_gpm_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_gpm_constraint_flag shall be equal to 1.

In another alternative method, explicit condition checks can be added to the syntax to constrain the flags instead of the bitstream conformance. One example is to signal one_subpic_per_pic_constraint_flag only when the value of one_slice_per_pic_constraint_flag is equal to 0, and the value of one_subpic_per_pic_constraint_flag is inferred to be equal to 1 when one_slice_per_pic_constraint_flag is equal to 1.

# General constraint flags for the coding tools

Several coding tools are missing in the general constraint information syntax. These coding tools flags should be added to provide the same general constraint controls as others. The missing flags are added into VVC 8 as follows:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_lfnst_constraint_flag | u(1) |
| no_mmvd_constraint_flag | u(1) |
| no_smvd_constraint_flag | u(1) |
| no_prof_constraint_flag | u(1) |
| no_palette_constraint_flag | u(1) |
| no_act_constraint_flag | u(1) |
| no_lmcs_constraint_flag | u(1) |

no_mrl_constraint_flag equal to 1 specifies that sps_mrl_enabled_flag shall be equal to 0.  no_mrl_constraint_flag equal to 0 does not impose such a constraint.

no_isp_constraint_flag equal to 1 specifies that sps_isp_enabled_flag shall be equal to 0.  no_isp_constraint_flag equal to 0 does not impose such a constraint.

no_mip_constraint_flag equal to 1 specifies that sps_mip_enabled_flag shall be equal to 0.  no_mip_constraint_flag equal to 0 does not impose such a constraint.

no_lfnst_constraint_flag equal to 1 specifies that sps_lfnst_enabled_flag  shall be equal to 0. no_lfnst_constraint_flag equal to 0 does not impose such a constraint.

no_mmvd_constraint_flag equal to 1 specifies that sps_mmvd_enabled_flag  shall be equal to 0. no_mmvd_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_mmvd_constraint_flag shall be equal to 1.

no_smvd_constraint_flag equal to 1 specifies that sps_smvd_enabled_flag  shall be equal to 0. no_smvd_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_smvd_constraint_flag shall be equal to 1.

no_prof_constraint_flag equal to 1 specifies that sps_affine_prof_enabled_flag  shall be equal to 0. no_prof_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_prof_constraint_flag shall be equal to 1.

no_palette_constraint_flag equal to 1 specifies that sps_palette_enabled_flag shall be equal to 0. no_palette_constraint_flag equal to 0 does not impose such a constraint.

no_act_constraint_flag equal to 1 specifies that sps_act_enabled_flag  shall be equal to 0. no_act_constraint_flag equal to 0 does not impose such a constraint.

no_lmcs_constraint_flag equal to 1 specifies that sps_lmcs_enabled_flag  shall be equal to 0. no_lmcs_constraint_flag equal to 0 does not impose such a constraint.

# General constraint flags for the single layer and single sublayer

New general constraint flags are added into the general constraint information syntax to define the features of single layer and single sublayer. The proposed changes on top of VVC 8 are shown by the yellow marks as follows:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| general_interlaced_source_flag | u(1) |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_non_projected_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| single_layer_constraint_flag | u(1) |
| single_sublayer_constraint_flag | u(1) |
| single_sublayer_per_layer_constraint_flag | u(1) |
| no_inter_layer_pred_constraint_flag | u(1) |
| no_res_change_in_clvs_constraint_flag | u(1) |
| one_tile_per_pic_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |

single_layer_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_video_parameter_set_id shall be equal to 0. single_layer_constraint_flag equal to 0 does not impose such a constraint.

single_sublayer_constraint_flag equal to 1 specifies it is a requirement of bitstream conformance that both sps_video_parameter_set_id and sps_max_sublayers_minus1 shall be equal to 0. single_sublayer_constraint_flag equal to 0 does not impose such a constraint.

single_sublayer_per_layer_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that sps_max_sublayers_minus1 shall be equal to 0. single_sublayer_per_layer_constraint_flag equal to 0 does not impose such a constraint.

no_inter_layer_pred_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that vps_all_independent_layers_flag shall be equal to 1. no_inter_layer_pred_constraint_flag equal to 0 does not impose such a constraint. When single_layer_constraint_flag is equal to 1, the value of no_inter_layer_pred_constraint_flag shall be equal to 1.

# General constraint flags for the VPS and PH

no_vps_constraint_flag and no_ph_constraint_flag are added into the general constraint information syntax to define the features of VPS and PH. The proposed changes on top of VVC 8 are shown by the yellow marks as follows:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| no_trail_constraint_flag | u(1) |
| no_stsa_constraint_flag | u(1) |
| no_rasl_constraint_flag | u(1) |
| no_radl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_vps_constraint_flag | u(1) |
| no_ph_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |

no_vps_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to VPS_NUT present in OlsInScope. no_vps_constraint_flag equal to 0 does not impose such a constraint.

no_ph_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to PH_NUT present in OlsInScope. no_ph_constraint_flag equal to 0 does not impose such a constraint.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).