| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0287 |
| --- | --- |

| Title: | AHG9: On high level control parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang Yao-Jen Chang Muhammed Coban Wei-Jung Chien Vadim Seregin Marta Karczewicz | Email: | hanhuang@qti.qualcomm.com yjchang@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

The contribution proposes the following changes on high level tool control syntax elements.

- Signal qtbtt_dual_tree_intra_flag in PH for dual-tree on/off control

- Signal log2_transform_skip_max_size_minus2 with u(2) used for the maximum block size used for transform skip.

# Proposal

## qtbtt_dual_tree_intra_flag

In the current VVC draft, syntax elements that control block partitioning can be signalled in a picture header (PH), but there is no such PH control for the dual-tree partitioning. The dual-tree on/off control is only present in SPS which limits an encoder flexibility.

This contribution proposes to add a dual-tree on/off control in PH similar to other block partitioning syntax elements. The proposed text change with the added text highlighted in yellow is as follows.

SPS:

| if( ChromaArrayType != 0 ) |  |
| --- | --- |
| sps_qtbtt_dual_tree_intra_flag | u(1) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |

no_qtbtt_dual_tree_intra_constraint_flag equal to 1 specifies that sps_qtbtt_dual_tree_intra_flag shall be equal to 0. no_qtbtt_dual_tree_intra_constraint_flag equal to 0 does not impose such a constraint.

sps_qtbtt_dual_tree_intra_flag equal to 1 specifies that, for I slices, each CTU is split into coding units with 64×64 luma samples using an implicit quadtree split, and these coding units are the root of two separate coding_tree syntax structure for luma and chroma. sps_qtbtt_dual_tree_intra_flag equal to 0 specifies separate coding_tree syntax structure is not used for I slices. When sps_qtbtt_dual_tree_intra_flag is not present, it is inferred to be equal to 0. When partition_constraints_override_enabled_flag is equal to 1, the sps_qtbtt_dual_tree_intra_flag can be overridden by qtbtt_dual_tree_intra_flag in PHs referring to the SPS.

PH:

| if( partition_constraints_override_enabled_flag ) |  |
| --- | --- |
| partition_constraints_override_flag | u(1) |
| if( ph_intra_slice_allowed_flag ) { |  |
| if( partition_constraints_override_flag ) { |  |
| if( sps_qtbtt_dual_tree_intra_flag ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| ph_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| ph_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| ph_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| ph_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| ph_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |

qtbtt_dual_tree_intra_flag equal to 1 specifies that, for I slices, each CTU is split into coding units with 64×64 luma samples using an implicit quadtree split, and these coding units are the root of two separate coding_tree syntax structure for luma and chroma. qtbtt_dual_tree_intra_flag equal to 0 specifies separate coding_tree syntax structure is not used for I slices. When qtbtt_dual_tree_intra_flag is not present, it is inferred to be equal to sps_qtbtt_dual_tree_intra_flag.

## log_transform_skip_max_size_minus2

In the current VVC design, the SPS syntax element log2_transform_skip_max_size_minus2 derives the maximum block size used for transform skip by 1  <<  ( log2_transform_skip_max_size_minus2 + 2 ), and it shall be in the range of 0 to 3. ue(v) is used for coding four different values. The binarization is as follows:

| log2_transform_skip_max_size_minus2 | ue(v) |
| --- | --- |
| 0 | 1 |
| 1 | 010 |
| 2 | 011 |
| 3 | 00100 |

It is proposed to replace ue(v) with u(2) to code log2_transform_skip_max_size_minus2 as follows:

| log2_transform_skip_max_size_minus2 | u(2) |
| --- | --- |
| 0 | 00 |
| 1 | 01 |
| 2 | 10 |
| 3 | 11 |

In CTC, the value of log2_transform_skip_max_size_minus2 is set equal to 3, u(2) codeword is shorter than ue(v) in this case.

SPS:

| sps_transform_skip_enabled_flag | u(1) |
| --- | --- |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | u(2) ue(v) |
| sps_bdpcm_enabled_flag | u(1) |
| } |  |

log2_transform_skip_max_size_minus2 specifies the maximum block size used for transform skip, and shall be in the range of 0 to 3, inclusive.

The variable MaxTsSize is set equal to 1  <<  ( log2_transform_skip_max_size_minus2 + 2 ).

Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).