| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0289 |
| --- | --- |

| Title: | [AHG16] On deblocking filter process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In Ticket #899, it was claimed that there is a mismatch between VVC draft 8 and VTM8.0 on deblocking filter process when CU size is greater than maximum transform block size and cu_coded_flag is equal to 0.

In VVC draft 8, implicit TU tiling is applied even though cu_coded_flag is equal to 0 for inter prediction mode. These internal transform block boundaries within the CU is considered as transform block boundary in deblocking filter process. However, in the VTM8.0, the above case is treated as a TU which has the same size as CU therefore there is no internal transform block boundary. Although the mismatch has already resolved by the ticket #899, this cause misalignment in terms of filter length on the each block boundary with same property.

In this proposal, to fix abovementioned behavior two solutions are suggested.

- Method1: Implicit TU tiling is restricted when cu_coded_flag is equal to 0.

- Method2: Implicit TU tiling is retained but edges inside CU is not treated as transform edge when cu_coded_flag is equal to 0.

Since Ticket #899 is reverted by Method 1 and deblocking filter process is fixed to be performed same as VTM8.0 by Method 2, the experimental results for both solutions are same as results of VTM8.0.

Introduction

In the decoding process of deblocking filter, CU, TU and/or subblock boundaries are checked to determine these boundaries to be filtered or not.

First, CU boundaries are checked. In this stage, whether the current CU boundaries are filtered is determined by boundary filter flags, e.g. loop_filter_across_subpic_enabled_flag, loop_filter_across_tiles_enabled_flag or loop_filter_across_slices_enabled flag. If current CU is aligned with these tile/slice/subpicture boundaries with flags is equal to 0, deblocking filtering is not performed at these boundary. Below table shows a part of this stage.

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0: - The left boundary of the current coding block is the left boundary of the picture. - The left boundary of the current coding block coincides with the left boundary of the current subpicture and loop_filter_across_subpic_enabled_flag[ CurrSubpicIdx ] or loop_filter_across_subpic_enabled_flag[ subpicIdx ] is equal to 0, where subpicIdx is the subpicture index of the subpicture for which the left boundary of the current coding block coincides with the right subpicture boundary of that subpicture. - The left boundary of the current coding block is the left boundary of the tile and loop_filter_across_tiles_enabled_flag is equal to 0. - The left boundary of the current coding block is the left boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0. - The left boundary of the current coding block is one of the vertical virtual boundaries of the picture and VirtualBoundariesPresentFlag is equal to 1.

Second, it is determined whether the edges in CU is transform block edge or not. In current CU, edges are checked every 4-pixels (8-pixels for Chroma) units to determine whether a transform block edge exists. Below table shows a part of this stage for vertical edges.

- The variable gridSize is set as follows: gridSize = cIdx = = 0 ? 4 : 8 (1246) - If edgeType is equal to EDGE_VER, the following applies: - The variable numEdges is set equal to Max( 1, nCbW / gridSize ). - For xEdge = 0..numEdges − 1 and y = 0..nCbH − 1, the following applies: - The horizontal position x inside the current coding block is set equal to xEdge * gridSize. - The value of edgeFlags[ x ][ y ] is derived as follows: - If VirtualBoundariesPresentFlag equal to 1 and ( xCb + x ) is equal to VirtualBoundariesPosX[ n ] for any n = 0..NumVerVirtualBoundaries − 1, edgeFlags[ x ][ y ] is set equal to 0. - Otherwise, if x is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag. - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge, edgeFlags[ x ][ y ] is set equal to 1.

Third, it is determined whether the edges in CU are subblock edge or not for luma component. In this process, edges in a CU are checked every 8-pixels unit to determine whether a subblock edge exists. If it is a subblock edge, filter length of each boundary is adjusted as following table.

- If x is equal to 0, the following applies: [CASE1] - When numSbX is greater than 1, the following applies: maxFilterLengthQs[ x ][ y ] = Min( 5, maxFilterLengthQs[ x ][ y ] ) (1249) - When inter_affine_flag[ xCb − 1 ][ yCb + y ] is equal to 1 or merge_subblock_flag[ xCb − 1 ][ yCb + y ] is equal to 1, the following applies: maxFilterLengthPs[ x ][ y ] = Min( 5, maxFilterLengthPs[ x ][ y ] ) (1250) - Otherwise, if edgeTbFlags[ x ][ y ] is equal to 1, the following applies: [CASE2] maxFilterLengthPs[ x ][ y ] = Min( 5, maxFilterLengthPs[ x ][ y ] ) (1251) maxFilterLengthQs[ x ][ y ] = Min( 5, maxFilterLengthQs[ x ][ y ] ) (1252) - Otherwise, if one or more of the following conditions are true: [CASE3] - ( x + 4 ) is greater than or equal to nCbW, - edgeTbFlags[ x − 4 ][ y ] is equal to 1, - edgeTbFlags[ x + 4 ][ y ] is equal to 1, the following applies: maxFilterLengthPs[ x ][ y ] = 1 (1253) maxFilterLengthQs[ x ][ y ] = 1 (1254) - Otherwise, if one or more of the following conditions are true: [CASE4] - xEdge is equal to 1, - xEdge is equal to ( nCbW / 8 ) − 1, - edgeTbFlags[ x − sbW ][ y ] is equal to 1, - edgeTbFlags[ x + sbW ][ y ] is equal to 1, the following applies: maxFilterLengthPs[ x ][ y ] = 2 (1255) maxFilterLengthQs[ x ][ y ] = 2 (1256) - Otherwise, the following applies: [CASE5] maxFilterLengthPs[ x ][ y ] = 3 (1257) maxFilterLengthQs[ x ][ y ] = 3 (1258)

These are five cases to adjust the filter length for subblock edges. Figure 1 simply shows the example CASE1, CASE2, CASE4 and CASE5. Especially, in this example, the filter length of CU boundary and TU boundary in a CU is adjusted from 7 to 5 to handle the parallel processing, respectively according to CASE1 and CASE2. Also, filter length of subblock boundaries, which is placed nearby CU or TU boundary are adjusted from 3 to 2 for same reason according to CASE4 and CASE5, respectively.

Figure 1

In Ticket #899, it was claimed that there is a mismatch between VVC draft 8 and VTM8.0on deblocking filter process for that case.

- The CU with large block size (greater than maximum transform block)

- cu_coded_flag(cu_cbf_flag in VVC draft 7) is equal to 0

In VVC draft 8, if transform block size is greater than maximum transform block, implicit TU tiling is applied even though cu_coded_flag is equal to 0 for inter prediction mode. Eventually, filter length for edges of transform block with all zero coefficients are derived and filter length for edged of subblock is adjusted to smaller value. But, in the VTM8.0, these cases is treated as an empty TU which has the same size as CU therefore there is no internal transform block boundary. Although the mismatch has already resolved by the ticket #899, this cause misalignment in terms of filter length on the each block boundary with same property.

Proposed method

The Figure 2 shows the proposed method to address above mentioned case when maximum transform block size is 32x32 as an example.

Figure 2

In this contribution, two solutions are proposed.

In the method 1, implicit TU tiling is restricted when cu_coded_flag is equal to 1.

In the derivation process for inter prediction mode, if cu_coded_flag is equal to 0, residual sample derivation process is not performed and residual samples in a transform block is set equal to 0. Since implicit TU tiling is restricted, there is no internal transform edge and subblock edges in a CU has same filter length with consistent rule.

In the method 2, implicit TU tiling is retained but edges inside CU is not treated as transform edge when cu_coded_flag is equal to 0.

In the transform edge derivation process, if there are edges in a CU, which has cu_coded_flag is equal to 0, the edges are not treated as a transform edge. And, subblock edges in a CU is not affected by transform edge.

Proposed spec text changes

Method 1

In decoding process for coding units coded in inter prediction mode (8.5.1), modified texts are marked as highlighted yellow text.

- The residual samples of the current coding unit are derived as follows:

- If cu_coded_flag[xCb][yCb] is equal to 1, the following applies:

- The decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the luma location ( xCb, yCb ), the width nTbW set equal to the luma coding block width cbWidth, the height nTbH set equal to the luma coding block height cbHeight and the variable cIdx set equal to 0 as inputs, and the array resSamplesL as output.

- When ChromaArrayType is not equal to 0. the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the width nTbW set equal to the chroma coding block width cbWidth / SubWidthC, the height nTbH set equal to the chroma coding block height cbHeight / SubHeightC and the variable cIdx set equal to 1 as inputs, and the array resSamplesCb as output.

- When ChromaArrayType is not equal to 0. the decoding process for the residual signal of coding blocks coded in inter prediction mode as specified in clause 8.5.8 is invoked with the location ( xTb0, yTb0 ) set equal to the chroma location ( xCb / SubWidthC, yCb / SubHeightC ), the width nTbW set equal to the chroma coding block width cbWidth / SubWidthC, the height nTbH set equal to the chroma coding block height cbHeight / SubHeightC and the variable cIdx set equal to 2 as inputs, and the array resSamplesCr as output.

- When cu_act_enabled_flag[ xCb ][ yCb ] is equal to 1, the residual modification process for residual blocks using colour space conversion as specified in clause 8.7.4.6 is invoked with the variable nTbW set equal to cbWidth, the variable nTbH set equal to cbHeight, the array rY set equal to resSamplesL, the array rCb set equal to resSamplesCb, and the array rCr set equal to resSamplesCr as inputs, and the output are modified versions of the arrays resSamplesL, resSamplesCb and resSamplesCr.

- Otherwise (cu_coded_flag[xCb][yCb] is equal to 0), the arrays resSamplesL, resSamplesCb and resSamplesCr are inferred to be equal to 0.

Method 2

In the derivation process of transform block boundary (8.8.3.3), modified texts are marked as highlighted yellow text.

#### 8.8.3.3 Derivation process of transform block boundary

Inputs to this process are:

- a location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left sample of the current picture,

- a variable nCbW specifying the width of the current coding block,

- a variable nCbH specifying the height of the current coding block,

- a variable cIdx specifying the colour component of the current coding block,

- a variable filterEdgeFlag,

- a two-dimensional (nCbW)x(nCbH) array edgeFlags,

- two-dimensional (nCbW)x(nCbH) arrays maxFilterLengthQs and maxFilterLengthPs,

- a variable edgeType specifying whether a vertical (EDGE_VER) or a horizontal (EDGE_HOR) edge is filtered.

Outputs of this process are:

- the modified two-dimensional (nCbW)x(nCbH) array edgeFlags,

- the modified two-dimensional (nCbW)x(nCbH) arrays maxFilterLengthQs, maxFilterLengthPs.

Depending on edgeType, the arrays edgeFlags, maxFilterLengthPs and maxFilterLengthQs are derived as follows:

- The variable gridSize is set as follows:

gridSize = cIdx  = =  0 ? 4 : 8		(1246)

- If edgeType is equal to EDGE_VER, the following applies:

- The variable numEdges is set equal to Max( 1, nCbW / gridSize ).

- For xEdge = 0..numEdges − 1 and y = 0..nCbH − 1, the following applies:

- The horizontal position x inside the current coding block is set equal to xEdge * gridSize.

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If VirtualBoundariesPresentFlag equal to 1 and ( xCb + x ) is equal to VirtualBoundariesPosX[ n ] for any n = 0..NumVerVirtualBoundaries − 1, edgeFlags[ x ][ y ] is set equal to 0.

    - Otherwise, if x is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge and cu_coded_flag[xCb][yCb] is equal to 1, edgeFlags[ x ][ y ] is set equal to 1.

- When edgeFlags[ x ][ y ] is equal to 1,the following applies:

    - If cIdx is equal to 0, the following applies:

    - The value of maxFilterLengthQs[ x ][ y ] is derived as follows:

    - If the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or less than 4, maxFilterLengthQs[ x ][ y ] is set equal to 1.

    - Otherwise, if the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or greater than 32, maxFilterLengthQs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - The value of maxFilterLengthPs[ x ][ y ] is derived as follows:

    - If the width in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or less than 4 , maxFilterLengthPs[ x ][ y ] is set equal to 1.

    - Otherwise, if the width in luma samples of the transform block at luma location ( xCb + x − 1, yCb + y ) is equal to or greater than 32, maxFilterLengthPs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthPs[ x ][ y ] is set equal to 3.

    - Otherwise (cIdx is not equal to 0), the values of maxFilterLengthPs[ x][ y ] and maxFilterLengthQs[ x ][ y ] are derived as follows:

    - If the width in chroma samples of the transform block at chroma location ( xCb + x, yCb + y ) and the width at chroma location ( xCb + x − 1,  yCb + y ) are both equal to or greater than 8, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 3.

    - Otherwise, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 1.

- Otherwise (edgeType is equal to EDGE_HOR), the following applies:

- The variable numEdges is set equal to Max( 1, nCbH / gridSize ).

- For yEdge = 0..numEdges − 1 and x = 0..nCbW − 1, the following applies:

- The vertical position y inside the current coding block is set equal to yEdge * gridSize.

- The value of edgeFlags[ x ][ y ] is derived as follows:

    - If VirtualBoundariesPresentFlag equal to 1 and ( yCb + y ) is equal to VirtualBoundariesPosY[ n ] for any n = 0..NumHorVirtualBoundaries − 1, edgeFlags[ x ][ y ] is set equal to 0.

    - Otherwise, if y is equal to 0, edgeFlags[ x ][ y ] is set equal to filterEdgeFlag.

    - Otherwise, if the location ( xCb + x , yCb + y ) is at a transform block edge and cu_coded_flag[xCb][yCb] is equal to 1, edgeFlags[ x ][ y ] is set equal to 1.

- When edgeFlags[ x ][ y ] is equal to 1,the following applies:

    - If cIdx is equal to 0, the following applies:

    - The value of maxFilterLengthQs[ x ][ y ] is derived as follows:

    - If the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or less than 4, maxFilterLengthQs[ x ][ y ] is set equal to 1.

    - Otherwise, if the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or greater than 32, maxFilterLengthQs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - The value of maxFilterLengthPs[ x ][ y ] is derived as follows:

    - If the height in luma samples of the transform block at luma location ( xCb + x, yCb + y ) is equal to or less than 4 or the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or less than 4, maxFilterLengthPs[ x ][ y ] is set equal to 1.

    - Otherwise, if the height in luma samples of the transform block at luma location ( xCb + x, yCb + y − 1 ) is equal to or greater than 32, maxFilterLengthPs[ x ][ y ] is set equal to 7.

    - Otherwise, maxFilterLengthPs[ x ][ y ] is set equal to 3.

    - Otherwise (cIdx is not equal to 0), the values of maxFilterLengthPs[ x][ y ] and maxFilterLengthQs[ x ][ y ] are derived as follows:

    - If the height in chroma samples of the transform block at chroma location ( xCb + x, yCb + y ) and the height in chroma samples of the transform block at chroma location ( xCb + x,  yCb + y − 1 ) are both equal to or greater than 8, the following applies:

    - If ( yCb + y ) % CtbHeightC is greater than 0, i.e. the horizontal edge do not overlap with the upper chroma CTB boundary, both maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 3

    - Otherwise ( ( yCb + y ) % CtbHeightC is equal to 0, i.e. the horizontal edge overlaps with the upper chroma CTB boundary), maxFilterLengthPs[ x ][ y ] is set equal to 1 and maxFilterLengthQs[ x ][ y ] is set equal to 3.

    - Otherwise, maxFilterLengthPs[ x ][ y ] and maxFilterLengthQs[ x ][ y ] are set equal to 1.

Conclusion

When cu_coded_flag is equal to 0 and a CU is implicitly split, transform blocks in a CU, which have all zero coefficient is treated as transform edge. But, from these process, sub-block edge has prediction error at boundary is not well controlled. In this contribution, two solutions are proposed to solve the addressed issue in deblocking process. The experimental results reportedly show no changes compared to VTM8.0. It is recommended to consider this method in the next VTM.

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 8)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-Q2001(JVET-Q2001-vE), 17th Meeting: Brussels, BE, 7–17 Jan. 2020.

- Frank Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-N1010, 14th Meeting: Geneva, CH, 19–27 Mar. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).