| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0290-v1 |
| --- | --- |

| Title: | AHG16: LMCS constraint cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd. |  |  |

_____________________________

# Abstract

It is asserted that the expression of constraints on LMCS parameters is needlessly convoluted. An alternative definition of constraints is proposed. While the proposed constraints are not strictly equivalent to the ones in VVC draft 8, no impact on coding efficiency is observed under common test conditions. It is asserted that the proposed constraint is much more straightforward.

# Proposal

LMCS parameters are constrained such that when applying the inverse LMCS function, the value space can be divided into 32 bins of equal size, where for each bin, a single comparison is required to determine the index of the pivot point to use for mapping.

In VVC draft 8 the lower bound on lmcs_delta_sign_cw_flag[ i ] is guided by two constraints:

- The value of lmcsCW[ i ] shall be in the range of OrgCW  >>  3 to ( OrgCW  <<  3 ) − 1, inclusive.

- It is a requirement of bitstream conformance that, for i = lmcs_min_bin_idx..LmcsMaxBinIdx, when the value of LmcsPivot[ i ] is not a multiple of 1  <<  ( BitDepth − 5 ), the value of ( LmcsPivot[ i ]  >>  ( BitDepth − 5 ) ) shall not be equal to the value of ( LmcsPivot[ i + 1 ]  >>  ( BitDepth − 5 ) ).

The first constraint expresses a general constraint on the range of lmcs_delta_sign_cw_flag. The second constraint guarantees that a single comparison is required to determine the pivot point index.

It is proposed to remove the second constraint, and modify the first constraint to:

- The value of lmcsCW[ i ] shall be in the range of OrgCW  / 2 − 1 to OrgCW * 8 − 1, inclusive.

The proposed change:

- Simplifies conformance checks, and may also simplify encoder design;

- Reduces the maximum value of InvScaleCoeff[ i ] from 16384 to 4681, thereby reducing the size of the multiplier in inverse LMCS function from Nx15 to Nx13 bits, where N is the sample bit depth;

- Preserves existing implementation properties (e.g., single comparison the determine pivot point index); and

- Reduces the set of possible values the LMCS function may take. However, no practical example is known where this has an impact.

Alternatively, the range could be further reduced to:

- The value of lmcsCW[ i ] shall be in the range of OrgCW  / 2 + 1 to OrgCW * 2 − 1, inclusive.

This reduced range is sufficient to represent all values encountered in common test conditions (VTM 8.0, SDR and HDR), where lmcsCW[ i ] ranges from  OrgCW  / 2 + 14 to OrgCW * 2 – 4.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).