| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0291-v3 |
| --- | --- |

| Title: | AHG16: On ALF attenuation near virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | +1 647 745 1691 fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd. |  |  |

_____________________________

# Abstract

At the 17th JVET meeting ALF attenuation near virtual boundaries was introduced. Two modifications are proposed: a) rounding is modified such as to match the increased shift amount when attenuation is applied b) attenuation is extended to CC-ALF, where attenuation factors are further increased to limit the amount of required line buffers when using 4:2:2 and 4:4:4 chroma sampling formats.

# Proposal

ALF attenuation is captured in equation (1467) of VVC draft 8:

sum = curr + ( ( sum + 64 )  >>  alfShiftY  )

where alfShiftY is set to 7 except for the rows immediately below and above virtual boundaries, where it is set to 10.

It is proposed to modify the rounding factor 64 such as to match the amount of shifting:

sum = curr + ( ( sum + ( 1 << alfShifY >> 1 ) )  >>  alfShiftY  )

Processing for chroma is modified similarly.

It is further proposed to use a similar process of CC-ALF where Table 47 is modified as below:

| Condition | ccalfShiftC | yP1 | yP2 |
| --- | --- | --- | --- |
| ( y * subHeightC = = CtbSizeY − 5 ¦ ¦ ( y * subHeightC = = CtbSizeY − 4 && subHeightC = = 2 ) ) && applyAlfLineBufBoundary = = 1 | 10 | 0 | 0 |
| (y * subHeightC = = CtbSizeY – 3 ¦ ¦ ( y * subHeightC = = CtbSizeY − 4 && subHeightC = = 1 ) ) && applyAlfLineBufBoundary = = 1 | BitDepth + 11 | 0 | 0 |
| y * subHeightC = = CtbSizeY − 6 && applyAlfLineBufBoundary = = 1 | 7 | 1 | 1 |
| Otherwise | 7 | 1 | 2 |

where a new variable ccalfShiftC is introduced to control the amount of shifting in equation (1533):

scaledSum = Clip3( −( 1 << ( BitDepth − 1 ) ), ( 1 << ( BitDepth  − 1 ) ) − 1, ( sum + ( 1 << ccalfShiftC >> 1 ) ) >> ccalfShiftC )

It should be noted that when subHeightC is equal to 1 (i.e., for 4:2:2 and 4:4:4 chroma formats), the value of ccalfShiftC is increased to BitDepth + 11 for the two rows immediately below the luma virtual boundary. Doing so results in the CC-ALF correction always being zero for these rows. This is done to deal with the spatial misalignment of luma and chroma virtual boundaries (in the 4:2:2 and 4:4:4 cases the luma virtual boundary is located 2 samples above the chroma virtual boundaries; such misalignment is not present in the 4:2:0 case).

# Results

The impact of the proposed changes on coding efficiency under CTC are summarized below.

|  |  |  | All Intra |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.00% | 0.06% | 0.07% | 0.02% | 99% | 101% |
| Class A2 | 0.00% | 0.09% | 0.12% | 0.02% | 99% | 101% |
| Class B | 0.00% | 0.11% | 0.08% | 0.02% | 99% | 101% |
| Class C | 0.00% | 0.02% | 0.03% | 0.01% | 99% | 99% |
| Class E | 0.00% | 0.03% | 0.06% | 0.01% | 99% | 101% |
| Overall | 0.00% | 0.07% | 0.07% | 0.02% | 99% | 100% |
| Class D | 0.00% | 0.02% | 0.03% | 0.00% | 99% | 101% |
| Class F | 0.00% | 0.04% | 0.04% | 0.01% | 100% | 101% |
|  |  |  | Random access |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.01% | -0.02% | 0.02% | 0.01% | 99% | 100% |
| Class A2 | 0.02% | -0.04% | 0.21% | 0.03% | 100% | 100% |
| Class B | 0.01% | 0.05% | 0.04% | 0.01% | 100% | 102% |
| Class C | 0.00% | 0.03% | -0.08% | -0.01% | 97% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.04% | 0.01% | 99% | 100% |
| Class D | 0.00% | 0.14% | -0.08% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | -0.08% | -0.01% | 100% | 102% |
|  |  |  | Low delay B |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.03% | 0.06% | 0.26% | 0.01% | 98% | 100% |
| Class C | -0.06% | -0.15% | -0.12% | -0.08% | 101% | 104% |
| Class E | -0.04% | 0.47% | -0.08% | 0.01% | 100% | 105% |
| Overall | -0.04% | 0.09% | 0.05% | -0.02% | 100% | 103% |
| Class D | -0.06% | -0.18% | -0.42% | -0.11% | 97% | 99% |
| Class F | -0.06% | 0.49% | 0.37% | 0.04% | 102% | 102% |
|  |  |  | Low delay P |  |  |  |
|  |  |  | Over VTM-8.0 |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.01% | -0.15% | 0.20% | 0.01% | 99% | 102% |
| Class C | 0.01% | -0.29% | -0.10% | -0.03% | 100% | 102% |
| Class E | 0.00% | -0.64% | -0.89% | -0.15% | 94% | 99% |
| Overall | 0.01% | -0.32% | -0.17% | -0.04% | 98% | 101% |
| Class D | -0.04% | -0.41% | -0.01% | -0.07% | 94% | 98% |
| Class F | -0.05% | -0.32% | -0.54% | -0.12% | 97% | 100% |

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).