| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0293-v2 |
| --- | --- |

| Title: | Fixes for 6-tap interpolation filtering for affine motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Ruoyang, Z. Zhang, J. Ström | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

It is asserted that the some of the current 6-tap interpolation filters for affine motion compensation have significant phase misalignments compared to the expected phases of the interpolation filters. This contribution proposes modifications of the filters for the fractional positions 4/16, 5/16, 6/16, 10/16, 11/16 and 12/16, and it is claimed that this fixes the phase misalignments of the current filters.

The modifications were reported to be tested under VTM-8.0 CTC. The impact on coding efficiency is reported to be -0.01%/-0.09% for RA/LDB. The modifications were reported to also be tested for the non-CTC) affine test set which was used to determine the performance of proposals in the past affine CEs. The impact on coding efficiency on that test set is reported to be -0.20% for RA.

# Introduction

Some of the current 6-tap interpolation filters have significant issues with phase alignment which can be seen in the below plots of phase delay and magnitude of frequency responses for fractional positions {4/16, 5/16, 6/16}. Here the green curves show the ideal phase delay or magnitude response and the red curves show the phase delay or magnitude response for the current filters. It can be seen that the current phase delay responses have significant deviations from the ideal phase delay responses, especially at low frequencies. It is asserted that such deviations at low frequencies are undesirable and should be fixed.

Figure 1: Plots of phase delay and magnitude of frequency response of 6-tap interpolation filters for fractional positions 4/16, 5/16 and 6/16.

# Proposal

The following modifications of the VVC specification are suggested, high-lighted in yellow:

Table 30 – Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p for affine motion mode

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | 0 | 1 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | 0 | 2 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | 0 | 3 | -11 | 59 | 16 | -4 | 1 | 0 |
| 5 | 0 | 3 | -11 | 54 | 23 | -7 | 2 | 0 |
| 6 | 0 | 2 | -9 | 48 | 29 | -8 | 2 | 0 |
| 7 | 0 | 3 | −11 | 45 | 34 | −10 | 3 | 0 |
| 8 | 0 | 3 | −11 | 40 | 40 | −11 | 3 | 0 |
| 9 | 0 | 3 | −10 | 34 | 45 | −11 | 3 | 0 |
| 10 | 0 | 2 | -8 | 29 | 48 | -9 | 2 | 0 |
| 11 | 0 | 2 | -7 | 23 | 54 | -11 | 3 | 0 |
| 12 | 0 | 1 | -4 | 16 | 59 | -11 | 3 | 0 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 2 | 0 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 1 | 0 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

Plots of phase and magnitude of frequency response of the modified 6-tap interpolation filters are shown below. Comparing to the phase responses in Figure 1, the suggested interpolation filters have smaller phase differences from the ideal phases, especially at the low frequencies.

Figure 3: Plots of phase delay and magnitude of frequency response of proposed 6-tap interpolation filters for fractional positions 4/16, 5/16 and 6/16.

# Results

The modifications were implemented in VTM-8.0 [2] and simulations were run under the VTM-8.0 CTC [3]. The objective results show a small benefit in coding efficiency. Encoding time is measured on simulations run on a cluster with machines with different capability and is not reliable. Decoding time is measured by running test and anchor on same machine without yuv output. Decoding time is similar to that of the anchor.

It should be noted that for affine-friendly sequences in the CTC, such as Cactus, the impact is -0.05% for RA and -0.14% for LDB. It can also be seen that the objective improvement for RA is better for the affine test set that was used in previous CEs, -0.20%.

## CTC

|  | Random access Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -0,02% | 0,06% | 102% | 100% |
| Class A2 | -0,04% | -0,03% | 0,07% | 102% | 100% |
| Class B | -0,02% | -0,04% | -0,08% | 89% | 101% |
| Class C | 0,01% | 0,05% | -0,12% | 84% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | -0,01% | -0,03% | 92% | 100% |
| Class D | 0,05% | 0,19% | -0,01% | 83% | 98% |
| Class F | 0,01% | -0,03% | -0,07% | 86% | 100% |

|  | Low-delay B Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,06% | 0,15% | -0,08% | 101% | 100% |
| Class C | -0,13% | -0,16% | -0,10% | 98% | 100% |
| Class E | -0,09% | 0,25% | -0,03% | 91% | 102% |
| Overall | -0,09% | 0,07% | -0,07% | 98% | 100% |
| Class D | 0,00% | -0,89% | 0,15% | 84% | 100% |
| Class F | -0,13% | 0,03% | -0,13% | 94% | 102% |

In combination with modification of the 4-tap interpolation filters from JVET-R0292 [4]:

|  | Random access Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,02% | -0,02% | -0,11% | 112% | 98% |
| Class A2 | -0,05% | -0,03% | -0,08% | 113% | 100% |
| Class B | -0,03% | -0,15% | -0,18% | 102% | 101% |
| Class C | -0,05% | -0,16% | -0,26% | 93% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,04% | -0,11% | -0,17% | 103% | 100% |
| Class D | 0,04% | 0,13% | -0,10% | 91% | 98% |
| Class F | -0,01% | -0,18% | -0,08% | 96% | 101% |

|  | Low-delay B Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,04% | -0,13% | -0,38% | 114% | 100% |
| Class C | -0,13% | -0,60% | -0,20% | 111% | 101% |
| Class E | -0,01% | -0,80% | -0,24% | 104% | 98% |
| Overall | -0,06% | -0,46% | -0,29% | 110% | 100% |
| Class D | -0,02% | -0,83% | -0,18% | 93% | 98% |
| Class F | -0,08% | 0,27% | -0,43% | 105% | 102% |

## Affine test set (non-CTC)

The modifications were further tested for the affine test which was used in the past affine CEs. The affine test set is 100 frames sequences from:

ftp://jvet@ftp.ient.rwth-aachen.de/testsequences/candidates/huawei/Affine/

Encoding time is measured on simulations run on a cluster with machines with different capability and is not reliable. . Decoding time is measured by running test and anchor on same machine without yuv output. Decoding time is similar as for the anchor.

|  | Random access Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| JetsPart_1280x720_25 | -0,13% | -0,63% | 0,07% | 98% | 104% |
| SpincalendarPart_1280x720_50 | -0,44% | -0,61% | -0,24% | 102% | 101% |
| BlueSkyPart_1920x1080_25 | -0,08% | -0,01% | 0,00% | 98% | 112% |
| ShieldsPart_1920x1080_50 | -0,18% | -2,01% | -0,62% | 99% | 97% |
| StationKtaPart_1920x1080_25 | -0,15% | -0,34% | -0,17% | 98% | 103% |
| TractorPart_1920x1080_25 | -0,21% | 0,00% | -0,04% | 97% | 96% |
| Overall | -0,20% | -0,60% | -0,17% | 99% | 100% |

In combination with modification of the 4-tap interpolation filters from JVET-R0292 [4]:

|  | Random access Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| JetsPart_1280x720_25 | -0,20% | -0,36% | -1,08% | 107% | 101% |
| SpincalendarPart_1280x720_50 | -0,44% | -0,32% | 0,02% | 108% | 98% |
| BlueSkyPart_1920x1080_25 | -0,04% | -0,14% | -0,30% | 107% | 95% |
| ShieldsPart_1920x1080_50 | -0,45% | -1,75% | 0,20% | 111% | 91% |
| StationKtaPart_1920x1080_25 | -0,32% | 0,19% | -0,14% | 115% | 101% |
| TractorPart_1920x1080_25 | -0,23% | -0,08% | -0,15% | 111% | 98% |
| Overall | -0,28% | -0,41% | -0,24% | 110% | 100% |

# Conclusions

# It has been shown that the phase delay responses of some of the current 6-tap interpolation filters have significant deviations from the ideal phase delay responses.

We propose to adopt the fixes of filter coefficients for the 6-tap interpolation filters for fractional sample positions 4/16, 5/16, 6/16, 10/16, 11/16 and 12/16 to VVC specification and software such that VVC has 6-tap interpolation filters for affine motion compensation with proper interpolation properties.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 8)", Document of Joint Video Experts Team, JVET-Q2001.

- VTM-8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010

- K.Andersson, R. Yu, Z. Zhang, J. Ström, “Fixes for 4-tap interpolation filtering”, April 2020, JVET-R0292

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).