| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0294 |
| --- | --- |

| Title: | AHG12: On subpicture extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Robert Skupin Yago Sanchez Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the subpicture extraction process:

- rewriting of conformance window

- replace decoded picture hash SEI messages

- bug fixes for subpicture width and height derivation and derivation of the subpicture index

The contribution also proposed to disallow subpictures to be located completely outside the conformance window.

# Proposed additions to the rewriting process

## Conformance window

The rewriting process currently specified in section C.7 is missing rewriting operations for the conformance window related parameters in the SPS and in the PPS.

For the conformance windows, it is proposed to copy all offset values that cross the subpicture to be extracted. In the example in Figure 1 for subpicture #0 the top and left offsets would be used in the rewritten SPS and PPS. If a subpicture lies completely inside the conformance window, no conformance window shall be signalled.

Figure 1 Four subpictures with conformance window (shown in red)

A special corner case is when a subpicture lies completely outside the conformance window. Possible solutions are discussed in the following:

- It is not allowed to specify a conformance window that results in no samples being output, hence, the conformance window could be disabled in the extracted bitstream. But this would result in samples inside a conformance window in the extracted bitstream, which were not inside the conformance window in the original bitstream.

- Disallow extraction of a subpicture that does not cover samples inside the conformance window, but it seems questionable, why such a subpicture would be defined in the first place.

- Disallow specifying a conformance window that results in a subpicture completely outside the conformance window as a bitstream constraint.

The authors could not find a valid use case, where specifying a subpicture outside the conformance window would be desirable. So the authors propose to disallow subpictures that do not cover samples inside the conformance window.

## Decoded picture hash SEI message

Special handling of decoded picture hash SEI messages is required as an extracted bitstream cannot conform to the hash of the original bitstream. The following is proposed:

- Decoded picture hash SEI messages are removed during extraction

- Decoded picture hash SEI messages are allowed to be nested inside of scalable nesting SEI messages, if subpicture nesting is signalled in the scalable nesting SEI message

- Decoded pictures hash SEI messages that are nested in a scalable nesting SEI message and associated with subpicId are extracted into the output bitstream

Specification text for these changes is provided below.

Alternatively, the decode picture hash SEI message could be extended with hashes for each subpicture.

# Proposed bug fixes

## Subpicture index

In issue #903, it was reported that SubpicIdList is missing. It is proposed to derive subpicIdx similar to CurrSubpicIdx for each slice, right after the definition of subpicId:

The variable subpicIdx is derived to be such that SubpicIdVal[ CurrSubpicIdx ] is equal to subpicId.

Then SubpicIdList[ subPicId ]  is replaced with subpicIdx.

## Picture size rewriting

A bug in the assignment of the picture size was found and also independently reported in issued #903.

The rewriting process contains the following text:

Rewrite the value of the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in all referred to SPS NAL units and the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all referred to PPS NAL units to be equal to subpic_width_minus1[ SubpicIdList[ subPicId ] ] and subpic_height_minus1[ SubpicIdList[ subPicId  ] ].

It was noted that subpic_width_minus1 and subpic_height_minus1 are in units of CTBs, while the other syntax elements are in units of luma samples.

It is proposed to derive the subpicture width and height as follows:

The variables subpicWidthInLumaSamples and subpicHeightInLumaSamples are derived as follows:

subpicWidthInLumaSamples = min( (subpic_ctu_top_left_x[ subpicIdx ] + subpic_width_minus1[ subpicIdx ] + 1 ) * CtbSizeY , pic_width_in_luma_samples) −  (subpic_ctu_top_left_x[ subpicIdx ] * CtbSizeY)

subpicHeightInLumaSamples = min( (subpic_ctu_top_left_y[ subpicIdx ] + subpic_height_minus1[ subpicIdx ] + 1 ) * CtbSizeY , pic_height_in_luma_samples) −  (subpic_ctu_top_left_y[ subpicIdx ] * CtbSizeY)

The these are used in the rewriting process as follows:

Rewrite the value of the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in all referred to SPS NAL units and the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all referred to PPS NAL units to be equal to subpicWidthInLumaSamples and subpicHeightInLumaSamples.

# Specification text for all changes

In the following changes related to cropping window are highlighted in green, changes related to bug fixes are marked in cyan, changes related to extraction of decoded picture hash SEI messages in purple.

Semantics of cropping window (SPS):

It is a requirement of bitstream conformance that for any value i in the range of 0 to sps_num_subpics_minus1 inclusive all of the following conditions are satisfied:

- ( subpic_ctu_top_left_x[ i ] * CtbSizeY ) shall be less than pic_width_max_in_luma_samples − ( SubWidthC * sps_conf_win_right_offset )

- ( subpic_ctu_top_left_y[ i ] * CtbSizeY ) shall be less than pic_height_max_in_luma_samples − ( SubHeightC * sps_conf_win_bottom_offset )

- ( ( subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] ) * CtbSizeY ) shall be greater than ( SubWidthC * sps_conf_win_left_offset )

- ( ( subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] ) * CtbSizeY ) shall greater than ( SubHeightC * sps_conf_win_top_offset )

Semantics of cropping window (PPS):

It is a requirement of bitstream conformance that for any value i in the range of 0 to sps_num_subpics_minus1 inclusive all of the floowing conditions are satisfied:

- ( subpic_ctu_top_left_x[ i ] * CtbSizeY ) shall be less than pic_width_in_luma_samples − ( SubWidthC * pps_conf_win_right_offset )

- ( subpic_ctu_top_left_y[ i ] * CtbSizeY ) shall be less than pic_height_in_luma_samples − ( SubHeightC * pps_conf_win_bottom_offset )

- ( ( subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ] ) * CtbSizeY ) shall be greater than ( SubWidthC * pps_conf_win_left_offset )

- ( ( subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] ) * CtbSizeY ) shall greater than ( SubHeightC * pps_conf_win_top_offset )

Subpicture extraction process (C.7):

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, a target highest TemporalId value tIdTarget, and an array of target subpicture ID values for each layer subpicIdTarget[ ].

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and subpicIdTarget[ ] equal to subpicture IDs present in the OLS, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ] and with slice_subpic_id equal to the value in subpicIdTarget[ i ] for each i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1.

The output sub-bitstream outBitstream is derived as follows:

–	The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget as inputs and the output of the process is assigned to outBitstream.

–	If some external means not specified in this Specification is available to provide replacement parameter sets for the sub-bitstream outBitstream, replace all parameter sets with the replacement parameter sets.

–	Otherwise, when subpicture level information SEI messages are present in inBitstream, the following applies:

–	Rewrite the value of general_level_idc in the ols_ptl_idx[ targetOlsIdx ]-th entry in the list of profile_tier_level() syntax structures in all referred to VPS NAL units to be equal to SubpicSetLevelIdc derived in D.3.8 for the set of subpictures consisting of the subpictures with subpicture ID equal to all entries in subpicIdTarget[ ].

–	Rewrite the values of cpb_size_value_minus1[ tIdTarget ][ j ] and bit_rate_value_minus1[ tIdTarget ][ j ] of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to VPS NAL units to correspond to SubpicSetCpbSizeVcl[ 0 ], SubpicSetCpbSizeNal[ 0 ], SubpicSetBitrateVcl[ 0 ], and SubpicSetBitrateNal[ 0 ] as derived in D.3.8 for the set of subpictures consisting of the subpictures with subpicture ID equal to all entries in subpicIdTarget[ ] and j in the range of 0 to hrd_cpb_cnt_minus1. [Ed. (RS): Precise values of BitRate are required to set the HRD parameters correctly.]

For the i-th layer with i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, the following applies.

–	subpicId is set equal to the value of subpicIdTarget[ i ].

–	The variable subpicIdx is derived to be such that SubpicIdVal[ CurrSubpicIdx ] is equal to subpicId.

–	Rewrite the value of general_level_idc in the profile_tier_level() syntax structure in all referred to SPS NAL units with sps_ptl_dpb_hrd_params_present_flag equal to 1 to be equal to SubpicSetLevelIdc derived in D.3.8 for the set of subpictures consisting of the subpicture with subpicture ID equal to subpicId.

–	Rewrite the values of cpb_size_value_minus1[ tIdTarget ][ j ] and bit_rate_value_minus1[ tIdTarget ][ j ] of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to SPS NAL units to correspond to SubpicCpbSizeVcl[ 0 ][ subpicIdxSubpicIdList[ subPicId ] ], SubpicCpbSizeNal[ 0 ][ subpicIdxSubpicIdList[ subPicId ] ], SubpicBitrateVcl[ 0 ][ subpicIdxSubpicIdList[ subPicId ] ], and SubpicBitrateNal[ 0 ][ subpicIdxSubpicIdList[ subPicId ] ] as derived in D.3.8 for the subpicture with subpicture ID equal to subpicId and j in the range of 0 to hrd_cpb_cnt_minus1.

–	The variables subpicWidthInLumaSamples and subpicHeightInLumaSamples are derived as follows:

subpicWidthInLumaSamples = min( (subpic_ctu_top_left_x[ subpicIdx ] + subpic_width_minus1[ subpicIdx ] + 1 ) * CtbSizeY , pic_width_in_luma_samples) − (subpic_ctu_top_left_x[ subpicIdx ] * CtbSizeY)

subpicHeightInLumaSamples = min( (subpic_ctu_top_left_y[ subpicIdx ] + subpic_height_minus1[ subpicIdx ] + 1 ) * CtbSizeY , pic_height_in_luma_samples) − (subpic_ctu_top_left_y[ subpicIdx ] * CtbSizeY)

–	Rewrite the value of the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in all referred to SPS NAL units and the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all referred to PPS NAL units to be equal to subpicWidthInLumaSamples subpic_width_minus1[ SubpicIdList[ subPicId ] ] and subpicHeightInLumaSamples subpic_height_minus1[ SubpicIdList[ subPicId  ] ].

–	Rewrite the value of sps_num_subpics_minus1 in all referenced SPS NAL units and pps_num_subpics_minus1 in all referenced PPS NAL units to 0.

–	Rewrite the syntax elements subpic_ctu_top_left_x[ subpicIdxSubpicIdList[ subPicId ] ], subpic_ctu_top_left_y[ subpicIdxSubpicIdList[ subPicId ] ], when present, in all referred to SPS NAL units to 0.

–	Remove the syntax elements subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ], subpic_treated_as_pic_flag[ j ], loop_filter_across_subpic_enabled_flag[ j ], and sps_subpic_id[ j ] in all referred to SPS NAL units and for each j for which SubpicIdList[ j ] is not equal to subPicIdx.

–	Rewrite the tiling and slice structure related syntax elements in all referred to PPS to remove all tiles rows, tile columns and slices that are not associated to the subpicture with a subpicture ID equal to subPicId.

– Rewrite the syntax elements sps_conformance_window_flag, sps_conf_win_right_offset, sps_conf_win_top_offset , sps_conf_win_bottom_offset and sps_conf_win_left_offset in all referred to SPS and pps_conformance_window_flag, pps_conf_win_right_offset, pps_conf_win_top_offset , pps_conf_win_bottom_offset and pps_conf_win_left_offset in all referred to PPS so that the same part of the subpicture with subpicture ID equal to subPicId lies within the conformance window as in the input bitstream inBistream.

–	Remove from outBitstream all VCL NAL units with nuh_layer_id equal to nuh_layer_id of the i-th layer and with slice_subpic_id not equal to subPicId.

–	When sli_cbr_constraint_flag is equal to 1, remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages that are not associated with the VCL NAL units of a subpicture in subpicIdTarget[ ] and set cbr_flag[ tIdTarget ][ j ] equal to 1 of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to VPS NAL units and SPS NAL units and j in the range of 0 to hrd_cpb_cnt_minus1. Otherwise, (sli_cbr_constraint_flag is equal to 0), remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages and set cbr_flag[ tIdTarget ][ j ] equal to 0.

–	When outBitstream contains SEI NAL units that contain a scalable nesting SEI message with nesting_ols_flag equal to 1 and nesting_subpic_flag equal to 1 that are applicable to outBitstream, extract appropriate non-scalable-nested SEI message with payloadType equal to 1 (picture timing), or 130 (decoding unit information), or 132 (decoded picture hash) from the scalable nesting SEI message and place the extracted SEI messages into outBitstream.

Semantics of Scalable Nesting SEI

–	An SEI message that has payloadType equal to 132 (decoded picture hash) shall only be contained in a scalable nesting SEI message, when nesting_subpic_flag is equal to 1.

–	 An SEI message that has payloadType equal to 133 (scalable nesting) shall not be contained in a scalable nesting SEI message.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).