| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0295-v1 |
| --- | --- |

| Title: | AHG12: On subpicture conformance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes several changes to the subpicture conformance related specification sections.

- The subpicture bitrate (SubpicBitRateVcl[ i ][ j ] and SubpicBitRateNal[ i ][ j ]) used in the bitstream extraction for conformance testing are based on the maximum bitrate of the first indicated reference level. This document asserts that the derivation is not correct and would lead to corrupt subpicture HRD parameters. For instance, given a large initial CPB removal delay, a large bitrate could easily overflow the CPB. Therefore, it is proposed to use the actual bitrate indicated in the bitstream for derivation of the subpicture bitrate in Section D.7.2.

- At the last meeting it was decided to constrain subpictures in multi-layer bitstreams to be aligned in terms of boundary position and subpicture ID across layers. Corresponding changes to the subpicture extraction process Annex C.7 are proposed as an asserted simplification.

- A bug in handling the sli_cbr_constraint_flag in the subpicture extraction process in Annex C.7 is reported and an asserted bug fix is proposed alongside several minor fixes (e.g. #903) and editorial improvements in Annex C.6 and C.7.

# Introduction

## Subpicture bitrate derivation

At the last JVET meeting, it was decided to enable conformance testing of subpictures, i.e. extracted subpictures are augmented with a set of specific HRD parameters and HRD-related SEI messages in an extraction process in the course of conformance testing.

Necessary additions to the subpicture level information SEI message from JVET-Q0395 included derivation of a subpicture-specific bitrate which is as follows:

The variables SubpicBitRateVcl[ i ][ j ] and SubpicBitRateNal[ i ][ j ] are derived as follows:

SubpicBitRateVcl[ i ][ j ] = Floor( CpbVclFactor * MaxBR * RefLevelFraction[ i ][ j ] ÷ 256)	(D.7)

SubpicBitRateNal[ i ][ j ] = Floor( CpbNalFactor * MaxBR * RefLevelFraction[ i ][ j ] ÷ 256)	(D.8)

Note that the same reference bitrate MaxBR of the reference level is used, similar to how MaxCPB is used as reference for deriving the CpbSize for the subpicture.

The issue is that while a too large CpbSize does not do harm to in the conformance testing, a large bitrate can easily lead to buffer overflows when, for instance, a large initial CPB removal delay is active. The issue is also pointed out in an editor’s note in section Annex C.7.

[Ed. (RS): Precise values of BitRate are required to set the HRD parameters correctly.]

Therefore, as proposed in from JVET-Q0395, it is proposed again to derive the variables SubpicBitRateVcl and SubpicBitRateNal in the subpicture level information SEI message from the actual bitrate indicated in the general_hrd_parameters( ) of the bitstream and ols_hrd_parameters( ) of the OLS.

## Subpicture alignment constraints

At the last meeting it was decided to constrain subpictures in multi-layer bitstreams to be aligned in terms of boundary position and subpicture ID across layers. This allows to simplify the subpicture extraction process Annex C.7 and corresponding changes are present in specification text in Section 2 below. Further minor fixes to the specification text in Annex C.6 and C.7 are proposed.

## Subpicture CBR constraint handling

The sli_cbr_constraint_flag in the subpicture level information SEI message is used in the subpicture extraction process Annex C.7 to treat filler data NAL units according to the value of sli_cbr_constraint_flag. A bug in the process rewrites the cbr_flag[ tIdTarget ][ j ] to 1 for all CPBs when sli_cbr_constraint_flag is equal to 1. It is proposed to remove the related text.

# Proposal

The following changes are proposed to Section D.7.2 Subpicture level information SEI message semantics.

ref_level_idc[ i ] indicates a level to which each subpicture conforms as specified in Annex A. Bitstreams shall not contain values of ref_level_idc other than those specified in Annex A. Other values of ref_level_idc[ i ] are reserved for future use by ITU-T | ISO/IEC. It is a requirement of bitstream conformance that the value of ref_level_idc[ 0 ] shall be equal to the value of general_level_idc of the bitstream and that the value of ref_level_idc[ i ] shall be less than or equal to ref_level_idc[ k ] for any value of k greater than i and i greater than 0.

[…]

The variables SubpicBitRateVcl[ i ][ j ] and SubpicBitRateNal[ i ][ j ] are derived as follows:

SubpicBitRateVcl[ i ][ j ] = Floor( CpbVclFactor * MaxValBR * RefLevelFraction[ 0i ][ j ] ÷ 256)	(D.7)

SubpicBitRateNal[ i ][ j ] = Floor( CpbNalFactor * MaxValBR * RefLevelFraction[ 0i ][ j ] ÷ 256)	(D.8)

Where the value of ValBR is derived as follows:

–	When bit_rate_value_minus1[ Htid ][ ScIdx ] is available in the respective HRD parameters in VPS or SPS, ValBR is set equal to ( bit_rate_value_minus1[ Htid ][ ScIdx ] + 1) * 2( 6 + bit_rate_scale ) where Htid is the considered index of temporal sublayer and ScIdx is the index of the considered scheduler.

–	Otherwise, ValBR is set equal to MaxBR of ref_level_idc[ 0 ].

[…]

The value of the subpicture set sequence level indicator, SubpicSetLevelIdc, is derived as follows:

SubpicSetLevelIdc = general_level_idc
SubpicSetLevelIdx = 0
for (i = num_ref_level_minus1; i  >=  01; i− − ) {
	if( SubpicSetNumTiles[ i ]  <=  ( MaxTileCols * MaxTileRows )  &&	(D.10)
			SubpicSetAccLevelFraction[ i ]  <=  256 ){
		SubpicSetLevelIdc = ref_level_idc[ i ]
		SubpicSetLevelIdx = i
	}
}

where MaxTileCols and MaxTileRows are specified in Table A.1 for ref_level_idc[ i ].

C.6	Sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, and a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and tIdTarget equal to any value in the range of 0 to 6, inclusive, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DCI_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	Remove from outBitstream all NAL units for which all of the following conditions are true:

–	nal_unit_type is not equal to IDR_W_RADL, IDR_N_LP, or CRA_NUT.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ j ].

–	Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has nesting_ols_flag equal to 1 and there is no value of i in the range of 0 to nesting_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx.

–	When LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all NAL units in the bitstream, the following applies:

–	Remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 0 (buffering period) or 130 (decoding unit information).

–	When general_same_pic_timing_in_all_ols_flag is equal to 0, remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 1 (picture timing).

–	When outBitstream contains SEI NAL units that contain a scalable nesting SEI message with nesting_ols_flag equal to 1 and are applicable to outBitstream (NestingOlsIdx[ i ] is equal to targetOlsIdx), the following applies:

–	If general_same_pic_timing_in_all_ols_flagsame_pic_timing_within_ols_flag is equal to 0, extract appropriate non-scalable-nested SEI message with payloadType equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information) from the scalable nesting SEI message and include those SEI messages in outBitstream.

–	Otherwise (general_same_pic_timing_in_all_ols_flagsame_pic_timing_within_ols_flag is equal to 1), extract appropriate non-scalable-nested SEI message with payloadType equal to 0 (buffering period) or 130 (decoding unit information) from the scalable nesting SEI message and include those SEI messages in outBitstream.

C.7	Subpicture sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, a target highest TemporalId value tIdTarget, and an array of a target subpicture ID values for each layer subpicIdTarget[ ].

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and subpicIdTarget[ ] equal to subpicture IDs present in the OLS, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ] and with slice_subpic_id equal to the value in subpicIdTarget[ i ] for each i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1.

The output sub-bitstream outBitstream is derived as follows:

–	The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget as inputs and the output of the process is assigned to outBitstream.

–	If some external means not specified in this Specification is available to provide replacement parameter sets for the sub-bitstream outBitstream, replace all parameter sets with the replacement parameter sets.

–	Otherwise, when subpicture level information SEI messages are present in inBitstream, the following applies:

–	The variable subpicIdx is derived to be such that SubpicIdVal[ CurrSubpicIdx ] is equal to subpicIdTarget.

–	Rewrite the value of general_level_idc in the ols_ptl_idx[ targetOlsIdx ]-th entry in the list of profile_tier_level() syntax structures in all referred to VPS NAL units to be equal to SubpicSetLevelIdc derived in Equation D.3.810 for the set of subpictures consisting of the subpictures with subpicture ID equal to all entries in subpicIdTarget[ ].

–	When VCL HRD parameters or NAL HRD parameter are present, Rrewrite the respective values of  cpb_size_value_minus1[ tIdTarget ][ j ] and bit_rate_value_minus1[ tIdTarget ][ j ] of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to VPS NAL units and in the ols_hrd_parameters() syntax structures in all referred to SPS NAL units by the i-th layer to correspond to SubpicSetCpbSizeVcl[ 0 ], SubpicSetCpbSizeNal[ 0 ], SubpicSetBitrateVcl[ 0 ], and SubpicSetBitrateNal[ 0 ] SubpicCpbSizeVcl[ SubpicSetLevelIdx ][ subpicIdx ], SubpicCpbSizeNal[ SubpicSetLevelIdx ][ subpicIdx ], SubpicBitrateVcl[ SubpicSetLevelIdx ][ subpicIdx ]  and SubpicBitrateNal[ SubpicSetLevelIdx ][ subpicIdx ] as derived in Equation D.3.87 and D.8 and with SubpicSetLevelIdx as derived in Equation C.10 for the set of subpictures consisting of the subpictures with subpicture ID equal to all entries in subpicIdTarget[ ] and j in the range of 0 to hrd_cpb_cnt_minus1 and with i in the range of 0 to NumLayersInOls[ targetOlsIdx ] – 1.

For the i-th layer with i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, the following applies.

–	subpicId is set equal to the value of subpicIdTarget[ i ].

–	Rewrite the value of general_level_idc in the profile_tier_level() syntax structure in all referred to SPS NAL units with sps_ptl_dpb_hrd_params_present_flag equal to 1 to be equal to SubpicSetLevelIdc derived in Equation D.3.810 for the set of subpictures consisting of the subpicture with subpicture ID equal to subpicIdTarget.

–	Rewrite the values of cpb_size_value_minus1[ tIdTarget ][ j ] and bit_rate_value_minus1[ tIdTarget ][ j ] of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to SPS NAL units by the layer to correspond to SubpicCpbSizeVcl[ 0 ][ SubpicIdList[ subPicId ] ], SubpicCpbSizeNal[ 0 ][ SubpicIdList[ subPicId ] ], SubpicBitrateVcl[ 0 ][ SubpicIdList[ subPicId ] ], and SubpicBitrateNal[ 0 ][ SubpicIdList[ subPicId ] ] as derived in D.3.8 and with SubpicSetLevelIdx as derived in C.10 for the subpicture with subpicture ID equal to subpicId and j in the range of 0 to hrd_cpb_cnt_minus1.

–	Rewrite the value of the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in all referred to SPS NAL units and the value of pic_width_in_luma_samples and pic_height_in_luma_samples in all referred to PPS NAL units to be equal to subpic_width_minus1[ SubpicIdList[ subPicId subpicIdx ] ] and subpic_height_minus1[ SubpicIdList[ subPicId subpicIdx ] ].

–	Rewrite the value of sps_num_subpics_minus1 in all referenced SPS NAL units and pps_num_subpics_minus1 in all referenced PPS NAL units to 0.

–	Rewrite the syntax elements subpic_ctu_top_left_x[ SubpicIdList[ subPicId subpicIdx ] ], subpic_ctu_top_left_y[ [ SubpicIdList[ subPicId subpicIdx ] ], when present, in all referred to SPS NAL units to 0.

–	Remove the syntax elements subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ], subpic_treated_as_pic_flag[ j ], loop_filter_across_subpic_enabled_flag[ j ], and sps_subpic_id[ j ] in all referred to SPS NAL units and for each j for which SubpicIdList[ j ] is not equal to subPicId subpicIdx.

–	Rewrite the tiling and slice structure related syntax elements in all referred to PPS to remove all tiles rows, tile columns and slices that are not associated to the subpicture with a subpicture ID equal to subPicIdsubpicIdTarget .

–	Remove from outBitstream all VCL NAL units with nuh_layer_id equal to nuh_layer_id of the i-th layer and with slice_subpic_id not equal to subPicIdsubpicIdTarget .

–	When sli_cbr_constraint_flag is equal to 1, remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages that are not associated with the VCL NAL units of a subpicture in subpicIdTarget[ ] and set cbr_flag[ tIdTarget ][ j ] equal to 1 of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to VPS NAL units and SPS NAL units and j in the range of 0 to hrd_cpb_cnt_minus1. Otherwise, (sli_cbr_constraint_flag is equal to 0), remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages and set cbr_flag[ tIdTarget ][ j ] equal to 0 of the j-th CPB in the ols_hrd_idx[ targetOlsIdx ]-th entry in the list of ols_hrd_parameters() syntax structures in all referred to VPS NAL units and SPS NAL units by the i-th layer and for j in the range of 0 to hrd_cpb_cnt_minus1 and with i in the range of 0 to NumLayersInOls[ targetOlsIdx ] – 1.

–	When outBitstream contains SEI NAL units that contain a scalable nesting SEI message with nesting_ols_flag equal to 1 and nesting_subpic_flag equal to 1 that are applicable to outBitstream, extract appropriate non-scalable-nested SEI message with payloadType equal to 1 ( picture timing), or 130 (decoding unit information) from the scalable nesting SEI message and place the extracted SEI messages into outBitstream.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)