| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0296-v1 |
| --- | --- |

| Title: | AHG9: On sublayer references |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution asserts that the semantics of max_tid_il_ref_pics_plus1 for a special mode indication in which only IRAP pictures are used as inter-layer reference of the current layer are not aligned with the meeting decision and proposes an asserted alignment. Furthermore, derivation of the variable NumSubLayersInLayerInOLS is asserted to be incorrect for the case that each_layer_is_an_ols_flag is equal to 1 and an asserted fix is proposed.

# Semantics of max_tid_il_ref_pics_plus1

At the last JVET meeting, allowing layer references to be sub-layer specific was adopted. As the notes or Q0398 state, a special mode indicates that only IRAP pictures of the reference layer are used for inter-layer prediction:

The contribution had been revised with text such that when there is a dependent layer, there is an indication of the max_tid_il_ref_pics_plus1 that the layer depends on, and if that value is 0, inter-layer prediction uses only IRAP pictures.

Decision (cleanup): Adopt, using the max_tid_il_ref_pics_plus1 syntax approach. Text was provided in a revision of the contribution.

In Draft 8, however, the current semantics of the max_tid_il_ref_pics_plus1 indicate a different relationship, namely “that inter-layer prediction is not used by non-IRAP pictures of the i-th layer”. It is therefore, proposed to change the semantics as follows to align with the decision at the last meeting.

max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that inter-layer prediction is not used by no non-IRAP pictures are used as ILRP for decoding pictures of the i-th layer. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that, for decoding pictures of the i-th layer, no picture with TemporalId greater than max_tid_il_ref_pics_plus1[ i ] − 1 is used as ILRP. When not present, the value of max_tid_il_ref_pics_plus1[ i ] is inferred to be equal to 7.

# Derivation of NumSubLayersInLayerInOLS

In the semantics of ols_output_layer_flag, the variable NumSubLayersInLayerInOLS is derived in Equation 40 as follows:

NumOutputLayersInOls[ 0 ] = 1
OutputLayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
NumSubLayersInLayerInOLS[ 0 ][ 0 ] = vps_max_sub_layers_minus1 + 1
[…]
for( i = 1; i < TotalNumOlss; i++ )
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc  = =  0 ) {
		NumOutputLayersInOls[ i ] = 1
		OutputLayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]
		for( j = 0; j < i  &&  ( ols_mode_idc  = =  0 ); j++ )
			NumSubLayersInLayerInOLS[ i ][ j ] = max_tid_il_ref_pics_plus1[ i ]
		NumSubLayersInLayerInOLS[ i ][ i ] = vps_max_sub_layers_minus1 + 1
	} else if( ols_mode_idc  = =  1 ) {
[…]
	} else if( ols_mode_idc  = =  2 ) {
[…]

For the case that each_layer_is_an_ols_flag is equal to 1, ols_mode_idc is neither signaled nor inferred and therefore, the highlighted condition is broken. In this case, the highlighted index i also seems to be wrong and should be equal to 0, since the OLS would only have one layer. It is proposed to separate the two cases for each_layer_is_an_ols_flag equal to 1 and ols_mode_idc equal to 0 in Equation 40 as follows:

NumOutputLayersInOls[ 0 ] = 1
OutputLayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
NumSubLayersInLayerInOLS[ 0 ][ 0 ] = vps_max_sub_layers_minus1 + 1
[…]
for( i = 1; i < TotalNumOlss; i++ )
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc  = =  0 ) {
		NumOutputLayersInOls[ i ] = 1
		OutputLayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]
		for( j = 0; j < i  &&  ( ols_mode_idc  = =  0 ); j++ )
			NumSubLayersInLayerInOLS[ i ][ j ] = max_tid_il_ref_pics_plus1[ i ]
		NumSubLayersInLayerInOLS[ i ][ 0i ] = vps_max_sub_layers_minus1 + 1
	} else if( ols_mode_idc  = =  0 ) {
		NumOutputLayersInOls[ i ] = 1
		OutputLayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]
		for( j = 0; j < i; j++ )
			NumSubLayersInLayerInOLS[ i ][ j ] = max_tid_il_ref_pics_plus1[ i ]
		NumSubLayersInLayerInOLS[ i ][ i ] = vps_max_sub_layers_minus1 + 1
	} else if( ols_mode_idc  = =  1 ) {
[…]
	} else if( ols_mode_idc  = =  2 ) {
[…]

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)