| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0297-v2 |
| --- | --- |

| Title: | AHG9: HRD bug-fixes and editorial clarifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes several bug-fixes for asserted bugs related to HRD and proposes some editorial clarifications.

# Introduction

This contribution proposes the following aspects:

- Editorial clarification of what n0…n5 mean for the number of conformance tests in Annex C.

- Bug fix for the inference rule for syntax elements in the syntax structure sublayer_hrd_parameters( ) not present. The inference rule is always using sps_max_sublayers_minus1 although the syntax structure sublayer_hrd_parameters( ) might be in the VPS.

- Bug-fix for n3 specifying number of conformance test for IRAPs that are not CRAs with associated RASL pictures and alternative timing.

- Bug fix for derivation of InitialCpbRemovalDelay aligning C.2.3 with derivation in C.2.2 to account for alternative timing.

- Bug-fix adding missing text to update values of CpbDelayOffset and DpbDelayOffset in C.2.3

- Bug-fix in equation C.11, as it was not accounting for CpbDelayOffset.

Version 2 of this document modifies the definition of n3 that defines the number of test bitstreams for conformance including GDR AUs as well as other IRAPs after offline discussion.

# Discussion

- The number of conformance tests in Annex C is specified as n0 * n1 * ( 2 * n2 + n3 + n4 + n5), but as indicated by an editors note, it is not clear what these n values apply to. A clarification note is suggested that provides that information:

  - n0: tests for Type I bitstream conformance and Type II bitstream conformance.

  - n1: tests for each CPB delivery schedule.

  - n2: tests for each bitstream starting at each CRA picture with associated RASL pictures and alternative initial CPB removal delay and delay offset present. The tests are done for once with keeping and once with removing RASL pictures associated with the CRA. Therefore 2*n2.

  - n3: Tests for bitstreams starting at each GDR or IRAP that is not a CRA with associated RASL pictures and alternative initial CPB removal delay and delay offset present.

  - n4: Tests for bitstreams starting with each IRAP with associated DRAP pictures with alternative timing information present and result of removing all access units between a DRAP picture with alternative timing information present and the preceding IRAP.

  - n5: Tests for AU-based conformance and DU-based if indicated by general_decoding_unit_hrd_params_present_flag.

- The inference rule for syntax elements in the syntax structure sublayer_hrd_parameters( ) should use sps_max_sublayers_minus1 when present in the SPS but otherwise it should use hrd_max_tid[ i ] when present in the VPS.

- n3 indicates the tests for IRAPs that are not accounted in n2. However, the conditions are not aligned. n3 needs to reflect that it corresponds to IRAPs that are not “CRAs with associated RALS picture having alternative timing information in the BP SEI message” instead of only that are not “CRAs with associated RASL pictures”.

- As stated by an editor note in C.2.3, the derivation of InitialCpbRemovalDelay is not aligned with C.2.2. and misses the presence of alternative timing information. Same text as in C.2.2. is taken for C.2.3.

- A BP SEI message that is not the first one in the bitstream might have alternative timing and might require update of CpbOffsetDelay and DpbOffsetDelay but these values are only set based on AU 0. This could happen for instance when splicing is performed and in the spliced CVS, RASL picture are removed. Text is added into C.2.3 to update CpbOffsetDelay and DpbOffsetDelay.

- The derivation of the nominal removal time of AUs that are not the first AU in a BP is done following equation C.11. Contrary to C.10, C.11 misses to take into consideration CpbOffsetDelay which is clearly an oversight.

All points listed above are integrated in the specification as shown in section 3.

# Specification Text

#### 7.4.6.3		Sublayer HRD parameters semantics

When a sublayer_hrd_parameters( ) syntax structure is included in the i-th ols_hrd_parameters( ) syntax structure in a VPS, the value of maxSublayersMinus1 is set to hrd_max_tid[ i ]. When a sublayer_hrd_parameters( ) syntax structure is included in the ols_hrd_parameters( ) syntax structure in an SPS, the value of maxSublayersMinus1 is set equal to sps_max_sublayers_minus1.

bit_rate_value_minus1[ i ][ j ] (together with bit_rate_scale) specifies the maximum input bit rate for the j-th CPB with Htid equal to i when the CPB operates at the AU level. bit_rate_value_minus1[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive. For any j greater than 0 and any particular value of i, bit_rate_value_minus1[ i ][ j ] shall be greater than bit_rate_value_minus1[ i ][ j − 1 ].

When DecodingUnitHrdFlag is equal to 0, the following applies:

–	The bit rate in bits per second is given by:

BitRate[ i ][ j ] = ( bit_rate_value_minus1[ i ][ j ] + 1 ) * 2( 6 + bit_rate_scale )	(112)

–	When the bit_rate_value_minus1[ i ][ j ] syntax element is not present, it is inferred as follows:

–	If general_hrd_params_present_flag is equal to 1, bit_rate_value_minus1[ i ][ j ] is inferred to be equal to bit_rate_value_minus1[ maxSublayersMinus1sps_max_sublayers_minus1 ][ j ].

–	Otherwise (general_hrd_params_present_flag is equal to 0), the value of BitRate[ i ][ j ] is inferred to be equal to CpbBrVclFactor * MaxBR for VCL HRD parameters and to be equal to CpbBrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, CpbBrVclFactor and CpbBrNalFactor are specified in Annex A.

cpb_size_value_minus1[ i ][ j ] is used together with cpb_size_scale to specify the j-th CPB size with Htid equal to i when the CPB operates at the AU level. cpb_size_value_minus1[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive. For any j greater than 0 and any particular value of i, cpb_size_value_minus1[ i ][ j ] shall be less than or equal to cpb_size_value_minus1[ i ][ j − 1 ].

When DecodingUnitHrdFlag is equal to 0, the following apples:

–	The CPB size in bits is given by:

CpbSize[ i ][ j ] = ( cpb_size_value_minus1[ i ][ j ] + 1 ) * 2( 4 + cpb_size_scale )	(113)

–	When the cpb_size_value_minus1[ i ][ j ] syntax element is not present, it is inferred as follows:

–	If general_hrd_params_present_flag is equal to 1, cpb_size_value_minus1[ i ][ j ] is inferred to be equal to cpb_size_value_minus1[ maxSublayersMinus1sps_max_sublayers_minus1 ][ j ].

–	Otherwise (general_hrd_params_present_flag is equal to 0), the value of CpbSize[ i ][ j ] is inferred to be equal to CpbBrVclFactor * MaxCPB for VCL HRD parameters and to be equal to CpbBrNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, CpbBrVclFactor and CpbBrNalFactor are specified in Annex A.

cpb_size_du_value_minus1[ i ][ j ] is used together with cpb_size_du_scale to specify the i-th CPB size with Htid equal to i when the CPB operates at DU level. cpb_size_du_value_minus1[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive. For any j greater than 0 and any particular value of i, cpb_size_du_value_minus1[ i ][ j ] shall be less than or equal to cpb_size_du_value_minus1[ i ][ j − 1 ].

When DecodingUnitHrdFlag is equal to 1, the following applies:

–	The CPB size in bits is given by:

CpbSize[ i ][ j ] = ( cpb_size_du_value_minus1[ i ][ j ] + 1 ) * 2( 4 + cpb_size_du_scale )	(114)

–	When the cpb_size_du_value_minus1[ i ][ j ] syntax element is not present, it is inferred as follows:

–	If general_hrd_params_present_flag is equal to 1, cpb_size_du_value_minus1[ i ][ j ] is inferred to be equal to cpb_size_du_value_minus1[ maxSublayersMinus1sps_max_sublayers_minus1 ][ j ].

–	Otherwise (general_hrd_params_present_flag is equal to 0), the value of CpbSize[ i ][ j ] is inferred to be equal to CpbVclFactor * MaxCPB for VCL HRD parameters and to be equal to CpbNalFactor * MaxCPB for NAL HRD parameters, where MaxCPB, CpbVclFactor and CpbNalFactor are specified in Annex A.

bit_rate_du_value_minus1[ i ][ j ] (together with bit_rate_scale) specifies the maximum input bit rate for the j-th CPB with Htid equal to i when the CPB operates at the DU level. bit_rate_du_value_minus1[ i ][ j ] shall be in the range of 0 to 232 − 2, inclusive. For any j greater than 0 and any particular value of i, bit_rate_du_value_minus1[ i ][ j ] shall be greater than bit_rate_du_value_minus1[ i ][ j − 1 ].

When DecodingUnitHrdFlag is equal to 1, the folowing applies:

–	The bit rate in bits per second is given by:

BitRate[ i ][ j ] = ( bit_rate_du_value_minus1[ i ][ j ] + 1 ) * 2( 6 + bit_rate_scale )	(115)

–	When the bit_rate_du_value_minus1[ i ][ j ] syntax element is not present, it is inferred as follows:

–	If general_hrd_params_present_flag is equal to 1, bit_rate_du_value_minus1[ i ][ j ] is inferred to be equal to bit_rate_du_value_minus1[ maxSublayersMinus1sps_max_sublayers_minus1 ][ j ].

–	Otherwise (general_hrd_params_present_flag is equal to 0), the value of BitRate[ i ][ j ] is inferred to be equal to BrVclFactor * MaxBR for VCL HRD parameters and to be equal to BrNalFactor * MaxBR for NAL HRD parameters, where MaxBR, BrVclFactor and BrNalFactor are specified in Annex A.

cbr_flag[ i ][ j ] equal to 0 specifies that to decode this bitstream by the HRD using the j-th CPB specification, the hypothetical stream scheduler (HSS) operates in an intermittent bit rate mode. cbr_flag[ i ][ j ] equal to 1 specifies that the HSS operates in a constant bit rate (CBR) mode.

When not present, the value of cbr_flag[ i ][ j ] it is inferred as follows:

–	When the cbr_flag[ i ][ j ] syntax element is not present, it is inferred as follows:

–	If general_hrd_params_present_flag is equal to 1, cbr_flag[ i ][ j ] is inferred to be equal to cbr_flag[ maxSublayersMinus1sps_max_sublayers_minus1 ][ j ].

–	Otherwise (general_hrd_params_present_flag is equal to 0), the value of cbr_flag[ i ][ j ] is inferred to be equal to 0.

## C.1	General

…

For each test, the following ordered steps apply in the order listed, followed by the processes described after these steps in this clause:

- An operation point under test, denoted as targetOp, is selected by selecting a target OLS with OLS index opOlsIdx, a highest TemporalId value opTid, and optionally, a list of target subpicture ID values opSubpicIdList[ ], one for each layer. The value of opOlsIdx is in the range of 0 to TotalNumOlss − 1, inclusive. The value of opTid is in the range of 0 to vps_max_sublayers_minus1, inclusive.

If opSubpicIdList[ ] is not present, targetOp consists of pictures, and each pair of selected values of opOlsIdx and opTid shall be such that the sub-bitstream that is the output by invoking the sub-bitstream extraction process as specified in clause C.6 with entireBitstream, opOlsIdx, and opTid as inputs satisify the following conditions:

–	There is at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ opOlsIdx ] in BitstreamToDecode.

–	There is at least one VCL NAL unit with TemporalId equal to opTid in BitstreamToDecode.

Otherwise (opSubpicIdList[ ] is  present), targetOp consists of subpictures, and the sub-bitstream that is the output by invoking the subpicture sub-bitstream extraction process as specified in clause C.7 with entireBitstream, opOlsIdx, opTid, and opSubpicIdList[ ] as inputs satisify the following conditions:

–	There is at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ opOlsIdx ] in BitstreamToDecode.

–	There is at least one VCL NAL unit with TemporalId equal to opTid in BitstreamToDecode.

–	There is at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ opOlsIdx ][ j ] and with slice_subpic_id equal to opSubpicIdList[ j ] for each j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1.

- If opSubpicIdList[ ] is not present the following applies:

–	If the layers in targetOp include all layers in entireBitstream and opTid is equal to the highest TemporalId value among all NAL units in entireBitstream, BitstreamToDecode is set to be identical to entireBitstream.

–	Otherwise, BitstreamToDecode is set to be the output by invoking the sub-bitstream extraction process as specified in clause C.6 with entireBitstream, opOlsIdx, and opTid as inputs.

Otherwise (opSubpicIdList[ ] is  present), BitstreamToDecode is set to be the output by invoking the subpicture sub-bitstream extraction process as specified in clause C.7 with entireBitstream, opOlsIdx, opTid and opSubpicIdList[ ] as inputs.

- The values of TargetOlsIdx and Htid are set equal to opOlsIdx and opTid, respectively, of targetOp.

- The general_hrd_parameters( ) syntax structure, the ols_hrd_parameters( ) syntax structure, and the sublayer_hrd_parameters( ) syntax structure applicable to BitstreamToDecode are selected as follows:

–	If NumLayersInOls[ TargetOlsIdx ] is equal to 1, the general_hrd_parameters( ) syntax structure and the ols_hrd_parameters( ) syntax structure in the SPS referenced by the layer in BitstreamToDecode (or provided through an external means not specified in this Specification) are selected. Otherwise, the general_hrd_parameters( ) syntax structure and the ols_hrd_idx[ TargetOlsIdx ]-th ols_hrd_parameters( ) syntax structure in the VPS (or provided through an external means not specified in this Specification) are selected.

–	Within the selected ols_hrd_parameters( ) syntax structure, for testing of the Type I bitstream conformance piont, the sublayer_hrd_parameters( Htid ) syntax structure that immediately follows the condition "if( general_vcl_hrd_params_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 0, and for testing of the Type II bitstream conformance piont, the sublayer_hrd_parameters( Htid ) syntax structure that immediately follows the condition "if( general_nal_hrd_params_present_flag )" is selected and the variable NalHrdModeFlag is set equal to 1. When BitstreamToDecode is a Type II bitstream and NalHrdModeFlag is equal to 0, all non-VCL NAL units except filler data NAL units, and all  leading_zero_8bits, zero_byte, start_code_prefix_one_3bytes and trailing_zero_8bits syntax elements that form a byte stream from the NAL unit stream (as specified in Annex B), when present, are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

- An AU associated with a BP SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) applicable to TargetOp is selected as the HRD initialization point and referred to as AU 0.

- When general_decoding_unit_hrd_params_present_flag in the selected general_hrd_parameters( ) syntax structure is equal to 1, the CPB is scheduled to operate either at the AU level (in which case the variable DecodingUnitHrdFlag is set equal to 0) or at the DU level (in which case the variable DecodingUnitHrdFlag is set equal to 1). Otherwise, DecodingUnitHrdFlag is set equal to 0 and the CPB is scheduled to operate at the AU level.

- For each AU in BitstreamToDecode starting from AU 0, the BP SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the AU and applies to TargetOlsIdx is selected, and the PT SEI message (present in BitstreamToDecode or available through external means not specified in this Specification) that is associated with the AU and applies to TargetOlsIdx is selected, and when DecodingUnitHrdFlag is equal to 1 and du_cpb_params_in_pic_timing_sei_flag is equal to 0, the DU information SEI messages (present in BitstreamToDecode or available through external means not specified in this Specification) that are associated with DUs in the AU and apply to TargetOlsIdx are selected.

- A value of ScIdx is selected. The selected ScIdx shall be in the range of 0 to hrd_cpb_cnt_minus1, inclusive.

- When the BP SEI message associated with AU 0 has cpb_alt_timing_info_present_flag equal to 1, either of the following applies for selection of the initial CPB removal delay and delay offset:

–	If NalHrdModeFlag is equal to 1, the default initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and nal_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message are selected. Otherwise, the default initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message are selected. The variable DefaultInitCpbParamsFlag is set equal to 1.

–	If NalHrdModeFlag is equal to 1, the alternative initial CPB removal delay and delay offset represented by nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and nal_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message and cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and cpb_alt_initial_removal_offset_delta[ Htid ][ ScIdx ], respectively, in the PT SEI message associated with the AU following AU 0 in decoding order are selected. Otherwise, the alternative initial CPB removal delay and delay offset represented by vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, in the selected BP SEI message and cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and cpb_alt_initial_removal_offset_delta[ Htid ][ ScIdx ], respectively, in the PT SEI message associated with the AU following AU 0 in decoding order are selected. The variable DefaultInitCpbParamsFlag is set equal to 0, and one of the following applies:

–	The RASL AUs associated with AU 0 are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

–	All AUs following AU 0 in decoding order up to an AU associated with a DRAP indication SEI message are discarded from BitstreamToDecode and the remaining bitstream is assigned to BitstreamToDecode.

Each conformance test consists of a combination of one option in each of the above steps. When there is more than one option for a step, for any particular conformance test only one option is chosen. All possible combinations of all the steps form the entire set of conformance tests. For each operation point under test, the number of bitstream conformance tests to be performed is equal to n0 * n1 * ( 2 * n2 + n3 + n4 + n5), where the values of n0, n1, n2, n3, n4, and n5 are specified as follows:

–	n0 is set equal to 2. [Ed. (YK): Clarify what is n0, which step is is about. Also better describe the correspondin step for each of the n* variables.]

–	n1 is equal to hrd_cpb_cnt_minus1 + 1.

–	n2 is the number of AUs in BitstreamToDecode that each is associated with a BP SEI message applicable to TargetOlsIdx and for which all of the following conditions are true:

–	nal_unit_type is equal to CRA_NUT.

–	The associated BP SEI message has bp_alt_cpb_params_present_flag equal to 1.

–	There is at least one RASL picture asscociated to the AU.

–	n3 is the number of GDR or IRAP AUs without associated RASL pictures in BitstreamToDecode that each is associated with a BP SEI message applicable to TargetOlsIdx and for which not all of the following conditions are true:

–	nal_unit_type is equal to CRA_NUT.

–	The associated BP SEI message has bp_alt_cpb_params_present_flag equal to 1.

–	There is at least one RASL picture associated to the AU.

–	n4 is the number of AUs in BitstreamToDecode that each is associated with a DRAP indication SEI message applicable to TargetOlsIdx and for each of which the associated PT SEI message has cpb_alt_timing_info_present_flag equal to 1.

–	n5 is derived as follows:

–	If general_decoding_unit_hrd_params_present_flag in the selected general_hrd_parameters( ) syntax structure is equal to 0, n5 is equal to 1.

–	Otherwise, n5 is equal to 2.

NOTE X – n0 corresponds to conformance tests for Type I bitstream conformance and Type II bitstream conformance. n1 corresponds to conformance tests for each CPB delivery schedule. n2 corresponds to conformance tests for bitstreams starting at each CRA pictures with associated RASL pictures and alternative initial CPB removal delay and delay offset present. These test are carried twice: once for bitstream keeping and once for bitstreams removing RASL pictures associated with the CRA. n3 corresponds to conformance tests for bitstreams starting at each GDR or IRAP AU that is not a CRA with associated RASL pictures and alternative initial CPB removal delay and delay offset present. n4 corresponds to conformance tests for bitstreams starting with each IRAP with associated DRAP pictures with alternative timing information present and result of removing all access units between a DRAP picture with alternative timing information present and the preceding IRAP. n5 corresponds to conformance tests for AU based conformance and DU-based if general_decoding_unit_hrd_params_present_flag is equal to 1

When BitstreamToDecode is a Type II bitstream, the following applies:

–	If the sublayer_hrd_parameters( Htid ) syntax structure that immediately follows the condition "if( general_vcl_hrd_params_present_flag )" is selected, the test is conducted at the Type I conformance point shown in Figure C.1, and only VCL and filler data NAL units are counted for the input bit rate and CPB storage.

–	Otherwise (the sublayer_hrd_parameters( Htid ) syntax structure that immediately follows the condition "if( general_nal_hrd_params_present_flag )" is selected), the test is conducted at the Type II conformance point shown in Figure C.1, and all bytes of the Type II bitstream, which may be a NAL unit stream or a byte stream, are counted for the input bit rate and CPB storage.

NOTE 2 – NAL HRD parameters established by a value of ScIdx for the Type II conformance point shown in Figure C.1 are sufficient to also establish VCL HRD conformance for the Type I conformance point shown in Figure C.1 for the same values of InitCpbRemovalDelay[ Htid ][ ScIdx ], BitRate[ Htid ][ ScIdx ] and CpbSize[ Htid ][ ScIdx ] for the variable bit rate (VBR) case (cbr_flag[ Htid ][ ScIdx ] equal to 0). This is because the data flow into the Type I conformance point is a subset of the data flow into the Type II conformance point and because, for the VBR case, the CPB is allowed to become empty and stay empty until the time a next picture is scheduled to begin to arrive.

…

## C.2	Operation of the CPB

### C.2.1	General

The specifications in this clause apply independently to each set of CPB parameters that is present and to both the Type I and Type II conformance points shown in Figure C.1 and the set of CPB parameters is selected as specified in clause C.1.

### C.2.2	Timing of DU arrival

If DecodingUnitHrdFlag is equal to 0, the variable decodingUnitParamsFlag is set equal to 0 and the process specified in the remainder of this clause is invoked with a DU being considered as an AU, for derivation of the initial and final CPB arrival times for AU n.

…

### C.2.3	Timing of DU removal and decoding of DU

The variables InitCpbRemovalDelay[ Htid ][ ScIdx ], and InitCpbRemovalDelayOffset[ Htid ][ ScIdx ], are derived as follows: [Ed. (VD): InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] is derived here but never used in this clause. (YK): Hmm, this is simialr as in HEVC. I don’t know why it was like this in HEVC; maybe inherited from AVC. We should study whether the derivation of InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] is really not needed herein, and if not, we can just remove it. Probably also for HEVC.]

–	If one or more of the following conditions are true, InitCpbRemovalDelay[ Htid ][ ScIdx ] and InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] are set equal to the values of the BP SEI message syntax elements nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and nal_initial_cpb_removal_offset[ Htid ][ ScIdx ] minus the values of the PT SEI message syntax elements cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and cpb_alt_initial_removal_offset_delta[ Htid ][ ScIdx ] of AU n+1, respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ] minus the values of the PT SEI message syntax elements cpb_alt_initial_removal_delay_delta[ Htid ][ ScIdx ] and cpb_alt_initial_removal_offset_delta[ Htid ][ ScIdx ] of AU n+1, respectively, when NalHrdModeFlag is equal to 0, where the BP SEI message containing the syntax elements is selected as specified in clause C.1:

–	UseAltCpbParamsFlag for AU n is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, if the value of decodingUnitParamsFlag is equal to 1, InitCpbRemovalDelay[ Htid ][ ScIdx ] and InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] are set equal to the values of the BP SEI message syntax elements nal_initial_alt_cpb_removal_delay[ Htid ][ ScIdx ] and nal_initial_alt_cpb_removal_offset[ Htid ][ ScIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_alt_cpb_removal_delay[ Htid ][ ScIdx ] and vcl_initial_alt_cpb_removal_offset[ Htid ][ ScIdx ], respectively, when NalHrdModeFlag is equal to 0, where the BP SEI message containing the syntax elements is selected as specified in clause C.1.

–	Otherwise (DecodingUnitHrdFlag is equal to 0) [Ed. (VD/YK): Something is wrong here, at least for the mixed use of decodingUnitParamsFlag, which is but should not be a local variable, and DecodingUnitHrdFlag. With the introduction of the alternative HRD parameters for CRA/RASL and DRAP cases, the condition of when to use the alternative set of HRD parameters definitely needs to be updated.], InitCpbRemovalDelay[ Htid ][ ScIdx ] and InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] are set equal to the values of the BP SEI message syntax elements nal_initial_cpb_removal_delay[ Htid ][ ScIdx ] and nal_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, when NalHrdModeFlag is equal to 1, or vcl_initial_cpb_removal_delay[ Htid ][ ScIdx ] and vcl_initial_cpb_removal_offset[ Htid ][ ScIdx ], respectively, when NalHrdModeFlag is equal to 0, where the BP SEI message containing the syntax elements is selected as specified in clause C.1.

The nominal removal time of the AU n from the CPB is specified as follows:

–	If AU n is the AU with n equal to 0 (the AU that initializes the HRD), the nominal removal time of the AU from the CPB is specified by:

NominalRemovalTime[ 0 ] = InitCpbRemovalDelay[ Htid ][ ScIdx ] ÷ 90000	(C.9)
[Ed. (YK): It's probably bettter to conistently either use an Htid-index for both InitCpbRemovalDelay and NominalRemovalTime, or not use this index for both, the latter of which means to drop the Htid-index in many other HRD parameters/variables as well, like BitRate, CpbSize, etc.]

–	Otherwise, the following applies: [Ed. (VD/YK): Throughout the document, carefully and consistently apply the prefixes "Au" and "Du" to variables FinalArrivalTime, NominalRemovalTime, CpbRemovalTime, etc.]

–	When AU n is the first AU of a BP that does not initialize the HRD, the following applies:

The nominal removal time of the AU n from the CPB is specified by:

if( !concatenationFlag ) {
	baseTime = NominalRemovalTime[ firstPicInPrevBuffPeriod ]
	tmpCpbRemovalDelay = CpbRemovalDelayVal
} else {
	baseTime1 = NominalRemovalTime[ prevNonDiscardablePic ]
	tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )
	baseTime2 = AuNominalRemovalTime[ n − 1 ]
	tmpCpbRemovalDelay2 =				(C.10)
			Ceil( ( InitCpbRemovalDelay[ Htid ][ ScIdx ] ÷ 90000 +
				FinalArrivalTime[ n − 1 ] − NominalRemovalTime[ n − 1 ] ) ÷ ClockTick )
	if( baseTime1 + ClockTick * tmpCpbRemovalDelay1 <
				baseTime2 + ClockTick * tmpCpbRemovalDelay2 ) {
		baseTime = baseTime2
		tmpCpbRemovalDelay = tmpCpbRemovalDelay2
	} else {
		baseTime = baseTime1
		tmpCpbRemovalDelay = tmpCpbRemovalDelay1
	}
}
NominalRemovalTime[ n ] = baseTime + ( ClockTick * tmpCpbRemovalDelay − CpbDelayOffset )

where NominalRemovalTime[ firstPicInPrevBuffPeriod ] is the nominal removal time of the first AU of the previous BP, NominalRemovalTime[ prevNonDiscardablePic ] is the nominal removal time of the preceding picture in decoding order with TemporalId equal to 0 that is not a RASL or RADL picture, CpbRemovalDelayVal is the value of CpbRemovalDelayVal[ Htid ] derived according to cpb_removal_delay_minus1[ Htid ] and cpb_removal_delay_delta_idx[ Htid ] in the PT SEI message, and cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ Htid ] ] in the BP SEI message, selected as specified in clause C.1, associated with AU n and concatenationFlag and CpbRemovalDelayDeltaMinus1 are the values of the syntax elements concatenation_flag and cpb_removal_delay_delta_minus1, respectively, in the BP SEI message, selected as specified in clause C.1, associated with AU n.

After the derivation of the nominal CPB removal time and before the derivation of the DPB output time of access unit n, the variables DpbDelayOffset and CpbDelayOffset are derived as:

–	If one or more of the following conditions are true, DpbDelayOffset is set equal to the value of the PT SEI message syntax element dpb_delay_offset[ Htid ] of AU n + 1, and CpbDelayOffset is set equal to the value of the PT SEI message syntax element cpb_delay_offset[ Htid ] of AU n + 1, where the PT SEI message containing the syntax elements is selected as specified in clause C.1:

–	UseAltCpbParamsFlag for AU n is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, DpbDelayOffset and CpbDelayOffset are set equal to 0.

–	When AU n is not the first AU of a BP, the nominal removal time of the AU n from the CPB is specified by:

NominalRemovalTime[ n ] = NominalRemovalTime[ firstPicInCurrBuffPeriod ] +
		ClockTick * ( CpbRemovalDelayVal − CpbDelayOffset )	(C.11)

where NominalRemovalTime[ firstPicInCurrBuffPeriod ] is the nominal removal time of the first AU of the current BP and CpbRemovalDelayVal is the value of CpbRemovalDelayVal[ OpTid ] derived according to cpb_removal_delay_minus1[ OpTid ] and cpb_removal_delay_delta_idx[ OpTid ] in the PT SEI message, and cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ OpTid ] ] in the BP SEI message, selected as specified in clause C.1, associated with AU n.

When DecodingUnitHrdFlag is equal to 1, the following applies:

–	The variable duCpbRemovalDelayInc is derived as follows:

–	If du_cpb_params_in_pic_timing_sei_flag is equal to 0, duCpbRemovalDelayInc is set equal to the value of du_spt_cpb_removal_delay_increment[ i ] in the DU information SEI message, selected as specified in clause C.1, associated with DU m.

–	Otherwise, if du_common_cpb_removal_delay_flag is equal to 0, duCpbRemovalDelayInc is set equal to the value of du_cpb_removal_delay_increment_minus1[ i ][ Htid ] + 1 for DU m in the PT SEI message, selected as specified in clause C.1, associated with AU n, where the value of i is 0 for the first num_nalus_in_du_minus1[ 0 ] + 1 consecutive NAL units in the AU that contains DU m, 1 for the subsequent num_nalus_in_du_minus1[ 1 ] + 1 NAL units in the same AU, 2 for the subsequent num_nalus_in_du_minus1[ 2 ] + 1 NAL units in the same AU, etc.

–	Otherwise, duCpbRemovalDelayInc is set equal to the value of du_common_cpb_removal_delay_increment_minus1[ Htid ] + 1 in the PT SEI message, selected as specified in clause C.1, associated with AU n.

–	The nominal removal time of DU m from the CPB is specified as follows, where AuNominalRemovalTime[ n ] is the nominal removal time of AU n:

–	If DU m is the last DU in AU n, the nominal removal time of DU m DuNominalRemovalTime[ m ] is set equal to AuNominalRemovalTime[ n ].

–	Otherwise (DU m is not the last DU in AU n), the nominal removal time of DU m DuNominalRemovalTime[ m ] is derived as follows:

if( du_cpb_params_in_pic_timing_sei_flag )
	DuNominalRemovalTime[ m ] = DuNominalRemovalTime[ m + 1 ] −
		ClockSubTick * duCpbRemovalDelayInc	(C.12)
else
	DuNominalRemovalTime[ m ] = AuNominalRemovalTime[ n ] −
		ClockSubTick * duCpbRemovalDelayInc

If DecodingUnitHrdFlag is equal to 0, the removal time of AU n from the CPB is specified as follows, where FinalArrivalTime[ n ] and NominalRemovalTime[ n ] are the final CPB arrival time and nominal CPB removal time, respectively, of AU n:

if( !low_delay_hrd_flag[ Htid ]  | |  NominalRemovalTime[ n ]  >=  FinalArrivalTime[ n ] )
	CpbRemovalTime[ n ] = NominalRemovalTime[ n ]
else				(C.13)
	CpbRemovalTime[ n ] = NominalRemovalTime[ n ] + ClockTick *
		Ceil( ( FinalArrivalTime[ n ] − NominalRemovalTime[ n ] ) ÷ ClockTick )

NOTE 1 – When low_delay_hrd_flag[ Htid ] is equal to 1 and NominalRemovalTime[ n ] is less than FinalArrivalTime[ n ], the size of AU n is so large that it prevents removal at the nominal removal time.

Otherwise (DecodingUnitHrdFlag is equal to 1), the removal time of DU m from the CPB is specified as follows:

if( !low_delay_hrd_flag[ Htid ]  | |  DuNominalRemovalTime[ m ]  >=  DuFinalArrivalTime[ m ] )
	DuCpbRemovalTime[ m ] = DuNominalRemovalTime[ m ]
else				(C.14)
	DuCpbRemovalTime[ m ] = DuFinalArrivalTime[ m ]

NOTE 2 – When low_delay_hrd_flag[ Htid ] is equal to 1 and DuNominalRemovalTime[ m ] is less than DuFinalArrivalTime[ m ], the size of DU m is so large that it prevents removal at the nominal removal time.

If DecodingUnitHrdFlag is equal to 0, at the CPB removal time of AU n, the AU is instantaneously decoded.

Otherwise (DecodingUnitHrdFlag is equal to 1), at the CPB removal time of DU m, the DU is instantaneously decoded, and when DU m is the last DU of AU n, the following applies:

–	Picture n is considered as decoded.

–	The final CPB arrival time of AU n, i.e., AuFinalArrivalTime[ n ], is set equal to the final CPB arrival time of the last DU in AU n, i.e., DuFinalArrivalTime[ m ].

–	The nominal CPB removal time of AU n, i.e., AuNominalRemovalTime[ n ], is set equal to the nominal CPB removal time of the last DU in AU n, i.e., DuNominalRemovalTime[ m ].

–	The CPB removal time of AU n, i.e., AuCpbRemovalTime[ m ], is set equal to the CPB removal time of the last DU in AU n, i.e., DuCpbRemovalTime[ m ].

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)