| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0300 |
| --- | --- |

| Title: | Additional fix for long luma deblocking decisions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution proposes to fix the long deblocking decision such that all lines of respective 4 samples boundary segment are checked to avoid over filtering of lines 1 and 2 due to decision based only on line 0 and line 3. It is asserted that the proposal ensures that the deblocking filtering is robust and that the fix does not increase worst case complexity for deblocking decisions.

The BD rate impact for luma for CTC SDR:

AI: 0.00%, RA: -0.01%, LDB: -0.05%, LDP: -0.01%.

The BD rate impact for CTC HDR:

AI:  DE:   0.01% PSNRL: 0.00% wPSNRY: 0.00%, PSNRY: 0.00%

RA: DE: -0.04% PSNRL: -0.03% wPSNRY: -0.02%, PSNRY: 0.00%

Similar encoding and decoding time as the anchor.

# Introduction

Currently the long deblocking decision are based on checks for line 0 and 3 for each 4 samples boundary segment which works fine in many cases. However in some corner cases when there is some structure/details on line 1 on samples p0,1 to p7,1 and q0,1 to q7,1  or on line 2 on samples p0,2to p7,2 and q0,2 to q7,2 deblocking can be applied since line 0 and line 3 pass the check which then can cause over filtering of 28 samples for a 4 sample boundary segment, e.g.  p0,1 to p6,1, q0,1 to q6,1, p0,2 to p6,2 and q0,2 to q6,2.

# Proposal

To ensure that the long deblocking decision is robust in all situations it is proposed to check long deblocking decisions for all lines and not only for line 0 and 3.

The changes to the draft is just to apply 8.8.3.6.5	Decision process for a luma sample, not only for lines 0 and 3 but also for lines 1 and 2 so that dSam1 and dSam2 are obtained in addition to dSam0 and dSam3 and then to require dSam0 to dSam3 to all be equal to 1 to set dE to 3, e.g. apply long deblocking filter. Exact text is attached to the contribution.

# Complexity analysis

Long deblocking filter can be used when at least one side has a width for vertical boundaries or height for horizontal boundaries of 32 samples or more and the other side has width for vertical boundaries or height for horizontal boundaries of 8 samples or more.

The current long filter decision for that case has for each 4 samples boundary segment to make calculations used for both long and strong deblocking filter but also calculations specific for long filter decision and then also calculations specific for strong filter decision in case long filter not is selected. In total about 2 segments*(28+46+14)/ (32*8)=2*88/(32*8)=176/(32*8) = 0.69 operations per sample.

The additional complexity for computation of long filter decision for line 1 and 2 are 2 segments * (28+46 -2)/(32*8)=2*72=144/(32*8)=0.56 operations per sample.

The total number of operations after the suggested fix is 0.69+0.56 = 1.25 operations per sample.

The worst case for deblocking decision calculations is not modified since that happens when all blocks are 8x8 where the number of operations per sample for decisions are about 2 segments * (28+14)/(8*8)=2*42/64=1.28 operations per sample.

Calculations in general for both long and strong deblocking filter (10 abs + 4 shifts + 14 adds = 28 op):

dp0 = Abs( p2,0 − 2 * p1,0 + p0,0 )		(1298)

dp3 = Abs( p2,3 − 2 * p1,3 + p0,3 )		(1299)

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )		(1300)

dq3 = Abs( q2,3 − 2 * q1,3 + q0,3 )		(1301)

sp0 = Abs( p3,0 − p0,0 )		(1302)

sq0 = Abs( q0,0 − q3,0 )		(1303)

spq0 = Abs( p0,0 − q0,0 )		(1304)

sp3 = Abs( p3,3 − p0,3 )		(1305)

sq3 = Abs( q0,3 − q3,3 )		(1306)

spq3 = Abs( p0,3 − q0,3 )		(1307)

Additional calculations for long filter of one side (6 abs + 6 shifts + 27 adds + 7cmp=46 op):

dp0L = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1	(1308)

dp3L = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1					   (1309)

sp0L = sp0 + Abs( p7,0 − p6,0 − p5,0 + p4,0)		(1317)

sp3L = sp3 + Abs( p7,3 − p6,3 − p5,3 + p4,3)		(1318)

When dL is less than β

dpq0L = dp0L + dq0L		(1325)

dpq3L = dp3L + dq3L		(1326)

dL = dpq0L + dpq3L		(1327)

2x :

sp = ( sp + Abs( p3 − p0 ) + 1 )  >>  1	(1386)

- dpq is less than ( β  >>  2 ),

- sp + sq is less than sThr,

- spq is less than ( 5 * tC + 1 )  >>  1.

Additional calculations for strong filter (7 adds + 7 cmp = 14op)

dpq0 = dp0 + dq0		(1296)

dpq3 = dp3 + dq3		(1297)

dp = dp0 + dp3		(1298)

dq = dq0 + dq3		(1299)

d = dpq0 + dpq3		(1300)

When d is less than β

2x:

- dpq is less than ( β  >>  2 ),

- sp + sq is less than sThr,

- spq is less than ( 5 * tC + 1 )  >>  1.

# Results

## SDR

The objective performance for the proposal is shown below in comparison to VTM-8.0 [2] using CTC [3]. The objective results show a small gain. Encoding time is measured on simulations run on a cluster with machines with same capability but they are not reliable. Decoding time is measured by running test and anchor on same machine without yuv output. Encoding and decoding time is similar as for the anchor.

|  | All intra Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | 0,01% | 0,00% | 98% | 99% |
| Class A2 | 0,00% | -0,01% | 0,00% | 101% | 99% |
| Class B | 0,00% | 0,00% | 0,01% | 97% | 99% |
| Class C | 0,00% | 0,00% | 0,00% | 91% | 100% |
| Class E | 0,00% | 0,00% | 0,00% | 90% | 98% |
| Overall | 0,00% | 0,00% | 0,00% | 95% | 99% |
| Class D | 0,00% | 0,01% | 0,00% | 87% | 101% |
| Class F | 0,00% | 0,00% | -0,01% | 96% | 100% |

|  | Random access Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,03% | -0,03% | -0,01% | 99% | 102% |
| Class A2 | -0,02% | -0,01% | 0,06% | 100% | 101% |
| Class B | 0,01% | -0,07% | 0,02% | 96% | 99% |
| Class C | -0,02% | 0,03% | -0,07% | 92% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | -0,02% | 0,00% | 96% | 101% |
| Class D | 0,03% | 0,03% | -0,11% | 91% | 97% |
| Class F | 0,05% | 0,01% | 0,00% | 93% | 101% |

|  | Low-delay B Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,02% | 0,22% | -0,09% | 102% | 99% |
| Class C | -0,04% | -0,15% | 0,03% | 98% | 102% |
| Class E | -0,12% | -0,23% | -0,26% | 98% | 99% |
| Overall | -0,05% | -0,02% | -0,09% | 100% | 100% |
| Class D | -0,10% | -0,09% | 0,30% | 90% | 97% |
| Class F | -0,01% | 0,75% | 0,22% | 95% | 102% |

|  | Low-delay P Over VTM-8.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,01% | 0,14% | 0,18% | 99% | 99% |
| Class C | -0,02% | -0,11% | -0,22% | 90% | 101% |
| Class E | -0,02% | -0,37% | 0,20% | 90% | 96% |
| Overall | -0,01% | -0,07% | 0,05% | 94% | 99% |
| Class D | -0,03% | 0,30% | -0,11% | 85% | 104% |
| Class F | -0,08% | -0,13% | -0,30% | 90% | 101% |

## HDR

HDR metrics was calculated using HDRTools v0.19.1 linked in to VTM-8.0 according to CTC for HDR [4]. Metrics are similar as for the anchor. Encoding simulations was run on different machines with similar capabilities but are not fully accurate. Decoding time is measured by running test and anchor on same machine without yuv output.

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0,01% | 0,00% | 0,00% | 0,01% | 0,02% | 0,00% | 0,01% | 0,02% | 93% | 100% |
| Class H2 |  |  |  |  |  | 0,00% | 0,00% | -0,01% | 98% | 101% |
| Overall | 0,01% | 0,00% | 0,00% | 0,01% | 0,02% | 0,00% | 0,01% | 0,01% | 95% | 100% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0,04% | -0,03% | -0,02% | 0,02% | 0,09% | -0,01% | 0,01% | 0,14% | 93% | 100% |
| Class H2 |  |  |  |  |  | 0,00% | -0,02% | 0,02% | 99% | 100% |
| Overall | -0,04% | -0,03% | -0,02% | 0,02% | 0,09% | 0,00% | 0,00% | 0,10% | 95% | 100% |

# Conclusions

It is proposed to fix the long luma deblocking decision such that not only lines 0 and 3 are checked but also lines 1 and 2 are checked before application of long luma deblocking filtering. It is proposed to include the fix to the specification and software to reduce the risk of over filtering of luma samples of lines 1 and 2 which can be achieved without increasing worst case complexity for deblocking decisions.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 8)", Document of Joint Video Experts Team, JVET-Q2001.

- VTM-8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010

- A. Segall, E. François, W. Husak, S. Iwamura and D. Rusanovskyy, “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-P2011

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).