| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0302-v1 |
| --- | --- |

| Title: | AHG12: On signaling of chroma QP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li, Xiang Li, Byeongdoo Choi, Stephan, Wenger, Shan Liu | Tel: Email: | {aurali, xlxiangli, bdchoi, swenger, shanl }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In the last meeting, alternatively signaling luma global QP offset in either picture header or slice header is adopted. In this proposal, applying similar mechanism to signal chroma global QP offset is proposed.

- Add a flag in PPS indicating whether chroma global QP offset is signaled in picture header or slice header, just like luma global QP offset.

- Depending on the abovementioned flag, chroma global QP offset is alternatively signaled either in picture header or slice header.

# Introduction

The syntax elements for alternatively signaling luma global QP offset in either picture header or slice header are described below.

Table 1. luma QP related syntax elements in PPS, PH and slice header of VVC Draft 8

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| init_qp_minus26 | se(v) |
| … |  |
| qp_delta_info_in_ph_flag | u(1) |
| … |  |
| } |  |

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( qp_delta_info_in_ph_flag ) |  |
| ph_qp_delta | se(v) |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( !qp_delta_info_in_ph_flag ) |  |
| slice_qp_delta | se(v) |
| … |  |
| } |  |

qp_delta_info_in_ph_flag equal to 1 specifies that QP delta information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. qp_delta_info_in_ph_flag equal to 0 specifies that QP delta information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure.

ph_qp_delta specifies the initial value of QpY to be used for the coding blocks in the picture until modified by the value of CuQpDeltaVal in the coding unit layer.

When qp_delta_info_in_ph_flag is equal to 1, the initial value of the QpY quantization parameter for all slices of the picture, SliceQpY, is derived as follows:

SliceQpY = 26 + init_qp_minus26 + ph_qp_delta

The value of SliceQpY shall be in the range of −QpBdOffset to +63, inclusive.

slice_qp_delta specifies the initial value of QpY to be used for the coding blocks in the slice until modified by the value of CuQpDeltaVal in the coding unit layer.

When qp_delta_info_in_ph_flag is equal to 0, the initial value of the QpY quantization parameter for the slice, SliceQpY, is derived as follows:

SliceQpY = 26 + init_qp_minus26 + slice_qp_delta

The value of SliceQpY shall be in the range of −QpBdOffset to +63, inclusive.

For the coherent of luma and chroma global QP offset, it is proposed to alternativley signal chroma global QP offset either in PH or SH too.

# Proposed Spec Text

Changes compared to the latest VVC Draft 8 vE [1] are highlighted.

## Adding a flag in PPS indicates whether chroma global QP offset is signaled in picture header or slice header

Table 2. proposed changes in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_chroma_tool_offsets_present_flag | u(1) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| … |  |
| chroma_qp_offset_info_in_ph_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |

chroma_qp_offset_info_in_ph_flag equal to 1 specifies that chroma QP offset information is present in the PH syntax structure and not present in slice headers referring to the PPS that do not contain a PH syntax structure. chroma_qp_offset_info_in_ph_flag equal to 0 specifies that QP delta information is not present in the PH syntax structure and may be present in slice headers referring to the PPS that do not contain a PH syntax structure. When chroma_qp_offset_info_in_ph_flag is not present, it is inferred to be equal to 0.

## Changes in picture header

Table 3. proposed changes in picture header

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if (ChromaArrayType !=0) { |  |
| if ( chroma_qp_offset_info_in_ph_flag) { |  |
| ph_cb_qp_offset | se(v) |
| ph_cr_qp_offset | se(v) |
| ph_joint_cbcr_qp_offset | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

ph_cb_qp_offset, ph_cr_qp_offset and ph_joint_cbcr_qp_offset specify the offsets to the luma quantization parameter Qp′Y used for deriving Qp′Cb, Qp′Cr and Qp′CbCr, respectively. The values of ph_cb_qp_offset, ph_cr_qp_offset and ph_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive. When not present, the values of ph_cb_qp_offset, ph_cr_qp_offset and ph_joint_cbcr_qp_offset are inferred to be equal to 0. The value of pps_cb_qp_offset + ph_cb_qp_offset shall be in the range of −12 to +12, inclusive. The value of pps_cr_qp_offset + ph_cr_qp_offset shall be in the range of −12 to +12, inclusive. The value of pps_ joint_cbcr _qp_offset + ph_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive.

## Changes in slice header

Table 4. proposed changes in slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if (ChromaArrayType !=0) { |  |
| if ( !chroma_qp_offset_info_in_ph_flag) { |  |
| slice_cb_qp_offset | se(v) |
| slice_cr_qp_offset | se(v) |
| slice_joint_cbcr_qp_offset | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

## Chroma QP derivation

It is proposed to use either QP offset signaled in PH or SH to derive chroma QP value to achieve different level of QP granularity.

Qp′Cb =  Clip3( −QpBdOffset, 63, qPCb + pps_cb_qp_offset+ (chroma_qp_offset_info_in_ph_flag？ph_cb_qp_offset ： slice_cb_qp_offset)+ CuQpOffsetCb )
	 + QpBdOffset		(1147)

Qp′Cr = Clip3( −QpBdOffset, 63, qPCr + pps_cr_qp_offset + (chroma_qp_offset_info_in_ph_flag？ph_cr_qp_offset ： slice_cr_qp_offset)+ CuQpOffsetCr )
	 + QpBdOffset 														(1148)

Qp′CbCr = Clip3( −QpBdOffset, 63, qPCbCr + pps_joint_cbcr_qp_offset_value +

(chroma_qp_offset_info_in_ph_flag？ph_joint_cbcr_qp_offset :  slice_joint_cbcr_qp_offset) +CuQpOffsetCbCr ) + QpBdOffset

# Conclusion

It is proposed to have a coherent global QP offset signaling mechanism for both luma and chroma. An PPS flag is introduced to indicate whether chroma global QP offset in PH or SH is signaled and used for chroma QP derivation. It is recommended to adopt this proposal into next version of VVC.

# Reference

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 8)” Document of Joint Video Experts Team (JVET), JVET-Q2001-vE, Brussels, 2020.

# Patent rights declaration

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).