| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Alpbach, AT, 15–24 April 2020 | Document: JVET-R0303 |
| --- | --- |

| Title: | Modified LFNST signalling for single tree blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Kyohei Unno Kei Kawamura 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-6887-2592 yo-kidani@kddi.com ky-unno@kddi-research.jp ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes modified LFNST signalling for single tree blocks. In the latest VVC Working Draft and VTM, a single LFNST index, which defines availability of LFNST, is signaled for both luma and chroma coding blocks in single tree. In single tree case, however, not only luma coding blocks but also chroma coding blocks are considered in the LFNST index parsing condition even though the application of LFNST into chroma coding block is prohibited in the decoding process. The proposed method is to modify the LFNST index parsing condition to remove the redundancy of checking single tree chroma coding blocks. Simulation results under common test condition (CTC) show an overall luma BD-rates of 0.01% for RA compared to VTM 8.0 without running time increment. In contrast, simulation results under non-CTC, where dual tree for I-slice is prohibited by default, show an overall luma BD-rates of 0.00%/0.01% for AI/RA compared to VTM 8.0 without running time increment.

Introduction

Low frequency Non-Separable Transform (LFNST) is adopted in the latest current VVC Working Draft 8 [1] and VTM 8.0 [2] to improves coding performance.

LFNST index, which defines availability of LFNST, is signaled based on the parsing condition which includes the three flags, i.e., LfnstNotTsFlag, LfnstDcOnly, and LfnstZeroOutSigCoeff. These three flags are derived from transform_skip_flag, the last scan position of the non-zero coefficient, and appearance of non-zero coefficients, respectively.

Application of transform skip for chroma coding block had been adopted in 16th Geneva meeting, and LfnstNotTsFlag considering transform_skip_flag for both luma and chroma coding blocks was adopted in 17th Brussel meeting. In the Brussel meeting, it was also adopted that the application of LFNST into single tree chroma coding blocks is prohibited in the decoding process.

In single tree case, a single LFNST index is signaled for luma and chroma coding blocks so that not only luma coding blocks but chroma coding blocks are considered in those three flags to determine LFNST signaling even though application of LFNST into single tree chroma coding blocks is prohibited.

Proposed methods

The proposed method is to modify the LFNST index parsing condition regarding LfnstNotTsFlag, LfnstDcOnly and LfnstZeroOutSigCoeff to remove the redundancy of checking single tree chroma coding blocks. In detail, the checking processes to update the values of LfnstNotTsFlag, LfnstDcOnly and LfnstZeroOutSigCoeff are skipped to single tree chroma coding blocks by specifying the block tree types or color components. The following yellow-highlighted parts show the difference in the proposed syntax table from that of VVC working draft 8 JVET-Q2001-v15.docx.

7.3.10.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| cu_act_enabled_flag | ae(v) |
| LfnstNotTsFlag = 1 |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |
| } |  |

7.3.10.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |
| if( tu_y_coded_flag[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( transform_skip_flag[ x0 ][ y0 ][ 0] ) |  |
| LfnstNotTsFlag = 0 |  |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), treeType, 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), treeType, 0 ) |  |
| } |  |
| if( tu_cb_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( transform_skip_flag[ x0 ][ y0 ][ 1] && treeType = = DUAL_TREE_CHROMA ) |  |
| LfnstNotTsFlag = 0 |  |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), treeType, 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), treeType, 1 ) |  |
| } |  |
| if( tu_cr_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cb_coded_flag[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( transform_skip_flag[ x0 ][ y0 ][ 2] && treeType = = DUAL_TREE_CHROMA ) |  |
| LfnstNotTsFlag = 0 |  |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), treeType, 2 ) |  |
| Else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), treeType, 2 ) |  |
| } |  |
| } |  |

7.3.10.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, treeType, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( cIdx = = 0 ¦ ¦ treeType = = DUAL_TREE_CHROMA) { |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| } |  |
| if( ( lastSubBlock > 0 ¦ ¦ lastScanPos > 0 ) && cIdx = = 0 ) |  |
| MtsDcOnly = 0 |  |
| QState = 0 |  |
| … |  |
| } |  |
| } |  |  |

Simulation results

The proposed method is implemented on the top of VTM 8.0.

Results under CTC

Table 1 shows the only RA simulation results of the proposed method compared to VTM 8.0, which are performed under the common test condition (CTC) [3]. It is noted that the proposed method affects only single tree cases so that AI configuration, where coding blocks are always dual tree by default, does not need to be evaluated.

Table 1 Simulation results of the prposed method vs VTM 8.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.07% | 0.03% | 100% | 100% |
| Class A2 | 0.01% | -0.03% | 0.10% | 100% | 100% |
| Class B | 0.01% | -0.02% | 0.04% | 100% | 100% |
| Class C | -0.02% | 0.09% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.05% | 100% | 100% |
| Class D | -0.01% | 0.04% | -0.01% | 100% | 101% |
| Class F | 0.01% | 0.03% | 0.00% | 100% | 100% |

Results under Non-CTC (Dual Tree for I-slice is off by default)

Table 2 shows the both AI and RA simulation results of the proposed method compared to VTM 8.0, which are performed under the CTC except “DualITree” of default configuration is changed to be off. This means that coding blocks in I-slice can be coded as a single tree in this case so that the proposed method affects not only RA configuration but also AI configuration.

Table 2 Simulation results of the prposed method vs VTM 8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class A2 | 0.00% | -0.08% | -0.03% | 100% | 100% |
| Class B | 0.00% | 0.07% | 0.04% | 101% | 100% |
| Class C | 0.01% | 0.04% | -0.06% | 99% | 100% |
| Class E | 0.00% | -0.13% | -0.06% | 101% | 100% |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class D | 0.01% | 0.07% | 0.01% | 100% | 100% |
| Class F | 0.00% | 0.01% | 0.06% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.01% | 0.05% | 100% | 100% |
| Class A2 | 0.01% | 0.00% | 0.01% | 100% | 100% |
| Class B | 0.01% | 0.07% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.11% | -0.20% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | -0.04% | 100% | 100% |
| Class D | -0.01% | -0.11% | 0.06% | 100% | 98% |
| Class F | -0.02% | 0.10% | 0.02% | 100% | 101% |

Conclusion

It was proposed to modify LFNST signalling to remove the redundancy of checking single tree chroma blocks. In the LFNST index parsing condition of the proposed method, the checking processes to update the values of LfnstNotTsFlag, LfnstDcOnly, LfnstZeroOutSigCoeff are skipped in single tree chroma blocks by specifying block tree types and color components. Simulation results show the no coding loss and no increment of running time.

This work was conducted under the project "Research and Development of Next generation video transmission and communication technology for ultra-high definition video application under versatile purpose and environment" of the Ministry of Internal Affairs and Communications (MIC), Japan.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Brussels, BE, 7-17 January 2020.

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 8 (VTM 8)”, JVET-Q2002, Brussels, BE, 7-17 January 2020.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, 19-27 March 2019.

Patent rights declaration

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).