| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0305-v1 |
| --- | --- |

| Title: | CU Level Transform Size Restriction for Adaptive Color Transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lien-Fei Chen, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A | Email: | lienfeichen@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In order to avoid large temporary buffer, it was decided to set max transform size to 32-point at SPS level when adaptive color transform (ACT) is enabled. However, disallowing 64-point transform leads to 1+% performance drop for RGB camera content. To compensate the coding loss, a simple fix is proposed in this contribution that max transform size is constrained to 32-point for CUs in ACT while for other CUs 64-point transform may still be used. The proposed method is implemented on top of VTM-8.0. Compared with VTM-8.0, the coding loss for RGB camera content is recovered. By using our method, the performance impact for RGB camera content is shown as following.

- AI: -1.5%/2.6%/0.1%

- RA: -1.5%/2.7%/-0.4%

- LB: -1.6%/3.4%/0.8%

Introduction

JVET-P0517[1] proposes to include the ACT tool in HEVC into the VVC framework to enhance the efficiency of 444 video coding. Figure 3 illustrates the decoding flowchart with the ACT be applied. Before doing the ACT, all YCoCg three color planes should be decoded at the spatial domain. It means that we need to keep three color buffers before performing the ACT. If the supported coding blocks in ACT is MN, the temporary buffer size is 3MN to buffer the decoded residual data of three planes. Because VVC supports 64-point transform, the inverse ACT must wait for all components of transform size that is 64x64x3 in the worst case.

In last meeting, JVET-Q0162[2] proposed a 64-point transform size restriction method that sps_max_luma_transform_size_64_flag is implicitly set to false when sps_act_enabled_flag is true. In JVET-Q0162 proposal, the maximum transform size is always restricted in 3232 when ACT is true for whole encoded sequences. Consequently, no 64x64 transform is allowed even for coding blocks which are not in ACT mode, which is not efficient especially for RGB camera content. For camera content sequences, the method in [2] has around 1.0%/0.0%/0.8% coding loss for all cases

Proposal

As mentioned in above, no 64x64 transform is allowed even for coding blocks which are not in ACT mode when sps_act_enabled_flag is true. In order to improve the coding efficiency, we propose to do transform unit (TU) split not only based on maximum transform size, but also based on whether ACT is enabled and predefined max transform size for ACT mode.

Therefore, a CU level maximum transform size is defined based on the predefined maximum transform size in SPS header as well as whether ACT mode is enabled. If ACT is used for current CU, the maximum transform size is restricted in 32-point transform size; Otherwise, the maximum transform size signaled in SPS header could be used to achieve coding gain improvement.

The spec. text changing is shown as follows, and our modification is highlight in Yellow.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ChromaArrayType = = 3 && !sps_max_luma_transform_size_64_flag ) |  |
| sps_act_enabled_flag | u(1) |
| … |  |
| } |  |

| transform_tree( x0, y0, tbWidth, tbHeight , treeType, chType ) { | Descriptor |
| --- | --- |
| InferTuCbfLuma = 1 |  |
| if( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !cu_sbt_flag ) { |  |
| if( tbWidth > CuMaxTbSizeY MaxTbSizeY ¦ ¦ tbHeight > CuMaxTbSizeYMaxTbSizeY ) { |  |
| verSplitFirst = ( tbWidth > CuMaxTbSizeYMaxTbSizeY && tbWidth > tbHeight ) ? 1 : 0 |  |
| trafoWidth = verSplitFirst ? ( tbWidth / 2 ) : tbWidth |  |
| trafoHeight = !verSplitFirst ? ( tbHeight / 2 ) : tbHeight |  |
| transform_tree( x0, y0, trafoWidth, trafoHeight, treeType, chType ) |  |
| if( verSplitFirst ) |  |
| transform_tree( x0 + trafoWidth, y0, trafoWidth, trafoHeight, treeType, chType ) |  |
| Else |  |
| transform_tree( x0, y0 + trafoHeight, trafoWidth, trafoHeight, treeType, chType ) |  |
| transform_unit( x0, y0, tbWidth, tbHeight, treeType, 0, chType ) |  |
| } |  |
| } else if( cu_sbt_flag ) |  |

where CuMaxTbSizeY = cu_act_enabled_flag ? 32 : MaxTbSizeY

Simulation Results

The proposed method is implemented on top of VTM-8. In this simulation, IBC, PLT, BDPCM, and inter HashME are all enabled for the anchor and the proposed method. Table 1 summarizes the result of the proposed method in case of dual-tree off.

Table 1: Simulation results in case of dual-tree off

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.1% | 0.4% | 0.4% |
| RGB, mixed content, 1440p & 1080p | -0.1% | -0.2% | -0.1% |
| RGB, Animation, 720p | -0.2% | 0.1% | 0.2% |
| RGB, camera captured, 1080p | -1.5% | 2.6% | 0.1% |
| TGM 1080p | 0.0% | 0.0% | 0.0% |
| TGM 720p | -0.2% | -0.1% | -0.2% |
| Animation | 0.0% | -0.1% | -0.1% |
| Mixed content | -0.1% | -0.1% | -0.1% |
| Camera-Captured content | -1.0% | 0.2% | -0.6% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.3% | 0.7% | 0.7% |
| RGB, mixed content, 1440p & 1080p | 0.0% | -0.1% | 0.1% |
| RGB, Animation, 720p | -0.5% | 0.2% | 0.4% |
| RGB, camera captured, 1080p | -1.5% | 2.7% | -0.4% |
| TGM 1080p | 0.0% | -0.1% | -0.1% |
| TGM 720p | -0.3% | -0.2% | -0.4% |
| Animation | -0.2% | -0.2% | -0.3% |
| Mixed content | -0.3% | -0.1% | -0.2% |
| Camera-Captured content | -0.9% | 0.1% | -0.7% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.8% | 0.9% | 0.9% |
| RGB, mixed content, 1440p & 1080p | -0.4% | -0.1% | 0.3% |
| RGB, Animation, 720p | -2.0% | 0.6% | 0.8% |
| RGB, camera captured, 1080p | -1.6% | 3.4% | 0.8% |
| TGM 1080p | -0.2% | -0.1% | -0.1% |
| TGM 720p | -0.5% | -1.0% | -1.1% |
| Animation | #VALUE! | #VALUE! | #VALUE! |
| Mixed content | -1.0% | -1.0% | -0.7% |
| Camera-Captured content | -0.8% | -0.5% | -0.6% |
| Enc Time[%] | #VALUE! |  |  |
| Dec Time[%] | #VALUE! |  |  |

Conclusion

This contribution proposes a maximum transform size restriction method for each CU based on the adaptive color transform (ACT) mode. By using this adaptive method, the huge temporary buffer between inversed transform and inversed ACT could be eliminated when the ACT mode is signalled. Moreover, the 64-point transform size is still valid for CUs which are not in the ACT mode.

Reference

- X. Xiu, Y.-W. Chen, T.-C. Ma, H.-J. Jhu, X. Wang, “Support of adaptive color transform for 444 video coding in VVC”, ISO/IEC JTC1/SC29/WG11 JVET-P0517, Oct. 2019.

- K. Kondo, M. Ikeda, and T. Suzuki, “A memory issue on adaptive color transform”, ISO/IEC JTC1/SC29/WG11 JVET-Q0162, Jan. 2020.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).