| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0307 |
| --- | --- |

| Title: | AHG8/AHG9: Positioning information SEI message of output independent layers |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Emmanuel Thomas Anna van Buerenplein 1 2595 DA Den Haag The Netherlands | Email: | emmanuel.thomas@tno.nl |
| Source: | TNO |  |  |

_____________________________

# Abstract

This contribution presents a multi-party video conferencing use case where a multi-layer bitstream with independent layers can be used. Each participant views can constitute an independent layer of a single multi-layer bitstream which forms a mosaic or also called grid view.

This contribution proposes defining a new SEI message to signal the positioning information of each output layer for a given output layer set.

In addition, the associated contribution JVET-R0308 provides the software implementation in VTM corresponding to the presented scenario that is:

- Implementation of a multi-layer decoding support

- Composition into a single picture of independent output layers of the same output layer set

# Introduction

This contribution argues for a better support of use cases where multi-layer bitstream with independent layers is used. As an example, multi-party video conferencing is a suitable candidate scenario where multiple participant views can be decoded as part as independent layers of a single multi-layer bitstream.

To this end, this contribution proposed the definition of a new SEI message to signal the positioning information of each output layer for a given output layer set.

In addition, the associated contribution JVET-R0308 provides the software implementation in VTM corresponding to the presented scenario that is:

- Implementation of a multi-layer decoding support

- Composition into a single picture of independent output layers of the same output layer set

Note that the r1 revision aligns the signalling with the software implementation provided in JVET-R0308.

# Single output layer set with independent layers

## Use case: Multi-party video conferencing

Video conferencing in the workplace is a well-established tool for communication. It is very common to run video calls with small groups from 5 to 20. In these scenarios, the participants may share their camera feed for a more intuitive meeting. Figure 1 shows an example of a video call with five participants wherein the four remote participants are combined into a 2x2 grid.

Figure 1 - Multi-party video conferencing system example

In term of architectures[1], video conferencing system may rely on three different principles that are:

- Mesh: where all the participants are sending and receiving video streams to each other

- Multipoint Control Unit (MCU): where participants are sending their audio and video to a central entity which then mix the incoming streams and send back a new video stream to each participant.

- Selective Forwarding Unit (SFU) – where a participant sends his media to a central entity, who routes all incoming media as he sees fit to participants – each one of them receiving usually more than a single stream

Figure 2 - Mesh, MCU and SFU principles [2]

Regardless of the architecture, we can assume that, at the input of the decoder residing in each participant’s device, the bitstream should contain a mosaic of the participant’s view as shown in the above screenshot. In this contribution, we assume that these views constitute independent layers.

## Syntax example of the multi-layer bitstream received of the mosaic

Taking the example in 2.1 above, we suppose that the bitstream decoded on the local device contains four layers where each layer is a participant view. Each layer is an independent layer since it was encoded separately by each participant of the video call. In addition, these layers do not contain similar visual information and as a result, it is not opportune to allow inter-layer prediction.

As for output layer and output layer set, the 0-th output layer set (OLS) contains only the lowest layer (as mandated by the latest draft specification) and the 1st OLS contains the four layers which are also output layers.

The following table shows the corresponding signalled VPS with the following color code; the signalled syntax in black front, the non signalled elements in grey font and the values of the relevant syntax elements are highlighted in green.

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 = 3 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag = 1 | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] = [0, 1 ,2 ,3] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) { |  |  |
| for( j = 0; j < i; j++ ) |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| max_tid_ref_present_flag[ i ] | u(1) |  |
| if( max_tid_ref_present_flag[ i ] ) |  |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| each_layer_is_an_ols_flag = 0 | u(1) |  |
| if( !each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| ols_mode_idc | u(2) |  |
| if( ols_mode_idc = = 2 ) { |  |  |
| num_output_layer_sets_minus1 = 1 | u(8) |  |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| ols_output_layer_flag[ i ][ j ] = 1 | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| vps_num_ptls_minus1 | u(8) |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_alignment_zero_bit /* equal to 0 */ | f(1) |  |
| for( i = 0; i <= vps_num_ptls_minus1; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( vps_num_ptls_minus1 > 0 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < TotalNumOlss; i++ ) { |  |  |
| if( NumLayersInOls[ i ] > 1 ) { |  |  |
| ols_dpb_pic_width[ i ] | ue(v) |  |
| ols_dpb_pic_height[ i ] | ue(v) |  |
| if( vps_num_dpb_params > 1 ) |  |  |
| ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| if( !each_layer_is_an_ols_flag ) |  |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 + 1 != TotalNumOlss && num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

# Proposal: Signalling of output layer composition in output layer set

For composing a single picture based on the decoded pictures of each output layer, the bitstream needs to contain the necessary metadata for describing the arrangement of each decoded picture. The composition of the different pictures may be executed by a process outside the decoder.

It is thus proposed to define the following SEI message called independent_layer_info.

For more information on the software implementation related to this signalling, please refer to JVET-R0308.

## Specification changes

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The AU containing the SEI message |
| DU information | The AU containing the SEI message |
| Scalable nesting | Depending on the scalable-nested SEI messages. Each scalable-nested SEI message has the same persistence scope as if the SEI message was not scalable-nested |
| Subpicture level information | The CLVS containing the SEI message |
| Independent layer info | The CLVS containing the SEI message |

D.x Independent layer info SEI message

### D.x.1 Independent layer info SEI message syntax

| independent_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| boundary_identifier_north_present_flag | u(1) |
| if(boundary_identifier_north_present_flag) |  |
| boundary_identifier_north | u(16) |
| boundary_identifier_east_present_flag |  | u(1) |  |
| if(boundary_identifier_east_present_flag) |  |  |  |
| boundary_identifier_east | u(16) |
| boundary_identifier_south_present_flag |  | u(1) |  |
| if(boundary_identifier_south_present_flag) |  |  |  |
| boundary_identifier_south | u(16) |
| boundary_identifier_west_present_flag |  | u(1) |  |
| if(boundary_identifier_west_present_flag) |  |  |  |
| boundary_identifier_west | u(16) |
| } |  |

boundary_identifier_north_present_flag, boundary_identifier_east_present_flag, boundary_identifier_south_present_flag and boundary_identifier_west_present_flag equal to 1 specify that the SEI message contains a boundary identifier, respectively, for the north, east, south and west boundary.

boundary_identifier_north, boundary_identifier_east, boundary_identifier_south and boundary_identifier_west specifiy the boundary identifier, respecitvely, at the north, east, south and west boundary of the decoded picture of the associated layer; the associated layer being the layer whose nuh_layer_id is equal to the nuh_layer_id of the SEI message. If not present, the boundary identifiers respecitvely at the north, east, south and west boundary of the decoded picture of the associated layer are not defined.

Figure 3 - Representation of where the bououndaries are on the layers

For two layers, the i-th and j-th layers, when the pair of the boundary_identifier_north value of the i-th layer and the boundary_identifier_south value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary north/south. For i-th and j-th layers, when the pair of the boundary_identifier_east value of the i-th layer and the boundary_identifier_west value of the j-th layer are equal then the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the composed output picture and they share a common boundary at the boundary east/west.

Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the composed picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the composed picture.

# Patent rights declaration(s)

TNO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- M. Westerlund and S. Wenger,RFC 7667: RTP Topologies, IETF, Nov.2015. [Online]. Available: https://datatracker.ietf.org/doc/rfc7667/

- https://webrtcglossary.com/sfu/