| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0309 |
| --- | --- |

| Title: | [AHG16] Clean-up on palette predictor update for local dual tree. |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Sunmi Yoo Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to clean-up regarding predictor palette update for local dual tree structure. Three alternative clean-up methods are suggested as described below:

- Clean-up 1. There is mismatch between SPEC and SW. Here, propose to fix SPEC to align with SW regarding local dual tree predictor palette update process.

- Clean-up 2. Update predictor palette entry only for luma component but not for chroma component in local dual tree structure.

- Clean-up 3. Remove palette predictor update process of local dual tree structure.

The suggested clean-up method 2 shows {0.00%, 0.07%, 0.XX%} and method 3 shows{0.XX%, 0.XX%, X.XX%} coding performance respectively at ClassF/SCC under CTC with palette on. It is asserted that proposed clean-up methods save H/W power consumption with closed to zero coding loss and also improve decoding latency by removing predictor palette update process for small block.

Problem statement

Palette prediction mode supports non-4:4:4 format for 4:4:4 profile. In order to support non-4:4:4 format, predictor palette is updated using default value for local dual tree[1]. In VVC draft 8, it is described that the predictor palette entry could be updated only after decoding chroma coding block which is coded by palette in local dual tree structure. It means that the predictor palette updating is not applied if the chroma coding block is not coded as palette prediction mode in local dual tree structure. But in SW, the predictor palette update is applied after decoding every component in this case. The update process for predictor palette entries for local dual tree structure causes decoding latency due to reusing the palette predictor entries from the previous palette coded block.

Proposal

This contribution proposes three suggestions.

First, there is a mismatch between Spec and SW. Method 1 proposes to fix the mismatch.

Method1: to align SPEC with SW.

- Fix SPEC to align with SW regarding local dual tree predictor palette update process.

When localDualTree is equal to 1, the following applies: - If treeType is equal to DUAL_TREE_LUMA, the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1: CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (450) CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (451) - Otherwise (if treeType is equal to DUAL_TREE_CHROMA), the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1: CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (452) - The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows: CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ] (453) startComp = 0 (454) numComps = 3 (455) maxNumPalettePredictorSize = 63 (456) When one of the following conditions is true: - cIdx is equal to 0 and numComps is equal to 1; - cIdx is equal to 2; - localDualTree is equal to 1; the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows: for( i = 0; i < CurrentPaletteSize[ startComp ]; i++ ) for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ ) newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ] newPredictorPaletteSize = CurrentPaletteSize[ startComp ] for( i = 0; i < PredictorPaletteSize[ startComp ] && newPredictorPaletteSize < maxNumPalettePredictorSize; i++ ) if( !PalettePredictorEntryReuseFlags[ i ] ) { for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ ) (457) newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] = PredictorPaletteEntries[ cIdx ][ i ] newPredictorPaletteSize++ } for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ ) for( i = 0; i < newPredictorPaletteSize; i++ ) PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ] PredictorPaletteSize[ startComp ] = newPredictorPaletteSize When qtbtt_dual_tree_intra_flag is equal to 0 or slice_type is not equal to I, the following applies: PredictorPaletteSize[ 1 ] = newPredictorPaletteSize (458) It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to maxNumPalettePredictorSize, inclusive.

Two alternative methods are introduced. Method 2 proposed to update predictor palette luma coding block of local dual tree structure as follow:

When localDualTree is equal to 1, the following applies: - If treeType is equal to DUAL_TREE_LUMA, the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1: CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (450) CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (451) - Otherwise (if treeType is equal to DUAL_TREE_CHROMA), the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1: CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (452) - The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows: CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ] (453) startComp = 0 (454) numComps = 3 (455) maxNumPalettePredictorSize = 63 (456) When one of the following conditions is true: - cIdx is equal to 0 and numComps is equal to 1; - cIdx is equal to 2; - localDualTree is equal to 1 and cIdx is equal to 0; the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows: for( i = 0; i < CurrentPaletteSize[ startComp ]; i++ ) for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ ) newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ] newPredictorPaletteSize = CurrentPaletteSize[ startComp ] for( i = 0; i < PredictorPaletteSize[ startComp ] && newPredictorPaletteSize < maxNumPalettePredictorSize; i++ ) if( !PalettePredictorEntryReuseFlags[ i ] ) { for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ ) (457) newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] = PredictorPaletteEntries[ cIdx ][ i ] newPredictorPaletteSize++ } for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ ) for( i = 0; i < newPredictorPaletteSize; i++ ) PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ] PredictorPaletteSize[ startComp ] = newPredictorPaletteSize When qtbtt_dual_tree_intra_flag is equal to 0 or slice_type is not equal to I, the following applies: PredictorPaletteSize[ 1 ] = newPredictorPaletteSize (458) It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to maxNumPalettePredictorSize, inclusive.

Method 3 proposes to remove predictor palette update process of local dual tree structure for complexity reduction.

When localDualTree is equal to 1, the following applies: - If treeType is equal to DUAL_TREE_LUMA, the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1: CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (450) CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (451) - Otherwise (if treeType is equal to DUAL_TREE_CHROMA), the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1: CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 ) (452) - The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows: CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ] (453) startComp = 0 (454) numComps = 3 (455) maxNumPalettePredictorSize = 63 (456) When localDualTree is equal to 0 and one of the following conditions is true: - cIdx is equal to 0 and numComps is equal to 1; - cIdx is equal to 2; the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows: for( i = 0; i < CurrentPaletteSize[ startComp ]; i++ ) for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ ) newPredictorPaletteEntries[ cIdx ][ i ] = CurrentPaletteEntries[ cIdx ][ i ] newPredictorPaletteSize = CurrentPaletteSize[ startComp ] for( i = 0; i < PredictorPaletteSize[ startComp ] && newPredictorPaletteSize < maxNumPalettePredictorSize; i++ ) if( !PalettePredictorEntryReuseFlags[ i ] ) { for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++ ) (457) newPredictorPaletteEntries[ cIdx ][ newPredictorPaletteSize ] = PredictorPaletteEntries[ cIdx ][ i ] newPredictorPaletteSize++ } for( cIdx = startComp; cIdx < ( startComp + numComps ); cIdx++ ) for( i = 0; i < newPredictorPaletteSize; i++ ) PredictorPaletteEntries[ cIdx ][ i ] = newPredictorPaletteEntries[ cIdx ][ i ] PredictorPaletteSize[ startComp ] = newPredictorPaletteSize When qtbtt_dual_tree_intra_flag is equal to 0 or slice_type is not equal to I, the following applies: PredictorPaletteSize[ 1 ] = newPredictorPaletteSize (458) It is a requirement of bitstream conformance that the value of PredictorPaletteSize[ startComp ] shall be in the range of 0 to maxNumPalettePredictorSize, inclusive.

Experimental result

The tests are experimented under CTC with PLT enabling.

Conclusion

It asserts to disable predictor palette update for local dual tree structure in order to save power consumption.

Reference

[1] R.-L. Liao, M. Sarwer, J. Chen, Y. Ye, J. Luo (Alibaba), Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm), “CE2-related: Palette mode for non 4:4:4 color format”, JVET-Q0504, 17th Meeting: Brussels, Jan 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).