| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0310 |
| --- | --- |

| Title: | [AHG16] Clean-up by removing parsing dependency for palette |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Sunmi Yoo Naeri Park Seunghwan Kim Jaehyun Lim | Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

The palette_predictor_run is used to determine the number of zeros that precede a non-zero entry in the array of palette predictor entry reuse flags (i.e., PalettePredictorEntryReuseFlags). Hence, in order to parse palette_predictor_run syntax, the predictor palette size, which is decided after previous coding unit decoding, is required to be available. Therefore, this contribution proposes to remove palette syntax parsing dependency by removing the availability of the predictor palette size. The experiment results show {0.00%, 0.0X%,0.0X%} under non-4:2:0 palette test configuration in JVET-Q2013.

Introduction

Palette represents color value for each sample using palette entries. The palette entries for current coding unit are constructed based on newly signaled palette entries, escaped value and palette entries which is in predictor palette. Currently palette_predictor_run syntax is used to indicate palette entries in predictor palette that is updated after decoding the previous palette coding unit. To save termination bit, this syntax are bounded by the number of predictor palette entry size even though it causes parsing dependency.

Proposal

This contribution proposes to remove parsing dependency for palette_predictor_run syntax by replacing predictor palette size (PredictorPaletteSize) with maximum number of palette entries (maxNumPaletteEntries). It is helpful for independent entropy decoder in real H/W implementation.

| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) { | Descriptor |
| --- | --- |
| startComp = ( treeType = = DUAL_TREE_CHROMA ) ? 1 : 0 |  |
| numComps = ( treeType = = SINGLE_TREE ) ? ( ChromaArrayType = = 0 ? 1 : 3 ) : ( treeType = = DUAL_TREE_CHROMA ) ? 2 : 1 |  |
| maxNumPaletteEntries = ( treeType = = SINGLE_TREE ) ? 31 : 15 |  |
| palettePredictionFinished = 0 |  |
| NumPredictedPaletteEntries = 0 |  |
| for( predictorEntryIdx = 0; predictorEntryIdx < maxNumPaletteEntries PredictorPaletteSize[ startComp ] && !palettePredictionFinished && NumPredictedPaletteEntries < maxNumPaletteEntries; predictorEntryIdx++ ) { |  |
| palette_predictor_run | ae(v) |
| if( palette_predictor_run != 1 ) { |  |
| if( palette_predictor_run > 1 ) |  |
| predictorEntryIdx += palette_predictor_run − 1 |  |
| PalettePredictorEntryReuseFlags[ predictorEntryIdx ] = 1 |  |
| NumPredictedPaletteEntries++ |  |
| } else |  |
| palettePredictionFinished = 1 |  |
| } |  |

From this proposed method, if the predictor palette entry size is smaller than maxNumPaletteEntries and the last predictor palette entry is used for the current coding block then the termination run code should be signaled other than VTM-8.0. Otherwise, if the last predictor palette entry is not used for the current coding block then signaled palette_predictor_run syntax is identical with VTM-8.0.

Figure 1. palette_predictor_run coding in VTM-8.0 when last predictor palette entry is used.

Figure 2. palette_predictor_run coding in proposed method when last predictor palette entry is used.

Figure 3. palette_predictor_run coding both in proposed method and VTM-8.0 when last predictor palette entry is not used.

Experimental result

The test are experimented under non-4:2:0 CTC defined by JVET-Q2013.

Conclusion

It asserts that parsing dependency for palette_predictor_run syntax should be removed

Reference

[1] R.-L. Liao, M. Sarwer, J. Chen, Y. Ye, J. Luo (Alibaba), Y.-H. Chao, C.-H. Hung, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm), “CE2-related: Palette mode for non 4:4:4 color format”, JVET-Q0504, 17th Meeting: Brussels, Jan 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).