| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0312-v2 |
| --- | --- |

| Title: | AHG2/AHG16: A fix on chroma ALF virtual boundary position |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Li Zhang2, Hongbin Liu1, Kai Zhang2, Zhipin Deng1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com lizhang.idm@bytedance.com liuhongbin.01@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In current VVC, luma and chroma ALF virtual boundaries are always set to four and two lines above the bottom luma CTB and chroma CTB boundaries, respectively. Such a design works well for 4:2:0 colour format since the height of a chroma CTB is half of that of a luma CTB. However, for 4:2:2 and 4:4:4 colour formats in which heights of a luma CTB and a chroma CTB are equal, it is asserted that the design could result in misaligned ALF virtual boundaries for luma and chroma samples. This contribution proposes to align ALF virtual boundaries of luma and chroma components for 4:2:2 and 4:4:4 colour formats. The simulation results for 4:4:4 colour format screen sequences, 4:4:4 and 4:2:2 natural sequences following the common test conditions are summarized as follows:

Dual tree on:

AI: {0.00%, -0.01%, 0.00%}; RA: {-0.02%, -0.02%, -0.03%}; LDB: {0.02%, -0.01%, 0.08%}

Dual tree off:

AI: {0.00%, 0.00%, 0.00%}; RA: {0.02%, 0.02%, -0.02%}; LDB: {0.01%, 0.07%, 0.05%}

Natural sequences:

YUV 4:4:4, AI: {0.00%, -0.02%, -0.04%}; RA: {0.01, -0.07%, 0.03%}; LDB: {}

YUV 4:2:2, AI: {0.00%, 0.00%, -0.02%}; RA: {0.01, -0.11%, -0.08%}; LDB: {0.00%, -0.15%, 0.01%}

Introduction

In current VVC [1], ALF virtual boundary is designed to reduce the line buffer size required in ALF, wherein samples across the ALF virtual boundary are not used in ALF.

Table 1: VB positions of luma ALF and chroma ALF in current VVC

| Colour format | Luma ALF with CTB height H |  | Chroma ALF |  |
| --- | --- | --- | --- | --- |
|  | CTB height | VB position | CTB height | VB position |
| 4:2:0 | H | H − 4 | H/2 | H/2 − 2 |
| 4:2:2 | H | H − 4 | H | H − 2 |
| 4:4:4 | H | H − 4 | H | H − 2 |

Table 1 summarizes the position of luma and chroma ALF Virtual Boundaries (VBs) for different colour formats. It could be noticed that the luma and chroma ALF VBs are always set to four and two lines above the bottom luma CTB and chroma CTB boundaries, respectively. Such a design works well for 4:2:0 colour format since the height of a chroma CTB is half of that of a luma CTB. However, the design could result in misaligned handling for luma and chroma samples, as shown in Figure 1. Assuming the CTU size to be 64×64 and 4:4:4 colour format is used, when processing luma samples, for the first CTU row, the top 60 luma rows are processed, followed by 64 luma rows (with 4 rows from the first CTU row and 60 rows from the second CTU row). However, for the chroma samples, for the first CTU row, the top 62 chroma rows are processed, followed by 64 chroma rows (with 2 rows from the first CTU row and 62 rows from the second CTU row). It will result in difficulties for both hardware and software implementation.

Figure 1. Example of ALF VBs for luma and chroma CTBs with 4:4:4 colour format

Proposed method

To address the above issues, it is proposed to align ALF virtual boundaries of luma and chroma components for 4:2:2 and 4:4:4 colour formats. Specifically, both luma and chroma ALF virtual boundaries are set equal to 4 lines above the bottom CTU boundary, as shown in Table 2 with changes highlighted in yellow.

Table 2: Proposed chroma ALF VB position

| Colour format | Luma ALF with CTB height H |  | Chroma ALF |  |
| --- | --- | --- | --- | --- |
|  | CTB height | VB position | CTB height | VB position |
| 4:2:0 | H | H − 4 | H/2 | H/2 − 2 |
| 4:2:2 | H | H − 4 | H | H – 4 |
| 4:4:4 | H | H − 4 | H | H − 4 |

Simulation results

The proposed method has been implemented on top of VTM-8.0 [2]. Simulations are performed following the common test conditions [3]. The results for Dual tree on and Dual tree off are shown in Table 3 and Table 4, respectively. The test results under non-420 CTC [3] for YUV 4:4:4 and YUV 4:2:2 natural sequences are shown in Table 5. More detailed results could be found in the attached excel sheets.

Table 3: Coding performance of Dual tree on

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 99% | 98% |
| TGM 720p | 0.00% | 0.00% | -0.01% | 96% | 96% |
| Animation | 0.00% | 0.01% | 0.01% | 100% | 102% |
| Mixed content | 0.00% | -0.01% | 0.01% | 100% | 99% |
| Camera-Captured content | 0.00% | -0.03% | -0.04% | 94% | 96% |
| Overall | 0.00% | -0.01% | 0.00% | 98% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | -0.01% | -0.01% | 99% | 99% |
| TGM 720p | 0.00% | -0.05% | -0.09% | 96% | 96% |
| Animation | 0.04% | 0.01% | 0.06% | 97% | 97% |
| Mixed content | -0.10% | -0.02% | -0.03% | 95% | 96% |
| Camera-Captured content | -0.05% | 0.01% | -0.06% | 105% | 104% |
| Overall | -0.02% | -0.02% | -0.03% | 98% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | -0.08% | -0.04% | 101% | 104% |
| TGM 720p | 0.07% | -0.04% | 0.30% | 100% | 99% |
| Animation | 0.06% | -0.02% | 0.08% | 95% | 95% |
| Mixed content | -0.03% | 0.05% | 0.06% | 96% | 97% |
| Camera-Captured content | 0.01% | 0.10% | -0.11% | 99% | 98% |
| Overall | 0.02% | -0.01% | 0.08% | 98% | 99% |

Table 4: Coding performance of Dual tree off

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.01% | 100% | 99% |
| TGM 720p | 0.00% | 0.01% | 0.00% | 100% | 101% |
| Animation | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Mixed content | 0.00% | 0.00% | 0.02% | 99% | 97% |
| Camera-Captured content | 0.00% | -0.03% | -0.05% | 101% | 103% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.03% | 0.03% | 0.05% | 99% | 100% |
| TGM 720p | 0.00% | -0.02% | -0.04% | 101% | 100% |
| Animation | 0.00% | 0.01% | -0.06% | 102% | 101% |
| Mixed content | 0.05% | 0.12% | 0.06% | 99% | 100% |
| Camera-Captured content | 0.00% | -0.04% | -0.15% | 100% | 101% |
| Overall | 0.02% | 0.02% | -0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | -0.05% | -0.02% | 102% | 101% |
| TGM 720p | 0.01% | 0.29% | 0.06% | 102% | 99% |
| Animation | -0.01% | -0.04% | 0.05% | 100% | 99% |
| Mixed content | 0.01% | 0.07% | 0.14% | 100% | 99% |
| Camera-Captured content | 0.00% | 0.01% | 0.01% | 98% | 101% |
| Overall | 0.01% | 0.07% | 0.05% | 101% | 100% |

Table 5: Coding performance for YUV 4:4:4 and YUV 4:2:2 natural squences

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.00% | -0.02% | -0.04% | 98% | 97% |
| YUV 4:2:2 | 0.00% | 0.00% | -0.02% | 99% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 | 0.01% | -0.07% | 0.03% | 100% | 100% |
| YUV 4:2:2 | 0.01% | -0.11% | -0.08% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| YUV 4:4:4 |  |  |  |  |  |
| YUV 4:2:2 | 0.00% | -0.15% | 0.01% | 100% | 100% |

Conclusions

It is proposed to align ALF virtual boundaries of luma and chroma components for 4:2:2 and 4:4:4 colour formats. It is recommended adopting the proposed method into the next version of VVC.

Proposed changes to working draft

The suggested specification changes on top of JVET-Q2001-vE [1] is described as follows. Deleted text is marked with red strikethrough, and newly added text is highlighted in green.

8.8.5.4 Coding tree block filtering process for chroma samples

…

  - The variable applyAlfLineBufBoundary is derived as follows:

  - If the bottom boundary of the current coding tree block is the bottom boundary of the picture and pic_height_in_luma_samples − ( yCtbC * SubHeightC ) < CtbSizeY − 4, applyAlfLineBufBoundary is set equal to 0.

  - Otherwise, applyAlfLineBufBoundary is set equal to 1.

The vertical sample position offsets y1, y2 and the variable alfShiftC are specified in Table 4546 according to the vertical chroma sample position y and applyAlfLineBufBoundary.

…

Table 46 – Specification of y1, y2 and alfShiftC according to the vertical chroma sample position y and applyAlfLineBufBoundary

| Condition | alfShiftC | y1 | y2 |
| --- | --- | --- | --- |
| ( y = = ctbHeightC − 2 ¦ ¦ y = = ctbHeightC − 3 )( y = = ( ctbSizeY − 4 ) / SubHeightC − 1 ¦ ¦ y = = ( ctbSizeY − 4 ) / SubHeightC ) && ( applyAlfLineBufBoundary = = 1 ) | 10 | 0 | 0 |
| ( y = = ctbHeightC − 1 ¦ ¦ y = = ctbHeightC − 4 )( y = = ( ctbSizeY − 4 ) / SubHeightC − 2 ¦ ¦ y = = ( ctbSizeY − 4 ) / SubHeightC + 1 ) && ( applyAlfLineBufBoundary = = 1 ) | 7 | 1 | 1 |
| Otherwise | 7 | 1 | 2 |

…

Reference

- B. Bross, J. Chen, S. Liu, and Y. Wang, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001-vE.

- VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-8.0

- Y. Chao, Y. Sun, J. Xu, and X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats,” Document of Joint Video Experts Team, JVET-Q2013.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).