| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0320-v2 |
| --- | --- |

| Title: | AHG11: Maximum QP for escape value in palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Weijia Zhu Kai Zhang 8910 University Center Ln, San Diego, CA 92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

Currently, EG(5) is used to code escape values in palette mode. It is observed that when QP is larger than or equal to 23, the bit length of escape values reaches its minimal, i.e. 6. Thus, it is proposed to limit the maximum QP to be 23 to reduce quantization distortion.

# Introduction

EG(5) is used to code escape values in palette mode. When base QP is 23, the maximum quantized escape value is 28. It already reaches the minimal bit length for EG(5). Beyond QP 23, any code word costs the same number of bits and distortion is increased when QP becomes larger.

Table I. Bit length of EG(5).

| Base QP | … | 18 | 19 | 20 | 21 | 22 | 23 | 24 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max escape value | … | 51 | 45 | 40 | 36 | 32 | 28 | 25 | … |
| Max bits of EG(5) | … | 8 | 8 | 8 | 8 | 8 | 6 | 6 | … |
| Min bits of EG(5) | … | 6 | 6 | 6 | 6 | 6 | 6 | 6 | … |

# Proposed methods

It is proposed to set the maximum base QP for escape values to be 23. As mentioned above, when base QP is beyond 23, the bit cost does not decrease but the distortion still increases when QP becomes larger. By restriction of the maximum base QP to be 23, we can achieve free coding gains.

# Performance

The performance compared with VTM-8.0 following the test conditions defined in JVET-P2022 with DualITree=0 is shown below.

Table II. Performance of the proposed method compared with VTM-8.0.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.03% | -0.08% | -0.07% | 99% | 98% |
| TGM 720p | 0.00% | 0.04% | -0.05% | 104% | 104% |
| Animation | -0.01% | 0.00% | 0.01% | 101% | 101% |
| Mixed content | -0.04% | -0.06% | -0.05% | 98% | 98% |
| Camera-Captured content | -0.02% | 0.00% | -0.01% | 100% | 101% |
| Overall | -0.02% | -0.02% | -0.04% | 101% | 101% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | -0.11% | -0.09% | 99% | 99% |
| TGM 720p | -0.04% | -0.10% | -0.09% | 99% | 100% |
| Animation | 0.01% | 0.01% | -0.02% | 101% | 102% |
| Mixed content | -0.02% | -0.15% | -0.17% | 101% | 101% |
| Camera-Captured content | -0.01% | 0.01% | -0.12% | 102% | 101% |
| Overall | -0.01% | -0.08% | -0.10% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.02% | 0.00% | -0.05% | 100% | 99% |
| TGM 720p | 0.14% | 0.09% | -0.09% | 96% | 98% |
| Animation | 0.02% | 0.05% | 0.01% | 100% | 101% |
| Mixed content | 0.08% | -0.20% | -0.01% | 102% | 100% |
| Camera-Captured content | -0.02% | -0.18% | -0.28% | 99% | 100% |
| Overall | 0.06% | -0.03% | -0.07% | 99% | 99% |

# Text changes

In the following, based on JVET-Q2001-vE document, we marked the newly added in turquoise.

8.4.5.3 Decoding process for palette mode

…

- Otherwise (bIsEscapeSample is equal to 1), the following ordered steps apply:

- The quantization parameter qP is derived as follows:

- If cIdx is equal to 0,

qP = Min( Max( QpPrimeTsMin, Qp′Y ), 23 + QpBdOffset )	(444)

- Otherwise, if cIdx is equal to 1,

qP = Min( Max( QpPrimeTsMin, Qp′Cb ), 23 + QpBdOffset )	(445)

- Otherwise (cIdx is equal to 2),

qP = Min( Max( QpPrimeTsMin, Qp′Cr ), 23 + QpBdOffset )	(446)

…

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).