| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0321-v1 |
| --- | --- |

| Title: | AHG3: Chroma QP table bug-fix and CTC update for RGB coding in VTM-8.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ji-Zheng Xu1, Li Zhang1, Weijia Zhu1 Xiaoyu Xiu2, Yi-Wen Chen2, Tsung-Chuan Ma2, Hong-Jheng Jhu2, Xianglin Wang2 | Email: | xujizheng@bytedance.com xiaoyuxiu@kwai.com |
| Source: | Bytedance Inc.1, Kwai Inc1. |  |  |

_____________________________

# Abstract

In VTM-8.0, when input video is in non-420 chroma format, one encoder configuration parameter UseIdentityTableForNon420Chroma is enabled which aims at using identical QPs for both luma and chroma components. This is done by manipulating the values of chroma QP table syntax elements in SPS. However, it was found that the currently used values of chroma QP table syntax could unintentionally lead to unequal luma and chroma QPs when the input QP is larger than 26. This contribution provides an encoder fix to appropriately set the equal chroma QP table for non-420 videos. Simulation results are summarized as follows:

For RGB SCC content, the average {G, B, R} BD-rate impacts of the proposed software fix are {−2.10%, 0.80%, 0.61%} and {−2.59%, 0.43%, 0.20%} for AI and RA configurations, respectively. For RGB natural content, the corresponding BD-rate changes are {−10.69%, 2.23%, 0.87%} and {−5.85%, 0.93%, −0.06%} for AI and RA configurations, respectively.

With the above results, it is suggested updating the current CTC for RGB coding.

# Introduction

In VVC draft 8 [1], chroma QP is derived from luma QP based on one chroma QP table which is signaled in SPS. The chroma QP table defines a piecewise luma-to-chroma QP mapping function by signalling a number of pivot points based on the following syntax elements:

| if( ChromaArrayType != 0 ) { |  |
| --- | --- |
| sps_joint_cbcr_enabled_flag | u(1) |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

At decoder, the chroma QP mapping table ChromaQpTable[ i ] for i = 0..numQpTables − 1 is built as follows:

Table 1 The chroma QP table generation in VVC draft 8

qpInVal[ i ][ 0 ] = qp_table_start_minus26[ i ] + 26
qpOutVal[ i ][ 0 ] = qpInVal[ i ][ 0 ]
for( j = 0; j  <=  num_points_in_qp_table_minus1[ i ]; j++ ) {
	qpInVal[ i ][ j + 1 ] = qpInVal[ i ][ j ] + delta_qp_in_val_minus1[ i ][ j ] + 1
	qpOutVal[ i ][ j + 1 ] = qpOutVal[ i ][ j ] + ( delta_qp_in_val_minus1[ i ][ j ] ^ delta_qp_diff_val[ i ][ j ] )
}
ChromaQpTable[ i ][ qpInVal[ i ][ 0 ] ] = qpOutVal[ i ][ 0 ]
for( k = qpInVal[ i ][ 0 ] − 1; k  >=  −QpBdOffset; k − − )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffset, 63, ChromaQpTable[ i ][ k + 1 ] − 1 )	(1)
for( j = 0; j  <=  num_points_in_qp_table_minus1[ i ]; j++ ) {
	sh = ( delta_qp_in_val_minus1[ i ][j ] + 1 )  >>  1
	for( k = qpInVal[ i ][ j ] + 1, m = 1; k  <=  qpInval[ i ][ j + 1 ]; k++, m++ )
		ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ qpInVal[ i ][ j ] ] +
			( ( qpOutVal[ i ][j + 1] − qpOutVal[ i ][j ] ) * m + sh ) / ( delta_qp_in_val_minus1[ i ][ j ] + 1 )
}
for( k = qpInVal[ i ][ num_points_in_qp_table_minus1[ i ] + 1 ] + 1; k  <=  63; k++ )
	ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffset, 63, ChromaQpTable[ i ][ k − 1 ] + 1 )

,

Additionally, when the input is non-420 video, one encoder configuration parameter UseIdentityTableForNon420Chroma is enabled which aims at using identical QPs for coding the luma and chroma components of the input video. Specifically, when the parameter is enabled, the chroma QP table syntax elements are set to the following values:

- same_qp_table_for_chroma = 1

- qp_table_start_minus26[0] = 0

- num_points_in_qp_table_minus1[0] = 0                                              (1)

- delta_qp_in_val_minus1[0][0] = 0

- delta_qp_diff_val[0][0] = 0

However, by following the chroma QP table generation process in Table 1, the above set values in equation (1) actually lead to one two-piece luma-to-chroma QP mapping function as defined as:

QPc=QPL,-BDOffset≤QPL≤26   QPL-1,QPL>26                    (2)

As can be seen in equation (2), unequal QP values are used for coding luma and chroma components when the input QP is larger than 26.

# Proposal

To solve above issue, this contribution provides cfg files to correctly set the equal chroma QP mapping table for RGB videos. Specifically, we set UseIdentityTableForNon420Chroma to 0 and define the chroma QP mapping table as follows:

SameCQPTablesForAllChroma           : 1

QpInValCb                           : 22 62

QpOutValCb                          : 22 62

# Simulations

The proposed bug-fix was implemented on top of VTM-8.0 [2] by following common test conditions (CTCs) for non-420 chroma format [3]. The screen and natural video contents in RGB formats are tested.

Table 2 The BD-rate impact of the proposed change for RGB 4:4:4 format.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM1080p | -1.15% | 0.34% | 0.37% | 97% | 97% |
| TGM720p | -2.21% | 0.38% | 0.19% | 98% | 97% |
| Animation | -3.10% | 1.43% | 0.93% | 98% | 100% |
| Mixed Content | -1.93% | 1.06% | 0.96% | 100% | 100% |
| Natural | -10.69% | 2.23% | 0.87% | 95% | 98% |
| Overall | -5.19% | 1.28% | 0.67% | 97% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-8.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM1080p | -1.49% | 0.38% | 0.41% | 95% | 98% |
| TGM720p | -2.53% | 0.12% | -0.25% | 97% | 103% |
| Animation | -3.75% | 1.01% | 0.52% | 95% | 98% |
| Mixed Content | -2.58% | 0.19% | 0.13% | 95% | 100% |
| Natural | -11.71% | 1.86% | -0.49% | 93% | 98% |
| Overall | -5.85% | 0.93% | -0.06% | 95% | 99% |

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020, Brussels, Belgium.

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-Q2013, Jan. 2020, Brussels, Belgium.

# Patent rights declaration(s)

ByteDance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).