| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0324 |
| --- | --- |

| Title: | AHG9: On syntax signaling conditions in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Wei Chen, Xianglin Wang | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this proposal, it is proposed to signal mvd_l1_zero_flag, ph_temporal_mvp_enabled_flag, ph_disable_bdof_flag and ph_disable_dmvr_flag depending on the whether the coded slices associated with the PH are bi-predictive, to prevent redundant signaling due to non-applicable syntax values in the picture header.

# Current draft text

In current VVC, mvd_l1_zero_flag is signaled in the picture header (PH) without any conditional constraint. However, the feature controlled by the flag mvd_l1_zero_flag is only applicable when the slice is a bi-predictive slice (B-slice). Therefore, the flag signaling is redundant when the slice associated with the picture header is not a B-slice.

Similarly, ph_temporal_mvp_enabled_flag, ph_disable_bdof_flag and ph_disable_dmvr_flag are signaled in the PH only when the corresponding enabling flags (e.g. ph_temporal_mvp_enabled_flag , sps_bdof_pic_present_flag, sps_dmvr_pic_present_flag) are true, respectively. As shown in the table below, however, the features controlled by the flags ph_temporal_mvp_enabled_flag, ph_disable_bdof_flag and ph_disable_dmvr_flag are only applicable when the slice is a bi-predictive slice (B-slice). Therefore, the signaling of these two flags is also redundant or useless when the slices associated with the picture header is not a B-slice.

| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| --- | --- |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| mvd_l1_zero_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| ph_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| ph_disable_dmvr_flag | u(1) |

# Proposal

To solve the issue, it is proposed that when the reference picture lists signaled in PH indicate that the current picture does not use two reference picture lists, the flags mvd_l1_zero_flag, ph_temporal_mvp_enabled_flag , ph_disable_bdof_flag and ph_disable_dmvr_flag need not to be signaled. The proposed changes to VVC are highlighted below.

| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| --- | --- |
| If ( num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| If (rpl_info_in_ph_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) |  |
| mvd_l1_zero_flag | u(1) |
| If (sps_bdof_pic_present_flag && rpl_info_in_ph_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) |  |
| ph_disable_bdof_flag | u(1) |
| If (sps_dmvr_pic_present_flag && rpl_info_in_ph_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) |  |
| ph_disable_dmvr_flag | u(1) |

# Reference

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, the 17th JVET meeting, Brussels, BE, 7–17 January 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).