| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0324 |
| --- | --- |

| Title: | AHG9: On syntax signaling conditions in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Tsung-Chuan Ma, Hong-Jheng Jhu, Wei Chen, Xianglin Wang | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In this proposal, it is proposed to signal mvd_l1_zero_flag, ph_collocated_from_l0_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag and num_l1_weights depending on the whether the coded slices associated with the PH are bi-predictive, to prevent redundant signaling due to non-applicable syntax values in the picture header.

# Current draft text

In current VVC, mvd_l1_zero_flag is signaled in the picture header (PH) without any conditional constraint. However, the feature controlled by the flag mvd_l1_zero_flag is only applicable when the slice is a bi-predictive slice (B-slice). Therefore, the flag signaling is redundant when the slice associated with the picture header is not a B-slice.

Similarly, ph_collocated_from_l0_flag, ph_disable_bdof_flag,ph_disable_dmvr_flag and num_l1_weights are signaled in the PH only when the corresponding enabling flags are true, respectively. As shown in the tables below, however, the features controlled by the flags ph_collocated_from_l0_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag and num_l1_weights are only applicable when the slice is a bi-predictive slice (B-slice). Therefore, the signaling of these two flags is also redundant or useless when the slices associated with the picture header is not a B-slice.

| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| --- | --- |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| … |  |
| mvd_l1_zero_flag | u(1) |
| … |  |
| if( sps_bdof_pic_present_flag ) |  |
| ph_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| ph_disable_dmvr_flag | u(1) |
| … |  |

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( wp_info_in_ph_flag ) |  |
| num_l0_weights | ue(v) |
| … |  |
| if( pps_weighted_bipred_flag && wp_info_in_ph_flag |  |
| num_l1_weights | ue(v) |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| … |  |
| } |  |

# Proposal

To solve the issue, it is proposed that when the reference picture lists signaled in PH indicate that the current picture does not use two reference picture lists, the flags mvd_l1_zero_flag, ph_collocated_from_l0_flag, ph_disable_bdof_flag, ph_disable_dmvr_flag and num_l1_weights need not to be signaled. The proposed changes to VVC are highlighted below.

| if( ph_temporal_mvp_enabled_flag && rpl_info_in_ph_flag ) { |  |
| --- | --- |
| if ( num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| … |  |
| If ( (!rpl_info_in_ph_flag ¦¦ (rpl_info_in_ph_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) ) |  |
| mvd_l1_zero_flag | u(1) |
| … |  |
| If (sps_bdof_pic_present_flag && (!rpl_info_in_ph_flag ¦¦ (rpl_info_in_ph_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1)) ) |  |
| ph_disable_bdof_flag | u(1) |
| If (sps_dmvr_pic_present_flag && (!rpl_info_in_ph_flag ¦¦ (rpl_info_in_ph_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1)) ) |  |
| ph_disable_dmvr_flag | u(1) |
| … |  |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && wp_info_in_ph_flag) |  |
| pred_weight_table( ) |  |

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| if( wp_info_in_ph_flag ) |  |
| num_l0_weights | ue(v) |
| … |  |
| if( pps_weighted_bipred_flag && wp_info_in_ph_flag && (!rpl_info_in_ph_flag ¦¦ (rpl_info_in_ph_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1&& num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1))) |  |
| num_l1_weights | ue(v) |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumWeightsL1; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| … |  |
| } |  |

The semantics are also modified to handle the case when they is not signaled.

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

When ph_collocated_from_l0_flag is not present, the following applies:

- If num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] is larger than 1, the value of ph_collocated_from_l0_flag is inferred to be 1.

- Otherwise (num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] is larger than 1 ), the value of ph_collocated_from_l0_flag is inferred to be 0.

mvd_l1_zero_flag equal to 1 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is not parsed and MvdL1[ x0 ][ y0 ][ compIdx ] and MvdCpL1[ x0 ][ y0 ][ cpIdx ][ compIdx ] are set equal to 0 for compIdx = 0..1 and cpIdx = 0..2. mvd_l1_zero_flag equal to 0 indicates that the mvd_coding( x0, y0, 1 ) syntax structure is parsed. When not present, the value of mvd_l1_zero_flag is inferred to be 0.

ph_disable_bdof_flag equal to 1 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled in the slices associated with the PH. ph_disable_bdof_flag equal to 0 specifies that bi-directional optical flow inter prediction based inter bi-prediction may or may not be enabled in the slices associated with the PH.

When ph_disable_bdof_flag is not present, the following applies:

- If sps_bdof_enabled_flag is equal to 1 and sps_bdof_pic_present_flag is equal to 0, the value of ph_disable_bdof_flag is inferred to be equal to 0.

- Else if sps_dmvr_enabled_flag is equal to 1 and sps_bdof_pic_present_flag is equal to 1, the value of ph_disable_dmvr_flag is inferred to be equal to 1.

- Otherwise (sps_bdof_enabled_flag is equal to 0), the value of ph_disable_bdof_flag is inferred to be equal to 1.

ph_disable_dmvr_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is disabled in the slices associated with the PH. ph_disable_dmvr_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction may or may not be enabled in the slices associated with the PH.

When ph_disable_dmvr_flag is not present, the following applies:

- If sps_dmvr_enabled_flag is equal to 1 and sps_dmvr_pic_present_flag is equal to 0, the value of ph_disable_dmvr_flag is inferred to be equal to 0.

- Else if sps_dmvr_enabled_flag is equal to 1 and sps_dmvr_pic_present_flag is equal to 1, the value of ph_disable_dmvr_flag is inferred to be equal to 1.

- Otherwise (sps_dmvr_enabled_flag is equal to 0), the value of ph_disable_dmvr_flag is inferred to be equal to 1.

num_l1_weights specifies the number of weights signalled for entries in reference picture list 1 when pps_weighted_bipred_flag and wp_info_in_ph_flag are both equal to 1. The value of num_l1_weights shall be in the range of 0 to Min( 15, num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] ), inclusive.

The variable NumWeightsL1 is derived as follows:

if( !pps_weighted_bipred_flag)
	NumWeightsL1 = 0

else if (wp_info_in_ph_flag  &&  rpl_info_in_ph_flag && (num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] ==0 || num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] >==0))

NumWeightsL1 = 0
else if( wp_info_in_ph_flag  )		(148)
	NumWeightsL1 = num_l1_weights
else
	NumWeightsL1 = NumRefIdxActive[ 1 ]

# Reference

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, the 17th JVET meeting, Brussels, BE, 7–17 January 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).