| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0325 |
| --- | --- |

| Title: | AHG14: Disabling dependent quantization and sign bit hiding for transform skip mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Xianglin Wang | Email: | tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

In VVC draft 8, slice_ts_residual_coding_disabled_flag is used to disable transform-skip residual coding and apply regular residual coding for transform-skip TUs. It is asserted that for the current transform skip mode is problematic because dependent quantization (DQ) and sign bit hiding (SBH) can be enabled for transform-skip TU when regular residual coding is applied to transform skip mode in the current VVC specification. In this proposal, two aspects are proposed to solve the problem.

In the first aspect, two methods are proposed to disabled the DQ for transform skip mode. In the first method, one high-level disabling scheme is proposed by conditioning slice_ts_residual_coding_disabled_flag on the presence of ph_dep_quant_enabled_flag so that those two syntaxes cannot be enabled at the same time. In the second method, one low-level disabling scheme is proposed to disable the dependent quantization related operations (including DQ state CABAC context update and multiplying decoded levels by 2) at parsing stage if regular residual coding is applied to transform-skip TUs.

In the second aspect, it is proposed to always disable the SBH when the regular residual coding is applied to transform skip TUs.

# Problem statement

In VVC draft 8 [1], slice_ts_residual_coding_disabled_flag [2] and ph_dep_quant_enabled_flag are signaled independently. When both two flags are enabled, dependent quantization is applied to transform-skip mode at the parsing stage by 1) using dependent quantization state for CABAC context determination and 2) multiplying the decoded levels by 2 for de-quantization. However, at the de-quantization stage, QP 1 increment that is used for dependent quantization is not applied to transform-skip mode. Therefore, the current dependent quantization is not working appropriately for transform-skip mode. It is noticed that one ticket #859 was submitted to the bug tracker for reporting the same issue. On the other hand, according to VVC draft 8, SBH can be applied to transform-skip TUs when regular residual coding is applied to transform skip mode, i.e., slice_ts_residual_coding_disabled_flag is equal to one.

# Proposal

To achieve a cleaner design of transform skip mode, the following two aspects are proposed to prevent the combination of dependent quantization, SDH with the transform-skip mode.

## Disabling dependent quantization for transform skip mode

In the first aspect, the contribution proposes to disable the DQ for the transform skip TUs. Specifically, the following two methods are proposed:

- In the first method, it is proposed to use ph_dep_quant_enabled_flag to condition the presence of slice_ts_residual_coding_disabled_flag. Specifically, when the flag ph_dep_quant_enabled_flag is equal to one (i.e., the dependent quantization is enabled at current picture), slice_ts_residual_coding_disabled_flag is not signalled but inferred to be 0. When the flag ph_dep_quant_enabled_flag is equal to zero, the slice_ts_residual_coding_disabled_flag is still signalled.

- In the second method, it is proposed to apply transform_skip_flag[ x0 ][ y0 ][ cIdx ] to bypass the operations of dependent quantization, including dependent quantization state for CABAC context determination and multiplying the decoded levels by 2, at the stage of regular residual coding.

## Disabling sign bit hiding for transform skip mode

In the second aspect, it is proposed to disable SBH when regular residual coding is applied to transform skip TUs.

Please note that the methods as proposed in Sec. 2.1 and 2.2 can be applied independently.

# Specification text changes

## Conditioned slice_ts_residual_coding_disabled_flag on ph_dep_quant_enabled_flag

7.3.7.1 General slice header syntax

| if(!ph_dep_quant_enabled_flag) |  |
| --- | --- |
| slice_ts_residual_coding_disabled_flag | u(1) |

## Apply transform_skip_flag to bypass dependent quantization

#### 7.3.10.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth = = 5 && log2TbHeight < 6 ) |  |  |
| log2ZoTbWidth = 4 |  |  |
| Else |  |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |  |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth < 6 && log2TbHeight = = 5 ) |  |  |
| log2ZoTbHeight = 4 |  |  |
| Else |  |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |  |
| if( log2TbWidth > 0 ) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| log2TbWidth = log2ZoTbWidth |  |
| log2TbHeight = log2ZoTbHeight |  |
| remBinsPass1 = ( ( 1 << ( log2TbWidth + log2TbHeight ) ) * 7 ) >> 2 |  |
| log2SbW = ( Min( log2TbWidth, log2TbHeight ) < 2 ? 1 : 2 ) |  |
| log2SbH = log2SbW |  |
| if( log2TbWidth + log2TbHeight > 3 ) |  |
| if( log2TbWidth < 2 ) { |  |
| log2SbW = log2TbWidth |  |
| log2SbH = 4 − log2SbW |  |
| } else if( log2TbHeight < 2 ) { |  |
| log2SbH = log2TbHeight |  |
| log2SbW = 4 − log2SbH |  |
| } |  |
| numSbCoeff = 1 << ( log2SbW + log2SbH ) |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock = ( 1 << ( log2TbWidth + log2TbHeight − ( log2SbW + log2SbH ) ) ) − 1 |  |
| do { |  |
| if( lastScanPos = = 0 ) { |  |
| lastScanPos = numSbCoeff |  |
| lastSubBlock− − |  |
| } |  |
| lastScanPos− − |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ lastSubBlock ][ 1 ] |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ lastScanPos ][ 1 ] |  |
| } while( ( xC != LastSignificantCoeffX ) ¦ ¦ ( yC != LastSignificantCoeffY ) ) |  |
| if( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) |  |
| LfnstDcOnly = 0 |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |
| if( ( lastSubBlock > 0 ¦ ¦ lastScanPos > 0 ) && cIdx = = 0 ) |  |
| MtsDcOnly = 0 |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( i < lastSubBlock && i > 0 ) { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( xS > 3 ¦ ¦ yS > 3 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = firstPosMode0 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| if( sig_coeff_flag[ xC ][ yC ] ) { |  |  |
| abs_level_gtx_flag[ n ][ 0 ] | ae(v) |
| remBinsPass1− − |  |
| if( abs_level_gtx_flag[ n ][ 0 ] ) { |  |
| par_level_flag[ n ] | ae(v) |
| remBinsPass1− − |  |
| abs_level_gtx_flag[ n ][ 1 ] | ae(v) |
| remBinsPass1− − |  |
| } |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| AbsLevelPass1[ xC ][ yC ] = sig_coeff_flag[ xC ][ yC ] + par_level_flag[ n ] + abs_level_gtx_flag[ n ][ 0 ] + 2 * abs_level_gtx_flag[ n ][ 1 ] |  |
| if( ph_dep_quant_enabled_flag && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ] |  |
| firstPosMode1 = n – 1 |  |
| } |  |
| for( n = firstPosMode0; n > firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) |  |
| abs_remainder[ n ] | ae(v) |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| } |  |
| for( n = firstPosMode1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] ) |  |
| dec_abs_level[ n ] | ae(v) |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| if( lastSigScanPosSb = = −1 ) |  |
| lastSigScanPosSb = n |  |
| firstSigScanPosSb = n |  |
| } |  |
| if( ph_dep_quant_enabled_flag && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| if( ph_dep_quant_enabled_flag ¦ ¦ !pic_sign_data_hiding_enabled_flag ) |  |
| signHidden = 0 |  |
| Else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( AbsLevel[ xC ][ yC ] > 0 ) && ( !signHidden ¦ ¦ ( n != firstSigScanPosSb ) ) ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( ph_dep_quant_enabled_flag && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] ) { |  |
| QState = startQStateSb |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 2 * AbsLevel[ xC ][ yC ] − ( QState > 1 ? 1 : 0 ) ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ] |  |
| } |  |
| } else { |  |
| sumAbsLevel = 0 |  |
| for( n = numSbCoeff − 1; n >= 0; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( AbsLevel[ xC ][ yC ] > 0 ) { |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = AbsLevel[ xC ][ yC ] * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| if( signHidden ) { |  |
| sumAbsLevel += AbsLevel[ xC ][ yC ] |  |
| if( ( n = = firstSigScanPosSb ) && ( sumAbsLevel % 2 ) = = 1 ) ) |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = −TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |  |

## Disable sign data hiding for transform skip mode

#### 7.3.10.11 Residual coding syntax

| if(transform_skip_flag[ x0 ][ y0 ][ cIdx ] ¦ ¦ ph_dep_quant_enabled_flag ¦ ¦ !pic_sign_data_hiding_enabled_flag ) |  |
| --- | --- |
| signHidden = 0 |  |
| Else |  |
| signHidden = ( lastSigScanPosSb − firstSigScanPosSb > 3 ? 1 : 0 ) |  |

# Conclusion

In this contribution, methods are proposed to prevent the combination of the dependent quantization, the sign data hiding with transform skip mode when regular residual coding is applied for transform-skip TUs. In the first aspect, two methods are proposed to disable the dependent quantization for the transform skip mode. In the first method, the signaling of slice_ts_residual_coding_disabled_flag is conditioned based on the presence of ph_dep_quant_enabled_flag. In the second method, it is proposed to use transform_skip_flag to bypass the dependent quantization related operation when regular residual coding is applied to transform skip mode. In the second aspect, it is proposed to disable the sign bit hiding when regular residual coding is applied to transform skip mode. It is suggested to adopt the proposed methods into VVC.

# Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020, Brussels, Belgium.

A. Nalci, H. Wang, M. Coban, M. Karczewicz, T. Tsukuba, M. Ikeda, Y. Yagasaki, T. Suzuki, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, T.-C. Ma, H.-J. Jhu, X. Xiu, Y.-W. Chen, X. Wang, “CE3-2.4: Luma and Chroma BDPCM for Lossless Coding with Regular Residual Coding”, JVET-Q0089, Brussels, BE, Jan 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).