| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0327-v1 |
| --- | --- |

| Title: | AHG 10: One-pass CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, SZ DJI Technology Co., Ltd.2 | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

In this contribution, a one-pass encoding algorithm is proposed for CCALF. The current CCALF encoding algorithm requires a lot of encoding passes (picture buffer accesses) which will increase external memory access, encoding latency, and power consumption significantly. Hence, we propose a method to estimate CCALF filtering distortion without conducting real filter operation. The number of encoding passes can be reduced from 152 to 1. The coding performance in VTM-8.0 is as follows,

AI:   0.00%	0.00%	-0.01%

RA:  0.00%	-0.10%	-0.05%

LDB: -0.02%	-0.06%	-0.07%

Introduction

Cross Component ALF (CCALF) [1], aimed at improving chroma fidelity by exploiting correlations between the luma and chroma channels, was adopted since JVET 17th meeting. CCALF is an essential coding tool in the current VVC-8.0 [2]. Specifically, CCALF applies a linear filter to the luma channel to determine a chroma residual that is added to a previously reconstructed chroma signal. As shown in Fig. 1, 4x3 filter shape is used for CCALF luma component, where C0~C7 represents the filter coefficients. The relative location of chroma being filtered and reference luma samples is shown in Fig. 2.

Fig. 1. Filter shape of CCALF

Fig. 2. Relative location of chroma sample being filtered and

its reference luma samples for 4:2:0 format

In the encoder, the filter coefficients of CCALF are designed by solving Wiener-Hopf equations, and the selection of best filter coefficients and the decision of CTU level filter index and on/off control are based on an RDO method. The CCALF encoding process is shown in Fig. 3. Firstly, CCALF derives available CCALF APSs and performs a real filtering process to calculate distortion. As shown in Fig. 3 (a), during this process, CCALF filtering is conducted 32 times at most (8 available APSs and 4 filters for each APS). Secondly, CCALF calculates new filters based on the covariance of the current picture, as shown in Fig. 3 (b). During the covariance calculation process, original chroma, reconstructed luma before ALF and reconstructed chroma after ALF are accessed. With the calculated CCALF coefficients, filtering is conducted. The parameter training process will be ended when there is no improvement compared to the last iteration or iter is larger than maxTrainingIterCount. During this process, CCALF filtering and covariance calculation are conducted 60 times at most (L = 4, maxTrainingIterCount = 15). So, the total picture buffer access number is 120 times (60 + 60).

The number of picture buffer access is regarded as the number of encoding pass, which means that one encoding pass requires one time of picture buffer access. The multi-pass filter selection process is harmful for hardware designs. In practical, the on-chip memory is very limited, so picture buffers are usually off-chip and allocated in external memory. Multiple times of picture buffer accesses indicates massive external memory bandwidth, and one off-chip data access may require 10 times of power consumption and latency in comparison with one on-chip data access. Therefore, the multi-pass CCALF encoding algorithm should be avoided.

- APS reference                            (b) New filter

Fig. 3. CCALF encoding flow in VTM-8.0

Proposed Method

In this contribution, one-pass CCALF is proposed. Details are given below.

Aspect 1: Optimization on frame level covariance calculation

As shown in Fig. 4 (b), CTU-level covariance is calculated before the parameter training process. In the training loop, frame-level covariance is updated according to the CTU level filter index and on/off control. With this optimization, there is no need to access the picture buffer during the training process. And this aspect does not affect the coding performance as it is purely software optimization.

- APS reference                            (b) New filter

Fig. 4. Proposed one-pass CCALF encoding flow

Aspect 2: Distortion estimation

The mean-square error of CCALF can be calculated as follows,

ε=Eyk-ok=Ei=0N-1wi×lk-i-l(k)+ck-o(k)2

where ok, lk, ck and y(k) represent the original signal of chroma (Cb / Cr), luma signal before ALF, chroma signal (Cb / Cr) after ALF and filtered chroma signal (Cb / Cr), respectively. N (N = 8) is the number of CCALF filter Tap. The filter coefficients, f=(w0,  w1, …,wN-1)T, is derived according to filter-on CTUs. The distortion can be estimated with the following equtation,

ε= <f, Akf-2Xk>+ k∈Ks(k)2

Ak= k=0K-1Rllk

Rk= k=0K-1Rcok

where <∙, ∙> represents inner product. K is the filter-on region. Rll(k) is the auto-correlation matrix of l(k). Rco(k) is the cross-correction vector of c(k) and o(k).

Simulation Results

The experiments were performed based on VTM-8.0 with VTM configuration files under JVET common test conditions[3]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

Table 1 Simulation results of the proposed one-pass CCALF

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 98% | 100% |
| Class A2 | 0.00% | 0.01% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | -0.01% | 97% | 100% |
| Class E | 0.00% | 0.00% | -0.05% | 65% | 100% |
| Overall | 0.00% | 0.00% | -0.01% | 92% | 100% |
| Class D | 0.00% | -0.01% | 0.02% | 58% | 101% |
| Class F | 0.00% | -0.02% | -0.01% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.19% | -0.10% | 100% | 100% |
| Class A2 | 0.00% | -0.10% | 0.11% | 142% | 100% |
| Class B | 0.01% | -0.14% | -0.08% | 96% | 100% |
| Class C | -0.02% | 0.02% | -0.09% | 113% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.05% | 109% | 100% |
| Class D | 0.02% | -0.02% | -0.01% | 101% | 100% |
| Class F | 0.03% | -0.01% | 0.03% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.18% | 0.07% | 98% | 100% |
| Class C | -0.08% | -0.27% | 0.04% | 99% | 100% |
| Class E | 0.06% | -0.18% | -0.44% | 99% | 100% |
| Overall | -0.02% | -0.06% | -0.07% | 98% | 100% |
| Class D | -0.04% | -0.22% | -0.05% | 101% | 100% |
| Class F | -0.02% | 0.50% | -0.10% | 105% | 100% |

Conclusion

In this contribution, a one-pass encoding algorithm is proposed for CCALF. With this optimization, the number of encoding passes can be reduced from 152 to 1. And there is small coding gain on average. We recommend to adopt this proposal to VTM software.

References

- K. Misra, F. Bossen, A. Segall, et.al, “CC-ALF: Integrated Text for the Cross Component Adaptive Loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q0795, Jan. 2020

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2001, Jan. 2020

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).