| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0328-v1 |
| --- | --- |

| Title: | AHG 10: ALF and CCALF encoder parallel design |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng1, Xiaozhen Zheng2, Shanshe Wang1, Siwei Ma1 Peking University1, SZ DJI Technology Co., Ltd.2 | Tel: Email: | xwmeng@pku.edu.cn xiaozhen.zheng@dji.com |
| Source: | Peking University, SZ DJI Technology Co., Ltd. |  |  |

_____________________________

- Abstract

In this contribution, ALF and CCALF encoder parallel design is proposed. Chroma signal before ALF is used to replace the current reconstructed chroma signal after ALF in CCALF parameter training process. With the proposed method, CCALF parameter training and ALF process can be parallel. The coding performance in VTM-8.0 is as follows,

AI:   0.00%	0.16%	0.19%

RA:  0.00%	0.23%	0.20%

LDB: -0.04%	0.13%	0.04%

Introduction

Cross Component ALF (CCALF) [1], aimed at improving chroma fidelity by exploiting correlations between the luma and chroma channels, was adopted since JVET 17th meeting. CCALF is an essential coding tool in the current VVC-8.0 [2]. Specifically, CCALF applies a linear filter to the luma channel to determine a chroma residual that is added to a previously reconstructed chroma signal. As shown in Fig. 1, 4x3 filter shape is used for CCALF luma component, where C0~C7 represents the filter coefficients. The relative location of chroma being filtered and reference luma samples is shown in Fig. 2.

Fig. 1. Filter shape of CCALF

Fig. 2. Relative location of chroma sample being filtered and

its reference luma samples for 4:2:0 format

In encoder, reconstructed chroma signal after ALF is needed in CCALF filter coefficients training process. Hence, parameter training of ALF and CCALF can not be parallel in encoder.

- APS reference                            (b) New filter

Fig. 3. CCALF encoding flow in VTM-8.0

Proposed Method

In this contribution, ALF and CCALF encoder parallel design is proposed. Chroma signal before ALF is used to replace the current reconstructed chroma signal in CCALF parameter training process. With the proposed method, CCALF parameter training and ALF process can be parallel.

Simulation Results

The experiments were performed based on VTM-8.0 with VTM configuration files under JVET common test conditions[3]. And, the experiments were conducted under Ubuntu Linux. The encoding time is unreliable.

Table 1 Simulation results of the proposed method

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.39% | 0.47% | 98% | 100% |
| Class A2 | 0.00% | 0.13% | 0.14% | 118% | 100% |
| Class B | 0.00% | 0.13% | 0.15% | 103% | 100% |
| Class C | 0.00% | 0.10% | 0.09% | 96% | 100% |
| Class E | 0.00% | 0.09% | 0.14% | 65% | 100% |
| Overall | 0.00% | 0.16% | 0.19% | 95% | 100% |
| Class D | 0.00% | 0.04% | 0.04% | 61% | 100% |
| Class F | 0.00% | 0.26% | 0.21% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.36% | 0.50% | 104% | 100% |
| Class A2 | 0.00% | 0.46% | 0.24% | 100% | 100% |
| Class B | 0.02% | 0.10% | 0.04% | 108% | 100% |
| Class C | -0.02% | 0.11% | 0.15% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.23% | 0.20% | 104% | 100% |
| Class D | 0.00% | -0.04% | -0.03% | 101% | 100% |
| Class F | 0.02% | 0.21% | 0.14% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.30% | 0.01% | 98% | 100% |
| Class C | -0.08% | -0.21% | 0.04% | 99% | 100% |
| Class E | 0.00% | 0.30% | 0.10% | 97% | 100% |
| Overall | -0.04% | 0.13% | 0.04% | 99% | 100% |
| Class D | -0.04% | -0.25% | -0.16% | 101% | 100% |
| Class F | -0.06% | 0.85% | -0.21% | 97% | 100% |

Conclusion

In this contribution, a CCALF/ALF encoder parallel design is proposed. We recommend to adopt this proposal to VTM software.

References

- K. Misra, F. Bossen, A. Segall, et.al, “CC-ALF: Integrated Text for the Cross Component Adaptive Loop Filter,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q0795, Jan. 2020

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-Q2001, Jan. 2020

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Mar. 2019

Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).