| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0329 |
| --- | --- |

| Title: | Mismatch on clipping input residuals to IACT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu1, Yi-Wen Chen1, Tsung-Chuan Ma1, Hong-Jheng Jhu1, Xianglin Wang1 Jie Zhao2 Seung Hwan Kim2 | Email: | xiaoyuxiu@kwai.com jie.zhao@lge.com |
| Source: | Kwai Inc. 1 & LG Electronics2 |  |  |

_____________________________

- Abstract

This contribution reports one mismatch between VVC draft 8 and test model software VTM-8.0 on clipping the input residuals to inverse adaptive color transform (IACT). In the VVC draft 8, the input residuals to the IACT are clipped to the range [-2BitDepth, 2BitDepth-1] while the clipping range in the VTM-8.0 is [-2BitDepth+1, 2BitDepth+1-1]. Given that the ACT transform increases the bit-depth of residuals by 1-bit, the full dynamic range of input residuals to the IACT are [-2BitDepth+1, 2BitDepth+1-1]. Therefore, it is proposed to change the specification text to align with the VTM-8.0 software. The proposed change has no impact on coding performance.

Problem statement and proposal

At the 18-th JVET meeting, reversible YCgCo transform, namely YCgCo-R, was adopted in VVC draft 8 [1] to support both lossless and lossy coding for adaptive color transform (ACT). When the YCgCo-R transform is applied, the bit-depth of the output residuals from the forward ACT transform is increased by 1-bit. Additionally, in the current ACT design, one clipping operation is applied to the input residuals to the inverse adaptive color transform (IACT) to reduce the memory requirement and operational bit-depth of the IACT. In this contribution, one mismatch was identified between the VVC draft 8 [1] and VTM-8.0 [2] software on the clipping range of the input residuals to the IACT. Specifically, in the VVC draft 8, the residuals are clipped to the range [-2BitDepth, 2BitDepth-1] before the IACT is applied. However, in the VTM-8.0, such clipping range is [-2BitDepth+1, 2BitDepth+1-1].

Because the YCgCo-R transform increases the residual bit-depth by 1-bit, the full dynamic range of the input residuals to the IACT is [-2BitDepth+1, 2BitDepth+1-1]. Therefore, in this contribution, the following changes are proposed to the current specification text to align with the VTM-8.0 software.

8.7.4.6 Residual modification process for blocks using colour space conversion

……

The (nTbW)x(nTbH) arrays of residual samples rY, rCb and rCr are modified as follows:

rY[ x ][ y ] = Clip3( −( 1 << (BitDepth + 1) ), ( 1 << (BitDepth + 1) ) − 1, rY[ x ][ y ] )	(1220)

rCb[ x ][ y ] = Clip3( −( 1 << (BitDepth + 1) ), ( 1 << (BitDepth + 1) ) − 1, rCb[ x ][ y ] )	(1221)

rCr[ x ][ y ] = Clip3( −( 1 << (BitDepth + 1) ), ( 1 << (BitDepth + 1) ) − 1, rCr[ x ][ y ] )	(1222)

tmp = rY[ x ][ y ] − ( rCb[ x ][ y ] >> 1 )			(1223)

rY[ x ][ y ] = tmp + rCb[ x ][ y ]                             	(1224)

rCb[ x ][ y ] = tmp − ( rCr[ x ][ y ] >> 1 )                      	(1225)

rCr[ x ][ y ] += rCrb[ x ][ y ]                                                     	(1226)

It is noted that the proposed change is necessary to support the lossless coding for the ACT mode. One ticket #993 was submitted in bug tracker for reporting the same mismatch.

Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020, Brussels, Belgium.

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).