| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0330 |
| --- | --- |

| Title: | AHG16: On clipping average luma value for chroma residual scaling factor derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In VVC draft 8, the average of neighboring reconstructed luma samples above and left to one 64×64 region is used to calculate the chroma residual scaling factor for the coding units (CUs) inside the region. Additionally, one clipping operation is applied to clip the luma average to the full range of the internal bit-depth, which is asserted to be unnecessary. It is proposed to remove such unnecessary clipping operation from the derivation of chroma residual scaling factor. Simulation results reportedly show that the proposed modification provides bit-exact BD-rate performance.

Introduction

In VVC draft 8 [1], when chroma residual scaling is applied, the corresponding scaling factor used for the coding units (CUs) inside one region with size of min(CtbSizeY, 64)×min(CtbSizeY, 64) is derived based on the average of reconstructed luma samples above and left to the region. Additionally, one clipping is applied to clip the average luma value to the full range of the internal bit-depth, [0, (1<<BitDepth)-1], as specified in section 8.7.5.3 in the VVC draft 8 as:

- For the derivation of the variable varScale the following ordered steps apply:

- The variable invAvgLuma is derived as follows:

……

- The variable invAvgLuma is derived as follows:

- If cnt is greater than 0, the following applies:

invAvgLuma = Clip1( ( k=0cnt-1recLuma k  + ( cnt  >>  1 ) )  >>  Log2( cnt ) )	(1235)

- Otherwise (cnt is equal to 0), the following applies:

invAvgLuma = 1  <<  ( BitDepth − 1 )	(1236)

Proposal

At the reconstruction process in VVC draft 8[1], after adding the residual samples to the prediction samples of one CU, the reconstructed luma samples used for the chroma scaling factor derivation are already clipped to the internal bit-depth range. Thus, their average cannot go beyond such range.

It is thus proposed to remove unnecessary clipping operation in equation (1235) from the chroma residual scaling factor derivation process in section 8.7.5.3 as follows

- For the derivation of the variable varScale the following ordered steps apply:

- The variable invAvgLuma is derived as follows:

……

- The variable invAvgLuma is derived as follows:

- If cnt is greater than 0, the following applies:

invAvgLuma = Clip1( ( k=0cnt-1recLuma k  + ( cnt  >>  1 ) )  >>  Log2( cnt ) )	(1235)

- Otherwise (cnt is equal to 0), the following applies:

invAvgLuma = 1  <<  ( BitDepth − 1 )	(1236)

if (pelnum == numNeighbor)

{

#if REM_CS_CLIPPING

lumaValue = (recLuma + (1 << (numNeighborLog - 1))) >> numNeighborLog;

#else

lumaValue = ClipPel((recLuma + (1 << (numNeighborLog - 1))) >> numNeighborLog, tu.cs->slice->clpRng(COMPONENT_Y));

#endif

}

else if (pelnum == (numNeighbor << 1))

{

#if REM_CS_CLIPPING

lumaValue = (recLuma + (1 << numNeighborLog)) >> (numNeighborLog + 1);

#else

lumaValue = ClipPel((recLuma + (1 << numNeighborLog)) >> (numNeighborLog + 1), tu.cs->slice->clpRng(COMPONENT_Y));

#endif

}

else

{

CHECK(pelnum != 0, "");

#if REM_CS_CLIPPING

lumaValue = valueDC;

#else

lumaValue = ClipPel(valueDC, tu.cs->slice->clpRng(COMPONENT_Y));

#endif

}

In VTM-8.0, only three lines in the function calculateChromaAdjVpduNei() are changed to avoid unnecessary clipping operations, as shown below:

Simulations

The proposed change was tested on top of VTM-8.0 [2] by following common test conditions (CTCs) [3]. As shown in Table 1, the proposed change produces bit-exact BD-rate results as the anchors.

Table 1 The BD-rate impact of the proposed change under the CTC

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 105% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 94% | 95% |

Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020, Brussels, Belgium.

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).