| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0330_r1 |
| --- | --- |

| Title: | AHG16: On clipping average luma value for chroma residual scaling factor derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In VVC draft 8, the average of neighboring reconstructed luma samples above and left to one 64×64 region is used to calculate the chroma residual scaling factor for the coding units (CUs) inside the region. In the chroma sample reconstruction process 8.7.5.3 in VVC draft 8, one clipping operation is applied to clip the luma average to the full range of the internal bit-depth when deriving the chroma residual scaling factor. Additionally, the same clipping operation is also applied when generating chroma samples even if the chroma residual samples are zeros, i.e., chroma CBF is zero. It is asserted that those two clipping operations are redundant. For a cleaner design, this contribution proposes to remove those unnecessary clipping operations from the current VVC specification.. Simulation results reportedly show that the proposed modification provides bit-exact BD-rate performance.

Introduction

In VVC draft 8 [1], when chroma residual scaling is applied, the corresponding scaling factor used for the coding units (CUs) inside one region with size of min(CtbSizeY, 64)×min(CtbSizeY, 64) is derived based on the average of reconstructed luma samples above and left to the region. Additionally, one clipping is applied to clip the average luma value to the full range of the internal bit-depth, [0, (1<<BitDepth)-1]. Meanwhile, when generating reconstructed chroma samples, the same clipping operation is also applied to chroma prediction samples if all chroma residual samples are zeros, i.e., chroma CBF is equal to zero.

- For the derivation of the variable varScale the following ordered steps apply:

- The variable invAvgLuma is derived as follows:

……

- The variable invAvgLuma is derived as follows:

- If cnt is greater than 0, the following applies:

invAvgLuma = Clip1( ( k=0cnt-1recLuma k  + ( cnt  >>  1 ) )  >>  Log2( cnt ) )	(1235)

- Otherwise (cnt is equal to 0), the following applies:

invAvgLuma = 1  <<  ( BitDepth − 1 )	(1236)

……

- The reconstructed chroma picture sample array recSamples is derived as follows:

- If tuCbfChroma is equal to 1, the following applies:

resSamples[ i ][ j ] = Clip3( −( 1 << BitDepth ), ( 1 << BitDepth ) − 1, resSamples[ i ][ j ] )	(1238)

recSamples[ xCurr + i ][ yCurr + j ] = Clip1( predSamples[ i ][ j ] + 	(1239)
	Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 ) )

- Otherwise (tuCbfChroma is equal to 0), the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1(predSamples[ i ][ j ] )	(1240)

Proposal

At the reconstruction process in VVC draft 8[1], after adding the residual samples to the prediction samples of one CU, the reconstructed luma samples used for the chroma scaling factor derivation are already clipped to the internal bit-depth range. Thus, their average cannot go beyond such range. On the other hand, according to the current specification, the prediction samples generated from various coding modes are also guaranteed to be within the full range of internal bit-depth. Therefore, the two clipping operations in equations (1235) and (1240) are redundant. For a cleaner design, it is proposed to remove those unnecessary clipping operations from the current specification, as described below:

- For the derivation of the variable varScale the following ordered steps apply:

- The variable invAvgLuma is derived as follows:

……

- The variable invAvgLuma is derived as follows:

- If cnt is greater than 0, the following applies:

invAvgLuma = Clip1( ( k=0cnt-1recLuma k  + ( cnt  >>  1 ) )  >>  Log2( cnt ) )	(1235)

- Otherwise (cnt is equal to 0), the following applies:

invAvgLuma = 1  <<  ( BitDepth − 1 )	(1236)

……

- The reconstructed chroma picture sample array recSamples is derived as follows:

- If tuCbfChroma is equal to 1, the following applies:

resSamples[ i ][ j ] = Clip3( −( 1 << BitDepth ), ( 1 << BitDepth ) − 1, resSamples[ i ][ j ] )	(1238)

recSamples[ xCurr + i ][ yCurr + j ] = Clip1( predSamples[ i ][ j ] + 	(1239)
	Sign( resSamples[ i ][ j ] ) * ( ( Abs( resSamples[ i ][ j ] ) * varScale + ( 1 << 10 ) ) >> 11 ) )

- Otherwise (tuCbfChroma is equal to 0), the following applies:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1(predSamples[ i ][ j ] )	(1240)

if (pelnum == numNeighbor)

{

#if REM_CS_CLIPPING

lumaValue = (recLuma + (1 << (numNeighborLog - 1))) >> numNeighborLog;

#else

lumaValue = ClipPel((recLuma + (1 << (numNeighborLog - 1))) >> numNeighborLog, tu.cs->slice->clpRng(COMPONENT_Y));

#endif

}

else if (pelnum == (numNeighbor << 1))

{

#if REM_CS_CLIPPING

lumaValue = (recLuma + (1 << numNeighborLog)) >> (numNeighborLog + 1);

#else

lumaValue = ClipPel((recLuma + (1 << numNeighborLog)) >> (numNeighborLog + 1), tu.cs->slice->clpRng(COMPONENT_Y));

#endif

}

else

{

CHECK(pelnum != 0, "");

#if REM_CS_CLIPPING

lumaValue = valueDC;

#else

lumaValue = ClipPel(valueDC, tu.cs->slice->clpRng(COMPONENT_Y));

#endif

}

In VTM-8.0, only three lines in the function calculateChromaAdjVpduNei() are changed to avoid unnecessary clipping operations, as shown below:

Simulations

The proposed change was tested on top of VTM-8.0 [2] by following common test conditions (CTCs) [3]. As shown in Table 1, the proposed change produces bit-exact BD-rate results as the anchors.

Table 1 The BD-rate impact of the proposed change under the CTC

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 105% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 101% |

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 105% |
| Overall | 0.00% | 0.00% | 0.00% | 98% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 94% | 95% |

Reference

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020, Brussels, Belgium.

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).