| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0331 |
| --- | --- |

| Title: | Cleanup of LFNST signalling in single tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In VVC draft 8, low-frequency non-separable transform (LFNST) is always disabled for chroma components when single-tree partition is applied, and applied to both luam and chroma components in dual-tree parition. However, it was found that the current LFNST signalling of luma component in single-tree is still conditioned on 1) chroma transform skip flags and 2) the positions of last non-zero coefficients of chroma components. In this contribution, two aspects are proposed to simplify the existing LFNST signaling design in VVC draft 8. In the first aspect, it is proposed to remove the signalling dependency of the luma LFNST on the chroma components in single-tree partition for a cleaner design. In the second aspect, it is proposed to disable the LFNST for chroma components in dual-tree partition. Simulation results show that the first aspect leads to negligible impact on coding efficiency.

Problem statement

At the 17th JVET meeting, one simplified low-frequency non-separable transform (LFNST) signalling [1] was adopted into VVC draft 8 [2]. With such adoption, the LFNST is always disabled when transform skip (TS) is applied to any of coded components one coding unit (CU). Specifically, for luma separate-tree, the LFNST is always disabled when the TS is used to the luma component; for chroma separate-tree, the LFNST is always disabled when the TS is applied to either Cb or Cr component. On the other hand, for single-tree, the LFNST is enabled for luma component but always disabled for chroma components. According to the current specification text, the LFNST is signaled when the following conditions are satisfied:

| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| --- | --- |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

As shown above, though the LFNST is only applied to luma components in single-tree, the corresponding LFNST signalling is still dependent on chroma components due to the following reasons:

- The presence of the LFNST signalling is conditioned on the transform skip flags of both luma and chroma components. The LFNST is disallowed when either luma or chroma applies transform skip.

- The variables LfnstDcOnly and LfnstZeroOutSigCoeffFlag (which indicate whether the LFNST DC only constraint and the LFNST zero-out constraint are satisfied or not, respectively) are determined based on the positions of last non-zero coefficients of both luma and chroma components.

Proposal

In this contribution, two aspects are proposed to simplify the existing LFNST signaling design in VVC draft 8.

In the first aspect, to achieve a cleaner design, the following two modifications are proposed to remove the dependency of the LFNST signalling upon chroma components in single-tree case:

- The LFNST signalling is only dependent on luma transform skip mode in single tree.

- Only the transform coefficients of luma component are considered when the DC only constraint and the zero-out region constraint are applied to the LFNST signalling in single-tree.

In the second aspect, it is proposed to also disabled LFNST for chroma components in dual-tree partition such that the LFNST is only applied to luma components in both dual-tree and single-tree partition cases.

Specification text changes

Specification text changes when enabling only the first aspect

- 7.3.10.5 Coding unit syntax

| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| --- | --- |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ treeType = = SINGLE_TREE ¦¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

- 7.3.10.11 Residual coding syntax

| if( ( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) && (treeType == DUAL_TREE_CHROMA ¦¦ cIdx == 0) ) |  |
| --- | --- |
| LfnstDcOnly = 0 |  |
| if( ( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) && (treeType == DUAL_TREE_CHROMA ¦¦ cIdx == 0) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |

Specification text changes when enabling the first and second aspects

- 7.3.10.5 Coding unit syntax

| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| --- | --- |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| if( (treeType != DUAL_TREE_CHROMA) && Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1(transform_skip_flag[ x0 ][ y0 ][ 0 ] == 0) && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

- 7.3.10.11 Residual coding syntax

| if( ( lastSubBlock = = 0 && log2TbWidth >= 2 && log2TbHeight >= 2 && !transform_skip_flag[ x0 ][ y0 ][ cIdx ] && lastScanPos > 0 ) && (cIdx == 0) ) |  |
| --- | --- |
| LfnstDcOnly = 0 |  |
| if( ( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) && (cIdx == 0) ) |  |
| LfnstZeroOutSigCoeffFlag = 0 |  |

- 7.4.11.4 Coding tree semantics

The variable ApplyLfnstFlag is derived as follows:

- If treeType is equal to SINGLE_TREE, the following applies:

ApplyLfnstFlag = ( lfnst_idx > 0 && cIdx  = =  0 ) ? 1 : 0	(176)

- Otherwise, the following applies:

ApplyLfnstFlag = ( lfnst_idx > 0 ) ? 1 : 0		(177)

Simulations

The proposed change was tested on top of VTM-8.0 [3] by following common test conditions (CTCs) [4].

BD-rate performance when enabling the first aspect

Table 1 The BD-rate impact of the proposed change

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 96% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.07% | 0.03% | 98% | 99% |
| Class A2 | 0.01% | -0.03% | 0.10% | 97% | 100% |
| Class B | 0.01% | -0.02% | 0.04% | 97% | 99% |
| Class C | -0.02% | 0.09% | 0.02% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.04% | 97% | 99% |
| Class D | -0.01% | 0.04% | -0.01% | 99% | 98% |
| Class F | 0.01% | 0.03% | 0.00% | 99% | 99% |

Table 2 The BD-rate impact of the proposed change under the CTC

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 100% | 103% |
| Class A2 | 0.00% | -0.08% | -0.03% | 101% | 99% |
| Class B | 0.00% | 0.07% | 0.04% | 102% | 103% |
| Class C | 0.01% | 0.04% | -0.06% | 99% | 92% |
| Class E | 0.00% | -0.13% | -0.06% | 102% | 99% |
| Overall | 0.00% | -0.01% | -0.01% | 101% | 99% |
| Class D | 0.01% | 0.07% | 0.01% | 103% | 101% |
| Class F | 0.00% | 0.01% | 0.06% | 100% | 100% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.01% | 0.05% | 101% | 101% |
| Class A2 | 0.01% | 0.00% | 0.02% | 102% | 101% |
| Class B | 0.01% | 0.07% | 0.01% | 99% | 100% |
| Class C | 0.00% | 0.11% | -0.20% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | -0.04% | 100% | 100% |
| Class D | -0.01% | -0.11% | 0.06% | 99% | 100% |
| Class F | -0.02% | 0.10% | 0.02% | 99% | 99% |

BD-rate performance of the first and second aspects

Reference

H. E. Egilmez, et al, “Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST Signaling, Latency Reduction and Scaling Process”, JVET-Q0784, Jan. 2020, Brussels, Belgium.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020, Brussels, Belgium.

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).