| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0331_r1 |
| --- | --- |

| Title: | Cleanup of LFNST signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang | Email: | xiaoyuxiu@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

- Abstract

In VVC draft 8, low-frequency non-separable transform (LFNST) is always disabled for chroma components when single-tree partition is applied, and applied to both luam and chroma components in dual-tree parition. However, it was found that the current LFNST signalling of luma component in single-tree is still conditioned on 1) chroma transform skip flags and 2) the positions of last non-zero coefficients of chroma components. In this contribution, three methods are proposed to simplify the existing LFNST signaling design in VVC draft 8. In the first method, it is proposed to only check DC only constraint for luma component for LFNST signaling in single-tree partition. In the second method, it is proposed to remove the signalling dependency of the luma LFNST on the chroma components in single-tree partition. In the third method, it is proposed to disable the LFNST for chroma components in dual-tree partition. In the second and third method, the signaling of LFNST and MTS indices are moved from the end of the CU to be after the TB of the corresponding last components that the LFNST and MTS are applied to. Simulation results show that both the first and second methods lead to negligible impact on coding efficiency. The third method leads to {0.12%, 1.14%, 1.37%} and {0.04%, 0.22%, 0.3%} for AI and RA configurations, respectively.

Problem statement

At the 17th JVET meeting, one simplified low-frequency non-separable transform (LFNST) signalling [1] was adopted into VVC draft 8 [2]. With such adoption, the LFNST is always disabled when transform skip (TS) is applied to any of coded components one coding unit (CU). Specifically, for luma separate-tree, the LFNST is always disabled when the TS is used to the luma component; for chroma separate-tree, the LFNST is always disabled when the TS is applied to either Cb or Cr component. On the other hand, for single-tree, the LFNST is enabled for luma component but always disabled for chroma components. According to the current specification text, the LFNST is signaled when the following conditions are satisfied:

| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| --- | --- |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

As shown above, though the LFNST is only applied to luma components in single-tree, the corresponding LFNST signalling is still dependent on chroma components due to the following reasons:

- The presence of the LFNST signalling is conditioned on the transform skip flags of both luma and chroma components. The LFNST is disallowed when either luma or chroma applies transform skip.

- The variables LfnstDcOnly and LfnstZeroOutSigCoeffFlag (which indicate whether the LFNST DC only constraint and the LFNST zero-out constraint are satisfied or not, respectively) are determined based on the positions of last non-zero coefficients of both luma and chroma components.

Moreover, according to the VVC draft 8, the LFNST index is signaled at the end of one intra CU. It is asserted such design can cause serious decoding latency issue because the inverse transform cannot be invoked until the transform coefficients of all components become available.

Proposal

In this contribution, three methods are proposed to simplify the existing LFNST signaling design in VVC draft 8.

Only check luma coefficients for DC only constraint for single-tree partition

In the first method, it is proposed to only check transform coefficients of luma component when determining if there exists at least one non-DC coefficient for the LFNST signaling in single-tree partition.

Removing signaling dependency between luma and chroma components for LFNST signaling

In the second aspect, to achieve a cleaner design, the following two modifications are proposed to remove the dependency of the LFNST signalling upon chroma components in single-tree case:

- The LFNST signalling is only dependent on luma transform skip mode in single tree.

- Only the transform coefficients of luma component are considered when the DC only constraint and the zero-out region constraint are applied to the LFNST signalling in single-tree.

Additionally, to reduce the decoding latency it is proposed to move the LFNST and MTS signaling from the end of the CU to be after the TB of the last component that the LFNST and MTS can be applied to. Specifically, for single-tree and dual-tree luma partition, the LFNST and MTS indices are signaled after the luma TB (when the ISP mode is applied, they are signaled after the last luma TB). For dual-tree chroma partition, because the MTS is only applied to luma component, only the LFNST index is signaled after the TB of Cr component.

Removing chroma LFNST in both single-tree and dual-tree

In the third method, it is proposed to also disabled LFNST for chroma components in dual-tree partition such that the LFNST is only applied to luma components in both dual-tree and single-tree partition cases. Additionally, similar to Method Two, the signaling of both the LFNST and MTS indices are moved to be after the TB of the last component that the LFNST and MTS can be applied to.

Simulations

The proposed change was tested on top of VTM-8.0 [3] by following common test conditions (CTCs) [4].

BD-rate performance of Method One

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 96% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 103% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 95% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.00% | 98% | 99% |
| Class A2 | -0.01% | 0.10% | 0.13% | 100% | 101% |
| Class B | -0.01% | -0.04% | 0.00% | 98% | 99% |
| Class C | -0.01% | 0.04% | 0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.04% | 99% | 100% |
| Class D | 0.00% | 0.05% | -0.01% | 98% | 99% |
| Class F | 0.00% | 0.05% | -0.04% | 98% | 99% |

BD-rate performance of Method Two

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 95% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 96% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class D | 0.00% | 0.00% | 0.00% | 103% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 98% | 96% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.07% | 0.03% | 98% | 99% |
| Class A2 | 0.01% | -0.03% | 0.10% | 97% | 100% |
| Class B | 0.01% | -0.02% | 0.04% | 97% | 99% |
| Class C | -0.02% | 0.09% | 0.02% | 98% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | 0.04% | 97% | 99% |
| Class D | -0.01% | 0.04% | -0.01% | 96% | 95% |
| Class F | 0.00% | 0.03% | 0.00% | 97% | 99% |

BD-rate performance of Method Three

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.56% | 1.97% | 92% | 99% |
| Class A2 | -0.04% | 1.58% | 1.60% | 90% | 100% |
| Class B | 0.12% | 1.05% | 1.47% | 96% | 102% |
| Class C | 0.25% | 0.66% | 0.92% | 88% | 94% |
| Class E | 0.22% | 1.08% | 0.98% | 102% | 108% |
| Overall | 0.12% | 1.14% | 1.37% | 93% | 101% |
| Class D | 0.26% | 0.86% | 0.98% | 90% | 95% |
| Class F | 0.23% | 0.54% | 0.49% | 94% | 97% |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | 0.39% | 0.38% | 98% | 99% |
| Class A2 | 0.01% | 0.38% | 0.54% | 99% | 101% |
| Class B | 0.04% | 0.58% | 0.76% | 98% | 101% |
| Class C | 0.14% | -0.46% | -0.49% | 96% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.22% | 0.30% | 97% | 99% |
| Class D | 0.18% | 0.58% | 0.69% | 96% | 94% |
| Class F | 0.23% | -0.16% | 0.20% | 97% | 99% |

Reference

H. E. Egilmez, et al, “Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST Signaling, Latency Reduction and Scaling Process”, JVET-Q0784, Jan. 2020, Brussels, Belgium.

B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020, Brussels, Belgium.

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019, Geneva, Switzerland.

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).