| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0333-v2 |
| --- | --- |

| Title: | AHG11: Mismatches related to palette prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports some mismatches between VVC draft 8 and test model software VTM-8.0 on updating the palette prediction in local dual tree cases. First, in the VVC draft 8, under local-dual tree case the update process of palette prediction is performed only for chroma CU while in the VTM-8.0 it is performed for both luma and chroma CUs. Secondly, in the VVC draft 8, the palette prediction for a luma CU under a local-dual tree does not use the chroma values of those entries in the palette predictor, and vice versa. But in the VTM-8.0, luma and chroma values of each entry in the palette predictor are used together in palette prediction.

In this contribution, one method is proposed to solve the first mismatch and two methods are proposed to solve the second mismatch in different ways. In the first change, it is proposed to change the specification text to align with the VTM-8.0 software for palette update in the first mismatch. The proposed change has no impact on coding performance. In the second change, it is proposed to change the specification text to align with the VTM-8.0 software for forming a palette in the second mismatch. The proposed change has no impact on coding performance. In the third change, in forming a palette, the same default values used for signaled palette entries are also used for predicted palette entries to fix the second mismatch. The proposed change reports negligible BD-rate changes compared to VTM8.0.

# Problem statement

At the 17-th JVET meeting, palette mode for non 4:4:4 color format was adopted in VVC draft 8 [1] to support palette mode (PLT) for all of the chroma formats in a 4:4:4 profile (that is, 4:4:4, 4:2:0, 4:2:2 and monochrome). For coding of the palette, a palette predictor is maintained. The predictor is initialized to 0 at the beginning of each slice for non-wavefront case and at the beginning of each CTU row for wavefront case. After a CU is coded in palette mode, the palette predictor will be updated using the current palette, and entries from the previous palette predictor that are not reused in the current palette will be added at the end of the new palette predictor until the maximum size allowed is reached.

In this contribution, some mismatches were identified between the VVC draft 8 [1] and VTM-8.0 [2] software on the updating the palette prediction in local dual tree cases. Firstly, in the VVC draft 8, the update process of palette prediction is performed only for chroma CU in local dual tree cases. However, in the VTM-8.0, the update process of palette prediction is performed for both luma and chroma CUs. Secondly, in the VVC draft 8, the palette predictor update for a luma CU under a local-dual tree does not use the chroma values of those entries in the palette predictor, and vice versa. But in the VTM-8.0, the full values (including both luma and chroma components) of each entry in the palette predictor may be used.

# Proposed changes

## Proposed change #1

In the first change, it is proposed to change the specification text to align with the VTM-8.0 software in the first mismatch. So that palette predictor update is performed for both luma and chroma CUs. The changes to the VVC Draft are highlighted below.

8.4.5.3 Decoding process for palette mode

……

When localDualTree is equal to 1, the following applies:

- If treeType is equal to DUAL_TREE_LUMA, the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1:

CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(450)

CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(451)

- Otherwise (if treeType is equal to DUAL_TREE_CHROMA), the following applies for i = 0..num_signalled_palette_entries[ startComp ] − 1:

CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries + i ] = 1 << ( BitDepth − 1 )	(452)

- The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows:

CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]	(453)

startComp = 0		(454)

numComps = 3		(455)

maxNumPalettePredictorSize = 63		(456)

When one of the following conditions is true:

- cIdx is equal to 0 and numComps is equal to 1;

- cIdx is equal to 0 and localDualTree is equal to 1;

- cIdx is equal to 2;

the value PredictorPaletteSize[ startComp ] and the array PredictorPaletteEntries are derived or modified as follows:

……

## Proposed change #2

In the second change, it is proposed to change the specification text to align with the VTM-8.0 software in the second mismatch. So that luma and chroma values of each entry in the palette predictor are used together in palette prediction under local dual tree cases. The changes to the VVC Draft are highlighted below.

7.4.11.6 Palette coding semantics

……

new_palette_entries[ cIdx ][ i ] specifies the value for the i-th signalled palette entry for the colour component cIdx.

The variable PredictorPaletteEntries[ cIdx ][ i ] specifies the i-th element in the predictor palette for the colour component cIdx.

The variable CurrentPaletteEntries[ cIdx ][ i ] specifies the i-th element in the current palette for the colour component cIdx and is derived as follows:

localDualTree = treeType  !=  SINGLE_TREE  &&  
( slice_type  !=  I  | |  ( slice_type = = I  &&  qtbtt_dual_tree_intra_flag = = 0 ) ) ? 1 : 0

numPredictedPaletteEntries = 0
for( i = 0; i < PredictorPaletteSize[ startComp ]; i++ )
	if( PalettePredictorEntryReuseFlags[ i ] ) {
		for( cIdx = localDualTree ? 0 : startComp; cIdx < localDualTree ? 3 : ( startComp + numComps ); cIdx++ )
			CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries ] = PredictorPaletteEntries[ cIdx ][ i ]
		numPredictedPaletteEntries++
		}

for( cIdx = startComp; cIdx < (startComp + numComps); cIdx++)	(181)
	for( i = 0; i < num_signalled_palette_entries[startComp]; i++ )
		CurrentPaletteEntries[ cIdx ][ numPredictedPaletteEntries + i ] = new_palette_entries[ cIdx ][ i ]

……

## Proposed change #3

In the third change, to fix the second mismatch, the same default values used for signaled palette entries are also used for predicted palette entries. The changes to the VVC Draft are highlighted below.

8.4.5.3 Decoding process for palette mode

……

When localDualTree is equal to 1, the following applies:

- If treeType is equal to DUAL_TREE_LUMA, the following applies for i = 0..( NumPredictedPaletteEntries + num_signalled_palette_entries[ startComp ] – 1):

CurrentPaletteEntries[ 1 ][ NumPredictedPaletteEntries i ] = 1 << ( BitDepth − 1 )	(450)

CurrentPaletteEntries[ 2 ][ NumPredictedPaletteEntries i ] = 1 << ( BitDepth − 1 )	(451)

- Otherwise (if treeType is equal to DUAL_TREE_CHROMA), the following applies for i = 0.. ( NumPredictedPaletteEntries + num_signalled_palette_entries[ startComp ] − 1):

CurrentPaletteEntries[ 0 ][ NumPredictedPaletteEntries i ] = 1 << ( BitDepth − 1 )	(452)

- The variables CurrentPaletteSize[ 0 ], startComp, numComps and maxNumPalettePredictorSize are derived as follows:

CurrentPaletteSize[ 0 ] = CurrentPaletteSize[ startComp ]	(453)

startComp = 0		(454)

numComps = 3		(455)

maxNumPalettePredictorSize = 63		(456)

……

# Simulation Results

The proposed change #3 was implemented based on the VTM-8.0 reference software [2]. Following table shows the simulation results of the palette mode for 4:2:0 color format

Table 1. Simulation results of the palette mode

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class SCC | 0.00% | 0.00% | 0.00% | 101% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | -0.04% | -0.02% | 100% | 100% |
| Class SCC | 0.11% | 0.08% | 0.09% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class SCC | 0.05% | 0.08% | 0.04% | 99% | 99% |

# Reference

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, the 17th JVET meeting, Brussels, BE, 7–17 January 2020.

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).