| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R03431-r1 |
| --- | --- |

| Title: | AHG9: A summary of proposals on parameter sets cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry 10225 Willow Creek Rd San Diego, CA 92131, USA | Email: | dr.hendry@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This contribution intended to provide a summary of the proposals on parameter sets cleanups submitted to this JVET meeting by the 3 April 2020 submission deadline.

It is suggested that this summary is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

The following change was made in the first revision of this document:

- VPS cleanups category was removed. Proposals in this category will be considered in the discussion related to scalability / multi-layer.

- The following proposals were added in the summary: JVET-R0125 and JVET-R0301

- Fix editorial typos (i.e,. R0210 to be R0201)

# List of proposed items

Summary of proposals on SPS cleanups:

- New condition for signalling of syntax elements

  - When sps_ptl_dpb_hrd_params_present_flag is equal to 1, inter_layer_ref_pics_present_flag is not present and inferred to be equal to 0 (JVET-R0156 proposal 2)

  - Condition the presence of sps_sublayer_dpb_params_flag on the value of sps_ptl_dpb_hrd_params_present_flag, in addition to sps_max_sublayer_minus1 (JVET-R0156 proposal 3) (JVET-R0170) (JVET-R0222 proposal 2)

  - Condition that sps_independent_subpics_flag is present only when there are at least two subpictures. (JVET-R0156 proposal 4)

  - Condition the presence of subpic_info_present_flag by res_change_in_clvs_allowed_flag (JVET-R0266 proposal 3)

  - Add new flag in SPS to indicate that intra-only (i.e., whether intra-coding is allowed). Use this flag to condition the presence of inter-coding related tools. (JVET-R0283 proposal 1) (JVET-R0335)

  - In similar train of thought as point above, do the same for PPS (JVET-R0283 proposal 2)

- Infer the value of sps_ccalf_enabled_flag to be equal to 0 when not present (i.e., when sps_alf_enabled_flag is equal to 0) (JVET-R0105).

- Change the value range of syntax element sps_max_sublayers_minus1. (JVET-R0125)

Current range: “The value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive”

Proposed range: “When when sps_seq_parameter_set_id is greater than 0, the value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive.  When when sps_seq_parameter_set_id is equal 0, the value of sps_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive”

- Add a constraint on the value of sps_max_sublayers_minus1 such that when sps_video_parameter_set_id is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, sps_max_sublayers_minus1 shall be equal to vps_max_sublayers_minus1. (JVET-R0125)

- Constraint the semantics of subpic_info_present_flag such that when it is equal to 1, at least either pic_width_max_in_luma_samples or the value of pic_height_max_in_luma_samples shall be larger than the CTB size CtbSizeY. (JVET-R0156 proposal 5)

- Change sps_reserved_zero_4bits to sps_reserved_one_4bits to prevent the SPS start code emulation. (JVET-R0266 proposal 1)

- Consolidate two syntax elements, sps_poc_msb_flag and poc_msb_len_minus1 into a single syntax element poc_msb_len (JVET-R0266 proposal 2)

- gdr_enabled_flag value constrained by no_gdr_constraint_flag (JVET-R0266 proposal 5)

- Group syntaxes in SPS based on slice type (i.e. intra or inter) (JVET-0332)

Summary of proposals on PPS cleanups:

- Require the value of pps_conformance_window_flag to be equal to 0 when the picture width and height are the maximum picture width and height, and infer the values of the PPS conformance window syntax elements to be the same as those signalled in the SPS if the picture width and height are the maximum picture width and height and to be equal to 0 otherwise. (JVET-R0068 proposal 6) (JVET-R0262 proposal 1 and 2)

- New condition for signalling of syntax elements

  - Not signal the flags that indicate if parameters are signalled in picture header or in slice header (e.q., rpl_info_in_ph_flag, sao_info_in_ph_flag, etc), when there is no picture partitioning. Infer them to be equal to 0. (JVET-R0113 proposal 2).

  - New syntax element pps_res_change_allowed_flag in PPS and use it to condition the presence of pic height, conformance window, and scaling window syntax element (JVET-R0262 propposal 3)

- Allow signalling of negative scaling window offsets so that negative vertical and horizontal offsets can be derived even when the referenced picture did not include a scaling window (JVET-R0114)

- Add a constraint for the cropping window offsets and scaling window offsets that at least one of the offsets is different than its default value when the flag that controls their presence is equal to 1 (JVET-R0115)

- Change the signalling for wraparound offset. Signal “picture width minus wraparound offset” instead of “wraparound offset” (JVET-R0162 proposal 1)

Summary of proposals on APS cleanups:

- Handling chroma related syntax elements in APS when ChromaArrayType is equal to 0

  - Define a constraint in PH, not in APS, such that the value of scaling_list_chroma_present_flag to be equal to 0 when the value of ChromaArrayType is equal to 0. Similarly, do the same for al_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, alf_cc_cr_filter_signal_flag, and lmcs_delta_abs_crs (JVET-R0074)

    - Remark: this is asserted to remove APS to SPS dependency in the semantics

  - Add flags (i.e., alf_chroma_present_flag and lmcs_chroma_present_flag) and constraint them to be equal to 0 when ChromaArrayType is equal to 0 (JVET-R0177 proposal 1)

  - Add a flag in APS (i.e., aps_chroma_present_flag) and use this flag to condition the presence of chroma presence flags in the APS. (JVET-R0177 proposal 2) (JVET-R0301)

- Change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture (JVET-R0149 proposal 1)

- Disallow interleaving of APS NAL units of different subpictures (JVET-R0149 proposal 2)

- Constrain suffix APS NAL units to be located after the last VCL NAL unit of the PU (JVET-R0201)

- Allow prefix and suffix APS NAL units with particular APS identifier and type to have different content (JVET-R0201)

- Constrain prefix APS NAL unit to be located before the first VCL NAL unit of the PU(JVET-R0201)

- Add a mode in PH to allow APS to be signalled within PH. Just like the mode of signalling PH in SH (JVET-R0273)

# List of proposals

### Parameter sets cleanups (23)

#### SPS cleanups (9)

JVET-R0105 AHG9: On CC-ALF Signalling in SPS [S. Deshpande, A. Segall, J. Samuelsson, P. Cowan (Sharp)]

When sps_alf_enabled_flag is equal to 0 and/or when ChromaArrayType is equal to 0, sps_ccalf_enabled_flag is not signalled.

A missing inference rule is proposed for sps_ccalf_enabled_flag for when it is not present.

The proposed inference is as follows:

When not present sps_ccalf_enabled_flag is inferred to be equal to 0.

JVET-R0125 AHG8/AHG9: On signalling max number of sublayers [B. Choi, S. Wenger, S. Liu (Tencent) ]

This contribution proposed the following clean-ups on sps_max_sublayers_minus1 semantics:

- Inference rule of the sps_max_sublayers_minus1 is modified, so that vps_max_sublayers_minus1 is not referred when VPS is not present.

- The value of sps_max_sublayers_minus1 is set equal to vps_max_sublayers_minus1, when vps_all_layers_same_num_sublayers_flag is equal to 1.

JVET-R0156 AHG8/AHG9: Signalling cleanup on SPS [B. Wang, S. Esenlik, A. M. Kotra, H. Gao, E. Alshina (Huawei)]

This contribution proposed following cleanups for SPS:

- When sps_video_parameter_set_id is equal to 0, sps_ptl_dpb_hrd_params_present_flag is not signaled but inferred to be equal to 1.

- When sps_ptl_dpb_hrd_params_present_flag is equal to 1, inter_layer_ref_pics_present_flag is not signaled but inferred to be equal to 0.

- The signaling of sps_sublayer_dpb_params_flag is based on not only sps_max_sublayers_minus1, but also sps_ptl_dpb_hrd_params_present_flag.

- The signaling of sps_independent_subpics_flag is enabled only when there are at least two subpictures,

- Add a constraint to the semantics of subpic_info_present_flag.

The proposed constraint: when the value of subpic_info_present_flag is 1, for the value of pic_width_max_in_luma_samples and the value of pic_height_max_in_luma_samples, at least one of them shall be larger than the CTB size CtbSizeY.

JVET-R0170 AHG9: Removed Coding Redundant DPB Related Flag [K. Naser, F. Le Léannec, T. Poirier (InterDigital)]

The decoded picture buffer (DPB) parameters are coded at VPS and SPS level. In VVC draft 8, the flag “sps_ptl_dpb_hrd_params_present_flag” is used to allow/disallow coding the parameters at SPS level. However, even when DBP parameters are not coded at SPS level, the sublayer DPB parameter flag (sps_sublayer_dpb_params_flag) is coded.  This leads to coding a useless value.

This contribution proposed to remove the coding of this flag and infer its value to be zero.

The same as item 3 in JVET-R0156

JVET-R0222 AHG9: SPS sublayer syntax cleanup [J. Luo, J. Chen, Y. Ye (Alibaba)]

Item 1 of this contribution belongs to this category.

This contribution included two aspects for sublayer syntax cleanup in SPS:

- When sps_video_parameter_set_id is equal to 0, infer the value of vps_max_sublayers_minus1 is inferred to be equal to 6.

- Signal sps_sublayer_dpb_params_flag only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

JVET-R0266 AHG9: Miscellaneous HLS topics [Y. He, Y-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Items 1-3, 5 of this contribution belong to this category.

The contribution proposed the following changes on SPS syntax elements.

- Change sps_reserved_zero_4bits to sps_reserved_one_4bits to prevent the SPS start code emulation without bit cost.

- Consolidate two syntax elements, sps_poc_msb_flag and poc_msb_len_minus1 into a single syntax element poc_msb_len.

- Condition the presence of subpic_info_present_flag by res_change_in_clvs_allowed_flag

- Code PPS ID, pps_pic_parameter_set_id, with u(6)

- gdr_enabled_flag value constrained by no_gdr_constraint_flag

- Code the locations of virtual boundaries, i.e., ps_virtual_boundaries_pos_x[ i ], sps_virtual_boundaries_pos_y[ i ], ph_virtual_boundaries_pos_x[ i ], and ph_virtual_boundaries_pos_y[ i ], with ue(v).

JVET-R0283 AHG9: Cleanup of inter predication HLS syntax elements [K. Naser, F. Le Léannec, M. Kerdranvat, P. de Lagrange (InterDigital)]

This contribution proposed the following:

- An SPS flag is added to indicate of intra-only is used, or alternatively expressed, if inter-coding is not allowed. This flag is used to condition the presence of syntax element needed only for inter-predicted slices. Also use the flag to condition the presence of ph_inter_slice_allowed_flag and finally update the semantics of ph_intra_slice_allowed_flag.

- In similar way, a PPS flag is added to indicate if inter slices are allowed.

JVET-R0332 AHG9: On syntax signalling order in SPS [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

This contribution proposes to rearrange the syntax elements related to inter slices in the sequence parameter set (SPS) in a similar manner as the grouping of the inter-slice related syntaxes in picture header. More specifically, it is proposed to also group syntaxes in SPS based on slice type (i.e. intra or inter).

JVET-R0335 AHG9: On SPS inter slice related syntaxes [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

In current VVC, one flag ph_inter_slice_allowed_flag is signaled in the picture header to control the signaling of those syntax elements that are related to inter slices. In this proposal, a similar flag sps_inter_slice_allowed_flag is proposed to be added in SPS to control the signaling of those inter slice related syntax elements in SPS. When it is signaled as false, those inter slice related syntaxes in SPS are not signaled; additionally in this case the signaling of ph_inter_slice_allowed_flag in picture header can be omitted and inferred as false.

      - PPS cleanups (7)

JVET-R0068 AHG8/AHG9/AHG12: Miscellaneous HLS topics [Y.-K. Wang, L. Zhang, Z. Deng, J. Xu, K. Zhang, K. Fan (Bytedance)]

Item 6 of this contribution belongs to this category.

- Require the value of pps_conformance_window_flag to be equal to 0 when the picture width and height are the maximum picture width and height, and infer the values of the PPS conformance window syntax elements to be the same as those signalled in the SPS if the picture width and height are the maximum picture width and height and to be equal to 0 otherwise.

JVET-R0113 AHG9: On Picture Parameter Set [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

Item 2 of this contribution belongs in this category

This contribution included two proposals for modifications in the picture parameter sets (PPS). It is asserted that the proposed modification removes redundancy by only signaling some flags when the value of them are actually useful.

- Not signal the flags that indicate if parameters are signalled in picture header or in slice header, when there is no picture partitioning.

JVET-R0114 AHG9: On scaling window offsets [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

This contribution included a proposal for allowing negative scaling window offsets to be signalled in VVC. In the current draft of VVC, the scaling window offsets for both the current picture and the referenced picture are used to determine the scale factor and horizontal and vertical offsets used in the motion compensation process during inter prediction. It is asserted that it is currently possible that the vertical and horizontal offset becomes negative, for example if the current picture and the reference picture have the same resolution but the scaling window is smaller in the reference picture than in the current picture.

It is proposed to allow signalling of negative scaling window offsets so that negative vertical and horizontal offsets can be derived even when the referenced picture did not include a scaling window.

JVET-R0115 AHG9: On signalling of cropping windows and scaling windows [J. Samuelsson, S. Deshpande, A. Segall (Sharp)]

This contribution included a proposal of a bitstream constraint for the cropping window offsets and scaling window offsets, requiring that at least one of the offsets is different than its default value when the flag that controls their presence is equal to 1.

JVET-R0162 AHG9: PPS and SH syntax cleanup [J. Chen, J. Luo, Y. Ye, R.-L. Liao (Alibaba)]

Item 1 of this contribution belongs to this category.

This contribution included three aspects for PPS and SH syntax cleanup as follows

- It is proposed to signal “picture width minus wraparound offset” instead of “wraparound offset” in PPS to achieve more efficient signaling;

JVET-R0262 AHG9: On PPS syntax [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes following PPS syntax element changes for such typical no resolution change case.

- Propose the value of pps_conformance_window_flag to be equal to 0 when the PPS picture width and height are the same as the SPS maximum picture width and height

- Propose to infer the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset when the value of pps_conformance_window_flag is 0

- Propose pps_res_change_allowed_flag to condition the presence of PPS picture height, conformance window and scaling window syntax elements.

JVET-R0266 AHG9: Miscellaneous HLS topics [Y. He, Y-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Item 4 of this contribution belongs to this category.

- Code PPS ID, pps_pic_parameter_set_id, with u(6)

#### APS cleanups (7)

JVET-R0074 AHG9: Removal of APS semantics dependencies on SPS [Z. Deng, L. Zhang, Y.-K. Wang, K. Zhang (Bytedance)]

In the latest VVC text, it is asserted that there are some APS semantics dependencies on SPS. It is further asserted that APS semantics dependencies on SPS should not occur, because there is no PPS ID or SPS ID in the APS syntax and consequently an APS may be applied to pictures (or slices of pictures) that refer to different SPSs.

This contribution proposed the following changes for removing of APS semantics dependencies on SPS:

- It is required that, when ph_scaling_list_aps_id is present, the value of scaling_list_chroma_present_flag of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to ph_scaling_list_aps_id shall be equal to 0 if ChromaArrayType is equal to 0 and shall be equal to 1 otherwise.

- It is required that, when ph_alf_aps_id_luma[ i ] is present and ChromaArrayType is equal to 0, the values of alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to ph_alf_aps_id_luma[ i ] shall all be equal to 0.

  - Moreover, it is required that, when slice_alf_aps_id_luma[ i ] is present and ChromaArrayType is equal to 0, the values of alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall all be equal to 0.

- It is required that, when ph_lmcs_aps_id is present, the value of lmcs_delta_abs_crs of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to ph_lmcs_aps_id shall be equal to 0 if ChromaArrayType is equal to 0 and shall be greater than 0 otherwise.

- Remove the existing constraints in the semantics of scaling_list_chroma_present_flag, alf_chroma_filter_signal_flag, alf_cc_cb_filter_signal_flag, and alf_cc_cr_filter_signal_flag.

JVET-R0132 On signalling of chroma related APS [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)] [miss]

JVET-R0149 AHG9/AHG12: Relaxing an APS constraint [M. M. Hannuksela, M. Homayouni, A. Hallapuro, A. Aminlou (Nokia)]

VVC Draft 8 includes a constraint that the content of all APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a picture unit shall have the same content. This contribution asserted that, because of this constraint, ALF needs to be turned off in many applications using adaptive subpicture-based delivery or decoding.

To enable the use of ALF in such applications, the contribution proposed:

- To change the constraint on APS NAL units to have the same content within a picture unit to apply within a subpicture.

- To disallow interleaving of APS NAL units of different subpictures.

JVET-R0177 AHG9: APS Cleanup [K. Naser, F. Le Léannec, T. Poirier, P. de Lagrange (InterDigital)]

In the last JVET meeting, a partial cleanup for APS was adopted in JVET-Q0505, where chroma related scaling matrices are not signaled in APS in monochrome mode. It was motivated by the principle “not signaling something which would never be used” according to the meeting notes. It is asserted, however, that this cleaning is not complete for APS. This is because the other APS parameters (ALF and LMCS) still signal redundant chroma information in the case of monochrome mode.

This contribution proposed removing this redundant signaling by conditioning it on the chroma type as follows:

- Similar to scaling matrices, a flag for the availability of chroma components is coded in ALF and LMCS (i.e., alf_chroma_present_flag and lmcs_chroma_present_flag)

- Have one flag in APS (i.e., aps_chroma_present_flag) and use this flag to condition the presence of chroma presence flags in the APS. This consequently remove the current flag scaling_list_chroma_present_flag).

JVET-R0201 AHG9: On prefix and suffix APSs [N. Ouedraogo, G. Laroche, P. Onno (Canon)]

In this contribution, it is asserted that the random access or the bitstream extraction operations starting at an IRAP picture may lead to the generation of non-conformant bitstream due to the obligation of copying all the needed APS before the IRAP. This is particularly the case when a PU with slices refers to APSs signalled in a previous PU.

To solve the asserted issue, it is proposed:

- To constrain suffix APS NAL units to be located after the last VCL NAL unit of the PU.

- To allow prefix and suffix APS NAL units with particular APS identifier and type to have different content.

Second, it is asserted that the VVC Draft 8 does not constrain two slices of a PU to refer to two APSs with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type but with different contents when one of the APSs is within a previous PU.

In addition to the previous constraints, it is proposed:

- To constrain prefix APS NAL unit to be located before the first VCL NAL unit of the PU.

JVET-R0273 AHG9: APS signalled in picture header [V. Seregin, M. Coban, Y. He, M. Karczewicz (Qualcomm)]

This contribution proposes to add a mode of APS signalling inside a PH NAL in a similar manner as picture header signalled inside a VCL NAL. This new mode is indicated by a flag in a picture header followed by the number of APSs included into a PH NAL.

In VVC draft 8, there is mode when PH structure can be signalled inside a VCL NAL for the purpose of saving NAL unit header overhead and byte alignment at the end of a NAL. For the same purpose and benefit, it is proposed to signal APSs inside a PH NAL. This new mode of APS signalling is indicated by a flag in PH followed by a syntax element indicating the number of included APSs.

The proposed APS signalling was implemented in VTM-8.0 and tested under common test condition. The gain was reported: For AI: 0.03%, 0.03%, and 0.03% for Y, U, V respectively. For RA: 0.03%, 0.03%, and 0.03% for Y, U, V, respectively. For LD-B: 0.05%, 0.06%, and 0.06% for Y, U, V, repectively.

It was remarked in the contribution that on the current CTC, APS is not signalled frequently, however if it is signalled per picture then the bit saving should be comparable to the results reported in JVET-Q0255 and may be even better if more than one APS is included into PH NAL and signalled per picture.

JVET-R0301 AHG12: on scaling_list_chroma_present_flag in APS [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

This contribution proposes two changes.

- Move existing scaling_list_chroma_present_flag forward in APS such that the syntax element could be used for both ALF and LMCS related syntax elements.

- Change the syntax element name to aps_chroma_present_flag to make it more general