| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0349-v1 |
| --- | --- |

| Title: | Crosscheck of AHG9/AHG12: On CTU row based slice chunks of a slice within a tile |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen Jiancong Luo Yan Ye | Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution reports crosscheck results of CTU row based slice chunks of a slice within a tile JVET-R0129[1]. It is reported that the coding results are identical with those reported by the proponent.

- Introduction

JVET-R0129 proposes to specify CTU row based slice chunks of a slice within a tile in decoding order. The slice chunks in sequence of a slice are delivered and decoded sequentially. Each slice chunk is contained in a single NAL unit.

Simulation results

Two tests are crosschecked in this contribution. The anchor is VTM8.0 CTC (common test condition) Lowdelay configuration only with 1 second. The results are shown in Tables 1 - 2

In Test 1, the single tile picture is partitioned into slices that each of the slices contains one CTU row. In Test 2, the single tile picture is partitioned into one slice (i.e., single tile and single slice in the picture) with the proposed slice chunks that each of the slice chunks contains one CTU row. Thus in both tests, one CTU row is contained in a single NAL unit and is simulated as the encoding latency for delivery.

Table 1.  LB short length results of Test 1 - Each slice contains one CTU row

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 6.19% | 7.56% | 8.84% | 117% | 51% |
| Class A2 | 4.80% | 5.12% | 5.31% | 116% | 49% |
| Class B | 5.12% | 5.61% | 6.13% | 111% | 48% |
| Class C |  |  |  |  |  |
| Class E | 11.77% | 12.15% | 10.70% | 118% | 51% |
| Overall | 6.71% | 7.33% | 7.52% | 115% | 50% |
| Class D |  |  |  | 105% | 98% |
| Class F |  |  |  | 105% | 104% |

Table 2.  LB short length results of Test 2 -Each slice chunk contains one CTU row

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM8.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.35% | 1.37% | 1.25% | 101% | 2984% |
| Class A2 | 1.04% | 0.98% | 0.97% | 101% | 3266% |
| Class B | 1.28% | 1.41% | 1.34% | 101% | 3350% |
| Class C |  |  |  |  |  |
| Class E | 6.22% | 6.58% | 6.43% | 101% | 2580% |
| Overall | 2.30% | 2.42% | 2.33% | 101% | 3073% |
| Class D |  |  |  | 101% | 100% |
| Class F |  |  |  | 100% | 98% |

References

[1] L. Chen, C.-W. Hsu, C.-C. Chen, Y.-L. Hsiao, C.-Y. Chen, T.-D. Chuang, C.-M. Tsai, Y.-W. Huang, S.-M. Lei, AHG9/AHG12: On CTU row based slice chunks of a slice within a tile, JVET-R0129, teleconference, April, 2020.