| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: Alpbach, AT, 15–24 April 2020 | Document: JVET-R0350-v4 |
| --- | --- |

| Title: | MIP for all channels in the case of 4:4:4-chroma format and of single tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | J. Pfaff, B. Stallenberger, P. Merkle, M. Schäfer, P. Helle, T. Hinz, H. Schwarz, D. Marpe, T. Wiegand | Tel: Email: | +49 jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In this document, for 4:4:4 chroma-format and single tree, it is proposed that on a chroma intra-block for which the chroma-intra mode is the DM mode and for which the luma intra-mode is a MIP mode, the chroma intra prediction signal is to be generated using this MIP mode. No change of the MIP-matrices or the MIP-modes themselves is proposed. The proposed method does not propose any change for content which is non-4:4:4 or non-single-tree. 
Experimental results of -0.25%/-0.23%/-0.22% and of -0.15%/-0.13%/-0.07% for 4:4:4 natural content in YUV and of -1.81%/-0.61%/-1.00% and -1.28%/-0.38%/-0.55% for 4:4:4 natural content in RGB (GBR-numbers) are reported for the AI and RA configurations respectively. For 4:4:4 screen content, experimental results of -0.49%/-0.40%/-0.37% and -0.35%/-0.28%/-0.30% for RGB (GBR-numbers) and of -0.09%/-0.06%/-0.01% and -0.04%/-0.04%/-0.11% for YUV are reported for the AI and RA configurations respectively.
During the discussion of the category 2 AHG pre-meeting on Tuesday, April 14th 2020, it was suggested that contributions related to 4:4:4 content should generate results that include the proposed bug-fix of the CTC for 4:4:4 content of document JVET-R0321. These results are included in version 2 of this document, where for RA, they are not complete yet and will be uploaded in a future version. The software that includes the proposed method as well as the changes needed for the update of the CTC is included in version 2 of this document.

Version 3 of this document contains presentation slides.

Version 4 contains complete results for the simulations with the proposed bugfix in anchor and test.

# Description of the proposed method

In the current VTM, MIP is used only for the luma-component [1]. If on a chroma intra-block the intra-mode is the direct mode (DM) and if the intra-mode of the co-located luma block is an MIP-mode, then the chroma block has to generate the intra-prediction signal using the planar mode. It is asserted that the main reason for this treatment of the DM-mode in the case of MIP is that for the 4:2:0 case or for the case of dual-tree, the co-located luma block can have a different shape than the chroma block. Thus, since an MIP-mode cannot be applied for all block-shapes, the MIP mode of the co-located luma block may not be applicable to the chroma block in that case.

On the other hand, it is asserted that for the case that the chroma format is 4:4:4 and that the single tree is used, the above non-applicability of a luma MIP-mode to the chroma components never holds. Thus, it is proposed that in this case, if on an intra-block the chroma-intra mode is the DM mode and if the luma intra mode is a MIP mode, the chroma intra-prediction signals are to be generated with this MIP mode.

It is asserted that this proposed change is relevant in particular if ACT is enabled since for the case that the ACT is used on a given block, the chroma mode is inferred to be the DM-mode. It is asserted that in particular for the ACT-case, a strong correlation of the prediction signals across all channels is beneficial and that such a correlation can be increased if the same intra-prediction mode is used for all three components of a block. It is asserted that the experimental results reported below support his view in the sense that the proposed changes has a significant impact for camera captured content in the RGB-format, which is asserted to be the intersection of the cases for which MIP and ACT were primarily designed.

# Experimental results

In this section, experimental results are reported for RGB 4:4:4 and YUV 4:4:4 according to the common test conditions for non-4:2:0 [2]. Results for 4:2:2 content are not reported since the proposed change does not change the bit-stream for such content. The single-tree is always enabled since for single-tree off, the proposed change does not change the bit-stream. All corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

In Table 1 and Table 2, results for 4:4:4 natural content are reported for the proposed change compared to the VTM-8.0 anchor for the AI and the RA configuration, respectively. Note that the simulations for the RGB 4:4:4 sequence DucksAndLegs and the YUV 4:4:4 sequence CrowdRun both crashed for the RA configuration at QP=22 due to an ALF-related bug in the VTM (issue #947). Those sequences are therefore not included in the results in Table 2. Additional results with ALF disabled in anchor and test can be found in the Tables 3 and 4. Here, all sequences are included.

In Table 5 and Table 6, results for RGB 4:4:4 screen content are presented for the proposed change compared against the VTM-8.0 anchor for the AI and the RA configuration, respectively. In Table 7 and Table 8 results for YUV 4:4:4 screen content are shown for the proposed change compared to the VTM-8.0 anchor for the AI and the RA configuration, respectively.

In Tables 9-14, results are reported where in anchor and test, the proposed bug-fix of the CTC for 4:4:4 content of document JVET-R0321 is included. Moreover, in anchor and test, the fix for the ALF-related bug of VTM (issue #947) is included. The software used to generate the results of Tables 9-14 is attached. In Table 9 and 10, results of the proposed method for 4:4:4 natural content are reported for the AI and the RA configuration, respectively. Here, for the RA configuration, only partial results are available so far. In Table 11 and Table 12, results of the proposed method for RGB screen content are reported for the AI and the RA configuration, respectively. In Table 13 and Table 14, results of the proposed method for YUV 4:4:4 screen content are reported for the AI and the RA configuration, respectively.

Table 1. Results of the proposed change for 4:4:4 natural content: Reference is VTM-8.0 anchor, test is VTM-8.0 with the proposed changes, AI configuration. Single tree is enabled in anchor and test.

|  | Y/G | U/B | V/R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| YUV 4:4:4 | -0.25% | -0.23% | -0.22% | 100% | 102% |
| RGB 4:4:4 | -1.81% | -0.61% | -1.00% | 97% | 101% |
| Overall | -0.81% | -0.33% | -0.48% | 99% | 102% |

Table 2. Results of the proposed change for 4:4:4 natural content: Reference is VTM-8.0 anchor, test is VTM-8.0 with the proposed changes, RA configuration. Single tree is enabled in anchor and test.

|  | Y/G | U/B | V/R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| YUV 4:4:4 | -0.15% | -0.13% | -0.07% | 101% | 102% |
| RGB 4:4:4 | -1.28% | -0.38% | -0.55% | 100% | 103% |
| Overall | -0.49% | -0.17% | -0.21% | 100% | 103% |

Table 3. Results of the proposed change for 4:4:4 natural content: Reference is VTM-8.0 anchor, test is VTM-8.0 with the proposed changes, AI configuration. Single tree is enabled in anchor and test. ALF is disabled in anchor and test.

|  | Y/G | U/B | V/R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| YUV 4:4:4 | -0.23% | -0.29% | -0.29% | 102% | 102% |
| RGB 4:4:4 | -1.76% | -0.65% | -1.05% | 99% | 100% |
| Overall | -0.71% | -0.33% | -0.47% | 101% | 102% |

Table 4. Results of the proposed change for 4:4:4 natural content: Reference is VTM-8.0 anchor, test is VTM-8.0 with the proposed changes, RA configuration. Single tree is enabled in anchor and test.  ALF is disabled in anchor and test.

|  | Y/G | U/B | V/R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| YUV 4:4:4 | -0.09% | -0.07% | -0.04% | 100% | 102% |
| RGB 4:4:4 | -1.18% | -0.38% | -0.57% | 99% | 100% |
| Overall | -0.46% | -0.16% | -0.22% | 99% | 100% |

Table 5. RGB-result of the proposed change for 4:4:4 screen content: Reference is VTM-8.0 anchor, test is VTM-8.0 with the proposed changes, AI configuration.

|  | G | B | R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| TGM 1080 | -0.05% | -0.05% | -0.03% | 100% | 101% |
| TGM 720 | -0.49% | -0.35% | -0.31% | 101% | 96% |
| Animation | -0.77% | -0.59% | -0.62% | 95% | 100% |
| Mixed | -0.81% | -0.74% | -0.65% | 99% | 99% |
| Overall | -0.49% | -0.40% | -0.37% | 99% | 99% |

Table 6. RGB-result of the proposed change for 4:4:4 screen content: Reference is VTM-8.0 anchor, test is VTM-8.0 with the proposed changes, RA configuration.

|  | G | B | R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| TGM 1080 | -0.03% | -0.01% | -0.03% | 99% | 93% |
| TGM 720 | -0.30% | -0.22% | -0.29% | 97% | 97% |
| Animation | -0.41% | -0.35% | -0.37% | 99% | 100% |
| Mixed | -0.81% | -0.63% | -0.59% | 99% | 101% |
| Overall | -0.35% | -0.28% | -0.30% | 99% | 97% |

Table 7. YUV-results of the proposed change for 4:4:4 screen content: Reference is VTM-8.0 anchor, test is VTM-8.0 with the proposed changes, AI configuration.

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| TGM 1080 | -0.01% | -0.02% | -0.02% | 101% | 99% |
| TGM 720 | -0.08% | 0.01% | -0.04% | 101% | 100% |
| Animation | -0.19% | -0.09% | -0.19% | 99% | 100% |
| Mixed | -0.10% | -0.17% | -0.19% | 99% | 101% |
| Overall | -0.09% | -0.06% | -0.10% | 100% | 100% |

Table 8. YUV-results of the proposed change for 4:4:4 screen content: Reference is VTM-8.0 anchor, test is VTM-8.0 with the proposed changes, RA configuration.

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| TGM 1080 | 0,04% | 0,03% | -0,01% | 101% | 104% |
| TGM 720 | -0,05% | 0,04% | -0,03% | 100% | 100% |
| Animation | -0,07% | -0,15% | -0,16% | 101% | 103% |
| Mixed | -0,14% | -0,15% | -0,26% | 101% | 97% |
| Overall | -0,05% | -0,04% | -0,10% | 100% | 101% |

Table 9. Results of the proposed change for 4:4:4 natural content, AI configuration. Anchor and test include bug-fix of JVET-Q0321 and fix for ALF-related bug. Single tree is enabled in anchor and test.

|  | Y/G | U/B | V/R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| YUV 4:4:4 | -0,26% | -0,15% | -0,17% | 101% | 101% |
| RGB 4:4:4 | -1,65% | -0,60% | -1,01% | 98% | 98% |
| Overall | -0,68% | -0,27% | -0,42% | 100% | 100% |

Table 10. Results of the proposed change for 4:4:4 natural content, RA configuration. Anchor and test include bug-fix of JVET-Q0321 and fix for ALF-related bug. Single tree is enabled in anchor and test.

|  | Y/G | U/B | V/R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| YUV 4:4:4 | -0,17% | -0,14% | -0,06% | 100% | 101% |
| RGB 4:4:4 | -1,24% | -0,35% | -0,52% | 99% | 98% |
| Overall | -0,50% | -0,17% | -0,21% | 100% | 99% |

Table 11. RGB-result of the proposed change for 4:4:4 screen content, AI configuration. Anchor and test include bug-fix of JVET-Q0321 and fix for ALF-related bug. Single tree is enabled in anchor and test.

|  | G | B | R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| TGM 1080 | -0,06% | -0,05% | -0,04% | 99% | 101% |
| TGM 720 | -0,48% | -0,30% | -0,34% | 100% | 99% |
| Animation | -0,64% | -0,50% | -0,57% | 98% | 102% |
| Mixed | -0,83% | -0,77% | -0,68% | 98% | 101% |
| Overall | -0,47% | -0,37% | -0,38% | 99% | 101% |

Table 12. RGB-result of the proposed change for 4:4:4 screen content, RA configuration. Anchor and test include bug-fix of JVET-Q0321 and fix for ALF-related bug. Single tree is enabled in anchor and test.

|  | G | B | R | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| TGM 1080 | -0,08% | -0,07% | -0,08% | 99% | 102% |
| TGM 720 | -0,30% | -0,29% | -0,28% | 99% | 101% |
| Animation | -0,47% | -0,44% | -0,43% | 100% | 97% |
| Mixed | -0,84% | -0,71% | -0,64% | 98% | 96% |
| Overall | -0,39% | -0,35% | -0,33% | 99% | 99% |

Table 13. YUV-results of the proposed change for 4:4:4 screen content, AI configuration. Anchor and test include bug-fix of JVET-Q0321 and fix for ALF-related bug. Single tree is enabled in anchor and test.

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| TGM 1080 | 0,00% | 0,00% | -0,02% | 100% | 98% |
| TGM 720 | -0,06% | -0,10% | 0,00% | 100% | 100% |
| Animation | -0,19% | -0,13% | -0,26% | 100% | 100% |
| Mixed | -0,09% | -0,06% | -0,12% | 100% | 99% |
| Overall | -0,08% | -0,07% | -0,09% | 100% | 99% |

Table 14. YUV-results of the proposed change for 4:4:4 screen content, RA configuration. Anchor and test include bug-fix of JVET-Q0321 and fix for ALF-related bug. Single tree is enabled in anchor and test.

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| TGM 1080 | -0,04% | -0,07% | -0,06% | 99% | 102% |
| TGM 720 | -0,13% | -0,04% | 0,07% | 100% | 102% |
| Animation | -0,05% | -0,09% | -0,01% | 100% | 102% |
| Mixed | -0,06% | -0,10% | 0,01% | 100% | 101% |
| Overall | -0,07% | -0,07% | 0,00% | 100% | 102% |

# Conclusion

In this document, it is proposed to enable MIP for all three channels in the case of 4:4:4 content and single tree. For this case, if on an intra-block the luma component uses a MIP mode and if the chroma intra mode is the DM-mode, it is proposed to generate the chroma intra-prediction signals with this MIP mode. It is proposed to adopt the technology described in the present document to the next working draft of the VVC.

# References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), Document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, JVET common test conditions and software reference configurations for non-4:2:0 colour formats, Document JVET-Q2013, Version 5, Brussels, Belgium, January 2020

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).