| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0351 |
| --- | --- |

| Title: | High bit depth coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Karl Sharman Steve Keating | Email: | adrian.browne@sony.com karl.sharman@sony.com steve.keating@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

This document presents the current capabilities of VVC when operating at a range of bit depths (8, 10, 12, 14 and 16 bit), and low QPs.  Such testing is designed to show the correctness of VVC under the currently defined profiles and to identify problems which will need to be solved for higher bit profiles that may defined in a future version of VVC.  This document identifies and tests a number of transforms where enhanced numerical accuracy can improve the results and extend the current operating range of VVC.  It is proposed that the extended precision flag is kept in the reference software, VTM, as this will allow further study of high bit depths in version 2.  Acceptance of a patch for a number of bugs encountered whilst studying VVC with high bit depths is proposed.

# Introduction

In a previous study on HEVC[1], a series of issues coding at higher bit depths were identified.  These resulted in the adoption of an extended precision flag in HEVC Range Extensions.  The purpose of this flag was, when set, to increase the value of MAX_TR_DYNAMIC_RANGE (which is the bit depth of coefficients between the inverse transform and inverse quantizer, excluding the sign bit) from 15 to the maximum of 15 and bitDepth + 6.  This increases the precision of transformed values passed from the transform, through to the entropy encoder.

In addition the proposal described the use of higher precision 14 bit forward transforms rather than the standard 6 bit transforms.  This was non-normative, but showed how PSNRs approaching the theoretical maximum achievable for each bit depth could be achieved (Table 1). This table shows that for each successive increase of 2 bits, a PSNR increase of 12dB should be achievable.

| MSE | Bit depth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 8 | 10 | 12 | 14 | 16 |
| 0.25 | 54.2 | 66.2 | 78.3 | 90.3 | 102.4 |
| 0.50 | 51.1 | 63.2 | 75.3 | 87.3 | 99.3 |
| 1.00 | 48.1 | 60.2 | 72.2 | 84.3 | 96.3 |

Table 1. Theoretical maximum PSNR (dB) for a range of bit depths and MSEs

The effect of using extended precision processing and high precision forward transforms can be seen by comparing the following three figures.  Each figure is produced by plotting PSNR against bitrate for Y/G  for CrowdRun (16 bit RGB) at multiple bit depths in AI configuration (first four frames only). All PSNRs quoted in this document are against this 16 bit source, rather than the input once it has been downshifted for the encoder.

In all the graphs for HM16.21, TransformSkip has been disabled to show the effect of transform calculations. For each bit depth, the range of QP used was:

20  ≤  QP  ≤  -6 * (bitDepth – 8)

Figure 1. HM16.21 Base

Figure 1 clearly shows that 14 bit and 16 bit performance is severely degraded at low QP.  In addition, at no QPs do they perform significantly better than 12 bit processing, which in turn does not achieve a 12dB improvement over 10 bit processing.  From this it is clear than HM without extended precision can accommodate 10 bit processing, but 12 and above bit processing is compromised.

Figure 2. HM16.21 with extended precision processing enabled

Comparing figures 1 and 2 it is clear that enabling extended precision has removed the larges losses at low QP for 14 and 16 bit processing.  However, 14 and 16 bit processing still show no improvement over 12 bit processing, and 12 bit processing fails to show the expected 12 dB improvement over 10 bit processing.

Figure 3. HM16.21 with extended precision and high precision forward transforms enabled

Figure 3 shows that enabling high precision transforms in HM16.21 not only significantly improves performance at 14 and 16 bit, it also lifts 12 bit performance by 8dB at the lowest QP, and 10 bit performance is also slightly improved.

Table 2 shows the PSNR at each bit depth at QP’=4, i.e. 4 above the minimum allowable QP for the bit depth. This is the theoretical point at which the encoding should become lossless, if the quantizer and inverse quantizer are perfectly matched.  By comparing the values in the third row of this table with the top row in table 1, it is clear that with extended precision and high precision forward transforms enabled the encoding is approaching lossless.

| Extended precision | High precision transform | Bit depth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 10 | 12 | 14 | 16 |
| N | N | 58.53 | 68.91 | 73.96 | 74.56 | 43.15 |
| Y | N | 57.70 | 68.38 | 74.33 | 75.60 | 75.30 |
| Y | Y | 58.63 | 69.94 | 82.73 | 94.30 | 112.89 |

Table 2. PSNR (dB) at QP’=4 for HM16.21.

It is suggested that to be a genuinely versatile codec VVC should attempt, in the long term, to achieve comparable performance improvements over higher bit depths as shown above.  For current and conceivable version 1 VVC profiles it should be shown that 8, 10 and 12 bit performance can meet these near lossless PSNRs.

# Current state of VTM

VTM 8.0 still contains the RExt__HIGH_BIT_DEPTH_SUPPORT and RExt__HIGH_BIT_PRECISION_FORWARD_TRANSFORM preprocessor directives from HM.  However, no high precision transforms have been defined, and 6 bit transforms are used throughout.  In addition, the extended precision flag has been carried over from HM to VTM and is contained in the SPS.

When RExt__HIGH_BIT_DEPTH_SUPPORT was set, VTM could not be built due to a large number of compiler errors.  These required quite significant syntax and type casting changes. In addition the decoder required a bug fix for coefficient coding due to the use of mixed signed and unsigned data types. A patch file for these errors had been created and a merge request will be made if requested.

When testing VTM the PLT, LMCS, DepQuant and TransformSkip tools were disabled.  PLT and LMCS were disabled due to their non-scalable implementation requirements, especially in the encoder. DepQuant was disabled as the code cautions against its use with extended precision. TransformSkip was disabled to be consistent with the HM 16.21 experiments.

Once the compiler and decoder errors were resolved, the graphs shown in figures 4 and 5 were obtained.

Figure 4. VTM8.0 with extended precision disabled

Figure 5. VTM8.0 with extended precision enabled

# VTM tool patches

As can be seen from comparing figures 2 and 5, 16 bit performance was subject to some form of gross numerical error.  On investigation this was caused by another mixed calculation of signed and unsigned data types.  In addition, a number of other tools (ALF, BIO and BCW) were identified with similar issues, or with numerical overflows in types not suitable for extended precision processing.  A further patch has been created with these fixes and a merge request will be made on request.

Once these fixes were in place, the graph in Figure 6 was achieved.

Figure 6.  Patched VTM 8.0 with extended precision enabled

When comparing figure 6 with figure 2 it can be seen that the PSNR achieved by 12, 14 and 16 bit processing for VTM is higher than that for HM when the high precision forward transform is disabled.  Table 3 contains QP’=4 figures for the patched VTM8.0 in various configurations, as further described in the following sections.

| Configuration |  |  |  | Bit depth |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High Precision Transform | High Precision LFNST | High Precision Quantization | Additional tool on | 8 | 10 | 12 | 14 | 16 |
|  |  |  |  | 58.23 | 68.89 | 79.68 | 86.96 | 88.84 |
| Y |  |  |  | 58.25 | 69.10 | 80.72 | 92.19 | 106.36 |
|  | Y |  |  | 58.23 | 68.89 | 79.68 | 86.96 | 88.84 |
|  |  | Y |  | 58.22 | 68.89 | 79.67 | 86.87 | 88.65 |
| Y |  |  | DepQuant | 57.94 | 68.61 | 80.20 | 91.73 | 101.02 |
| Y |  |  | LMCS | 58.25 | 69.11 | 80.72 | 92.19 | 106.36 |
| Y |  |  | Transform Skip | 58.43 | 69.14 | 80.72 | 92.19 | 106.37 |

Table 3. PSNR (dB) at QP’=4 for patched VTM 8.0 with extended precision enabled.

# High precision forward transform

HEVC Range Extensions achieved higher PSNR when extended precision was enabled by using a forward transform with 14 fractional bits rather than the standard 6 fractional bits.  The need for the number of fractional bits to be at least (bitDepth -2) was established previously in HEVC[1],  Transforms with 14 fractional bits have been developed for DCT2, DCT8 and DST7 for all required transform sizes from 2x2 to 64x64. To ensure the accuracy of these transforms they were developed by creating a perfect inverse to the 6 fractional bit inverse transform specified in VVC.  These was then rounded to provide 14 fractional bits. Details of the transforms can be found in Appendix A.

Figure 7 shows the result of using high precision forward transforms in VTM. For each bit depth, the PSNR at the lowest applicable QP for each bit depth exceeds the expected PSNR in table 1, implying that the coding was approaching lossless.

Figure 7. Patched VTM 8.0, high precision forward transform and extended precision enabled

# High precision forward LFNST

LFNST is a new tool in VTM, and the limitations of the restricted bit depth of the 8 bit transforms may also introduce numerical errors. Introducing a separate forward LFNST transform with the same number of fractional bits as the inverse was investigated (figure 8), as well as a high precision forward LFNST transform with 8 more bits than the inverse (figure 9).  Unlike the forward transform described in section 4, neither appears to offer a significant gain.

Figure 8.  Patched VTM 8.0 with forward LFNST and extended precision enabled

Figure 9. Patched VTM 8.0 with high precision forward LFNST and extended precision enabled

# High precision quantization matrix

Another potential source of error is the quantization matrix.  A replacement with 4 additional fractional bits was tested (figure 10), but the results do not show significant gain.

Figure 10. Patched VTM 8.0 with higher precision quantization and extended precision enabled

# Testing with disabled tools

Previously all graphs for VTM have been created from simulations in which the following tools have been disabled: DepQuant, LMCS, TransformSkip and PLT. The following graphs show results if DepQuant, LMCS and Transform Skip are enabled respectively.  Palette mode proved impracticable to enable as its encoder memory requirements were excessive for high bit depths (arrays in the order of 232 elements).

Figure 11. Patched VTM 8.0 with DepQuant, high precision forward transform, and extended precision enabled.

Figure 12. Patched VTM 8.0 with LMCS, high precision forward transform, and extended precision enabled.

Figure 13. Patched VTM 8.0 with TransformSkip, high precision forward transform, and extended precision enabled.

From these results, it is clear that enabling either LMCS or TransformSkip does not significantly change the results.  However DepQuant cannot achieve the expected PSNR at the lowest QPs, probably due to a numerical overflow.

# Discussion

Given the results presented above it is recommended that the following actions are undertaken:

- The extended precision flag is not removed from VTM.  This flag allows further investigation of tools and techniques beyond the existing established profiles and into areas which will make VVC truly versatile.

- The bug fixes reported here are adopted (a merge request will be made when required).  None of these changes requires a specification change.

- In any future version 2 of VVC, higher bit depths up to 16 bit are studied for application areas including medical imaging.

# Conclusions

This document has presented the current capabilities of VVC when operating at a range of bit depths (8, 10, 12, 14 and 16 bit), and low QPs.  The results provided indicate that although the current numerical accuracy is adequate for 8, 10 and 12 bit processing, the performance of bit depths of 12, 14 and 16 could be improved by increasing the numerical accuracy within VVC.  In addition a number of methods to further improve the capability of the VVC at high bit depths have been presented.  One of these, based on a method from HEVC Range Extensions raises the PSNR achievable at high bit depths towards the theoretical maximum.

It is strongly suggested that the extended precision flag, which enables higher numerical accuracy, is kept in VTM despite not being used in any currently proposed profiles.   It is proposed that a new high bit depth study is undertaken in version 2.  In order to facilitate this future work, it is proposed that VTM is patched with the fixes identified in this study.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] “AHG 5 and 18: Internal Precision for High Bit Depths”, JCT-VCN0188, K. Sharman, N. Saunders and J. Gamei

[2] “Versatile Video Coding (Draft 8)”, JVET-Q2001-vE, B. Bross, J. Chen, S. Liu and Y-K. Wang

Appendix A: High Precision Forward Transform Matrices

This appendix details the transform matrices referred to in Section 5.

DCT2

For ease of implementation, a single 64x64 matrix M32 can be described for DCT2, from which each smaller NxN DCT2 matrix MN is derived through subsampling according to the following:

MN[x][y] = M64[x][(2(6 – log2(N)))y]      for x, y = 0..(N – 1).

Due to the size of the matrix, the complete matrix M64 has been split into four quadrants for the ease of presentation.

The top left quadrant of the DCT2 64x64 matrix is of the form:

| aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | Aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bf | bg | bh | bi | bj | bk | bl | bm | bn | bo | bp | bq | br | bs | bt | bu | bv | bw | Bx | by | bz | ca | cb | cc | cd | ce | cf | cg | ch | ci | cj | ck |
| ap | aq | ar | as | at | au | av | aw | ax | ay | az | ba | bb | bc | bd | be | -be | -bd | -bc | -bb | -ba | -az | -ay | -ax | -aw | -av | -au | -at | -as | -ar | -aq | -ap |
| bg | bj | bm | bp | bs | bv | by | cb | ce | ch | ck | -ci | -cf | -cc | -bz | -bw | -bt | -bq | -bn | -bk | -bh | -bf | -bi | -bl | -bo | -br | -bu | -bx | -ca | -cd | -cg | -cj |
| ah | ai | aj | ak | al | am | an | ao | -ao | -an | -am | -al | -ak | -aj | -ai | -ah | -ah | -ai | -aj | -ak | -al | -am | -an | -ao | ao | an | am | al | ak | aj | ai | ah |
| bh | bm | br | bw | cb | cg | -ck | -cf | -ca | -bv | -bq | -bl | -bg | -bi | -bn | -bs | -bx | -cc | -ch | cj | ce | bz | bu | bp | bk | bf | bj | bo | bt | by | cd | ci |
| aq | at | aw | az | bc | -be | -bb | -ay | -av | -as | -ap | -ar | -au | -ax | -ba | -bd | bd | ba | Ax | au | ar | ap | as | av | ay | bb | be | -bc | -az | -aw | -at | -aq |
| bi | bp | bw | cd | ck | -ce | -bx | -bq | -bj | -bh | -bo | -bv | -cc | -cj | cf | by | br | bk | Bg | bn | bu | cb | ci | -cg | -bz | -bs | -bl | -bf | -bm | -bt | -ca | -ch |
| ad | ae | af | ag | -ag | -af | -ae | -ad | -ad | -ae | -af | -ag | ag | af | ae | ad | ad | ae | Af | ag | -ag | -af | -ae | -ad | -ad | -ae | -af | -ag | ag | af | ae | ad |
| bj | bs | cb | ck | -cc | -bt | -bk | -bi | -br | -ca | -cj | cd | bu | bl | bh | bq | bz | ci | -ce | -bv | -bm | -bg | -bp | -by | -ch | cf | bw | bn | bf | bo | bx | cg |
| ar | aw | bb | -bd | -ay | -at | -ap | -au | -az | -be | ba | av | aq | as | ax | bc | -bc | -ax | -as | -aq | -av | -ba | be | az | au | ap | at | ay | bd | -bb | -aw | -ar |
| bk | bv | cg | -ce | -bt | -bi | -bm | -bx | -ci | cc | br | bg | bo | bz | ck | -ca | -bp | -bf | -bq | -cb | cj | by | bn | bh | bs | cd | -ch | -bw | -bl | -bj | -bu | -cf |
| ai | al | ao | -am | -aj | -ah | -ak | -an | an | ak | ah | aj | am | -ao | -al | -ai | -ai | -al | -ao | am | aj | ah | ak | an | -an | -ak | -ah | -aj | -am | ao | al | ai |
| bl | by | -ck | -bx | -bk | -bm | -bz | cj | bw | bj | bn | ca | -ci | -bv | -bi | -bo | -cb | ch | Bu | bh | bp | cc | -cg | -bt | -bg | -bq | -cd | cf | bs | bf | br | ce |
| as | az | -bd | -aw | -ap | -av | -bc | ba | at | ar | ay | -be | -ax | -aq | -au | -bb | bb | au | Aq | ax | be | -ay | -ar | -at | -ba | bc | av | ap | aw | bd | -az | -as |
| bm | cb | -cf | -bq | -bi | -bx | cj | bu | bf | bt | ci | -by | -bj | -bp | -ce | cc | bn | bl | Ca | -cg | -br | -bh | -bw | ck | bv | bg | bs | ch | -bz | -bk | -bo | -cd |
| ab | ac | -ac | -ab | -ab | -ac | ac | ab | ab | ac | -ac | -ab | -ab | -ac | ac | ab | ab | ac | -ac | -ab | -ab | -ac | ac | ab | ab | ac | -ac | -ab | -ab | -ac | ac | ab |
| bn | ce | -ca | -bj | -br | -ci | bw | bf | bv | -cj | -bs | -bi | -bz | cf | bo | bm | cd | -cb | -bk | -bq | -ch | bx | bg | bu | -ck | -bt | -bh | -by | cg | bp | bl | cc |
| at | bc | -ay | -ap | -ax | bd | au | as | bb | -az | -aq | -aw | be | av | ar | ba | -ba | -ar | -av | -be | aw | aq | az | -bb | -as | -au | -bd | ax | ap | ay | -bc | -at |
| bo | ch | -bv | -bh | -ca | cc | bj | bt | -cj | -bq | -bm | -cf | bx | bf | by | -ce | -bl | -br | -ck | bs | bk | cd | -bz | -bg | -bw | cg | bn | bp | ci | -bu | -bi | -cb |
| aj | ao | -ak | -ai | -an | al | ah | am | -am | -ah | -al | an | ai | ak | -ao | -aj | -aj | -ao | Ak | ai | an | -al | -ah | -am | am | ah | al | -an | -ai | -ak | ao | aj |
| bp | ck | -bq | -bo | -cj | br | bn | ci | -bs | -bm | -ch | bt | bl | cg | -bu | -bk | -cf | bv | Bj | ce | -bw | -bi | -cd | bx | bh | cc | -by | -bg | -cb | bz | bf | ca |
| au | -be | -at | -av | bd | as | aw | -bc | -ar | -ax | bb | aq | ay | -ba | -ap | -az | az | ap | Ba | -ay | -aq | -bb | ax | ar | bc | -aw | -as | -bd | av | at | be | -au |
| bq | -ci | -bl | -bv | cd | bg | ca | -by | -bi | -cf | bt | bn | ck | -bo | -bs | cg | bj | bx | -cb | -bf | -cc | bw | bk | ch | -br | -bp | cj | bm | bu | -ce | -bh | -bz |
| ae | -ag | -ad | -af | af | ad | ag | -ae | -ae | ag | ad | af | -af | -ad | -ag | ae | ae | -ag | -ad | -af | af | ad | ag | -ae | -ae | ag | ad | af | -af | -ad | -ag | ae |
| br | -cf | -bg | -cc | bu | bo | -ci | -bj | -bz | bx | bl | ck | -bm | -bw | ca | bi | ch | -bp | -bt | cd | bf | ce | -bs | -bq | cg | bh | cb | -bv | -bn | cj | bk | by |
| av | -bb | -ap | -bc | au | aw | -ba | -aq | -bd | at | ax | -az | -ar | -be | as | ay | -ay | -as | Be | ar | az | -ax | -at | bd | aq | ba | -aw | -au | bc | ap | bb | -av |
| bs | -cc | -bi | -cj | bl | bz | -bv | -bp | cf | bf | cg | -bo | -bw | by | bm | -ci | -bh | -cd | Br | bt | -cb | -bj | -ck | bk | ca | -bu | -bq | ce | bg | ch | -bn | -bx |
| ak | -am | -ai | ao | ah | an | -aj | -al | al | aj | -an | -ah | -ao | ai | am | -ak | -ak | am | Ai | -ao | -ah | -an | aj | al | -al | -aj | an | ah | ao | -ai | -am | ak |
| bt | -bz | -bn | cf | bh | ck | -bi | -ce | bo | by | -bu | -bs | ca | bm | -cg | -bg | -cj | bj | Cd | -bp | -bx | bv | br | -cb | -bl | ch | bf | ci | -bk | -cc | bq | bw |
| aw | -ay | -au | ba | as | -bc | -aq | be | ap | bd | -ar | -bb | at | az | -av | -ax | ax | av | -az | -at | bb | ar | -bd | -ap | -be | aq | bc | -as | -ba | au | ay | -aw |
| bu | -bw | -bs | by | bq | -ca | -bo | cc | bm | -ce | -bk | cg | bi | -ci | -bg | ck | bf | cj | -bh | -ch | bj | cf | -bl | -cd | bn | cb | -bp | -bz | br | bx | -bt | -bv |

The top right quadrant of the DCT2 64x64 matrix is of the form:

| aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | Aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa | aa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -ck | -cj | -ci | -ch | -cg | -cf | -ce | -cd | -cc | -cb | -ca | -bz | -by | -bx | -bw | -bv | -bu | -bt | -bs | -br | -bq | -bp | -bo | -bn | -bm | -bl | -bk | -bj | -bi | -bh | -bg | -bf |
| -ap | -aq | -ar | -as | -at | -au | -av | -aw | -ax | -ay | -az | -ba | -bb | -bc | -bd | -be | be | bd | Bc | bb | ba | az | ay | ax | aw | av | au | at | as | ar | aq | ap |
| cj | cg | cd | ca | bx | bu | br | bo | bl | bi | bf | bh | bk | bn | bq | bt | bw | bz | Cc | cf | ci | -ck | -ch | -ce | -cb | -by | -bv | -bs | -bp | -bm | -bj | -bg |
| ah | ai | aj | ak | al | am | an | ao | -ao | -an | -am | -al | -ak | -aj | -ai | -ah | -ah | -ai | -aj | -ak | -al | -am | -an | -ao | ao | an | am | al | ak | aj | ai | ah |
| -ci | -cd | -by | -bt | -bo | -bj | -bf | -bk | -bp | -bu | -bz | -ce | -cj | ch | cc | bx | bs | bn | Bi | bg | bl | bq | bv | ca | cf | ck | -cg | -cb | -bw | -br | -bm | -bh |
| -aq | -at | -aw | -az | -bc | be | bb | ay | av | as | ap | ar | au | ax | ba | bd | -bd | -ba | -ax | -au | -ar | -ap | -as | -av | -ay | -bb | -be | bc | az | aw | at | aq |
| ch | ca | bt | bm | bf | bl | bs | bz | cg | -ci | -cb | -bu | -bn | -bg | -bk | -br | -by | -cf | Cj | cc | bv | bo | bh | bj | bq | bx | ce | -ck | -cd | -bw | -bp | -bi |
| ad | ae | af | ag | -ag | -af | -ae | -ad | -ad | -ae | -af | -ag | ag | af | ae | ad | ad | ae | Af | ag | -ag | -af | -ae | -ad | -ad | -ae | -af | -ag | ag | af | ae | ad |
| -cg | -bx | -bo | -bf | -bn | -bw | -cf | ch | by | bp | bg | bm | bv | ce | -ci | -bz | -bq | -bh | -bl | -bu | -cd | cj | ca | br | bi | bk | bt | cc | -ck | -cb | -bs | -bj |
| -ar | -aw | -bb | bd | ay | at | ap | au | az | be | -ba | -av | -aq | -as | -ax | -bc | bc | ax | As | aq | av | ba | -be | -az | -au | -ap | -at | -ay | -bd | bb | aw | ar |
| cf | bu | bj | bl | bw | ch | -cd | -bs | -bh | -bn | -by | -cj | cb | bq | bf | bp | ca | -ck | -bz | -bo | -bg | -br | -cc | ci | bx | bm | bi | bt | ce | -cg | -bv | -bk |
| ai | al | ao | -am | -aj | -ah | -ak | -an | an | ak | ah | aj | am | -ao | -al | -ai | -ai | -al | -ao | am | aj | ah | ak | an | -an | -ak | -ah | -aj | -am | ao | al | ai |
| -ce | -br | -bf | -bs | -cf | cd | bq | bg | bt | cg | -cc | -bp | -bh | -bu | -ch | cb | bo | bi | Bv | ci | -ca | -bn | -bj | -bw | -cj | bz | bm | bk | bx | ck | -by | -bl |
| -as | -az | bd | aw | ap | av | bc | -ba | -at | -ar | -ay | be | ax | aq | au | bb | -bb | -au | -aq | -ax | -be | ay | ar | at | ba | -bc | -av | -ap | -aw | -bd | az | as |
| cd | bo | bk | bz | -ch | -bs | -bg | -bv | -ck | bw | bh | br | cg | -ca | -bl | -bn | -cc | ce | Bp | bj | by | -ci | -bt | -bf | -bu | -cj | bx | bi | bq | cf | -cb | -bm |
| ab | ac | -ac | -ab | -ab | -ac | ac | ab | ab | ac | -ac | -ab | -ab | -ac | ac | ab | ab | ac | -ac | -ab | -ab | -ac | ac | ab | ab | ac | -ac | -ab | -ab | -ac | ac | ab |
| -cc | -bl | -bp | -cg | by | bh | bt | ck | -bu | -bg | -bx | ch | bq | bk | cb | -cd | -bm | -bo | -cf | bz | bi | bs | cj | -bv | -bf | -bw | ci | br | bj | ca | -ce | -bn |
| -at | -bc | ay | ap | ax | -bd | -au | -as | -bb | az | aq | aw | -be | -av | -ar | -ba | ba | ar | Av | be | -aw | -aq | -az | bb | as | au | bd | -ax | -ap | -ay | bc | at |
| cb | bi | bu | -ci | -bp | -bn | -cg | bw | bg | bz | -cd | -bk | -bs | ck | br | bl | ce | -by | -bf | -bx | cf | bm | bq | cj | -bt | -bj | -cc | ca | bh | bv | -ch | -bo |
| aj | ao | -ak | -ai | -an | al | ah | am | -am | -ah | -al | an | ai | ak | -ao | -aj | -aj | -ao | Ak | ai | an | -al | -ah | -am | am | ah | al | -an | -ai | -ak | ao | aj |
| -ca | -bf | -bz | cb | bg | by | -cc | -bh | -bx | cd | bi | bw | -ce | -bj | -bv | cf | bk | bu | -cg | -bl | -bt | ch | bm | bs | -ci | -bn | -br | cj | bo | bq | -ck | -bp |
| -au | be | at | av | -bd | -as | -aw | bc | ar | ax | -bb | -aq | -ay | ba | ap | az | -az | -ap | -ba | ay | aq | bb | -ax | -ar | -bc | aw | as | bd | -av | -at | -be | au |
| bz | bh | ce | -bu | -bm | -cj | bp | br | -ch | -bk | -bw | cc | bf | cb | -bx | -bj | -cg | bs | Bo | -ck | -bn | -bt | cf | bi | by | -ca | -bg | -cd | bv | bl | ci | -bq |
| ae | -ag | -ad | -af | af | ad | ag | -ae | -ae | ag | ad | af | -af | -ad | -ag | ae | ae | -ag | -ad | -af | af | ad | ag | -ae | -ae | ag | ad | af | -af | -ad | -ag | ae |
| -by | -bk | -cj | bn | bv | -cb | -bh | -cg | bq | bs | -ce | -bf | -cd | bt | bp | -ch | -bi | -ca | Bw | bm | -ck | -bl | -bx | bz | bj | ci | -bo | -bu | cc | bg | cf | -br |
| -av | bb | ap | bc | -au | -aw | ba | aq | bd | -at | -ax | az | ar | be | -as | -ay | ay | as | -be | -ar | -az | ax | at | -bd | -aq | -ba | aw | au | -bc | -ap | -bb | av |
| bx | bn | -ch | -bg | -ce | bq | bu | -ca | -bk | ck | bj | cb | -bt | -br | cd | bh | ci | -bm | -by | bw | bo | -cg | -bf | -cf | bp | bv | -bz | -bl | cj | bi | cc | -bs |
| ak | -am | -ai | ao | ah | an | -aj | -al | al | aj | -an | -ah | -ao | ai | am | -ak | -ak | am | Ai | -ao | -ah | -an | aj | al | -al | -aj | an | ah | ao | -ai | -am | ak |
| -bw | -bq | cc | bk | -ci | -bf | -ch | bl | cb | -br | -bv | bx | bp | -cd | -bj | cj | bg | cg | -bm | -ca | bs | bu | -by | -bo | ce | bi | -ck | -bh | -cf | bn | bz | -bt |
| -aw | ay | au | -ba | -as | bc | aq | -be | -ap | -bd | ar | bb | -at | -az | av | ax | -ax | -av | Az | at | -bb | -ar | bd | ap | be | -aq | -bc | as | ba | -au | -ay | aw |
| bv | bt | -bx | -br | bz | bp | -cb | -bn | cd | bl | -cf | -bj | ch | bh | -cj | -bf | -ck | bg | Ci | -bi | -cg | bk | ce | -bm | -cc | bo | ca | -bq | -by | bs | bw | -bu |

The bottom left quadrant of the DCT2 64x64 matrix is of the form:

| aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| bv | -bt | -bx | br | bz | -bp | -cb | bn | cd | -bl | -cf | bj | ch | -bh | -cj | bf | -ck | -bg | Ci | bi | -cg | -bk | ce | bm | -cc | -bo | ca | bq | -by | -bs | bw | bu |
| ax | -av | -az | at | bb | -ar | -bd | ap | -be | -aq | bc | as | -ba | -au | ay | aw | -aw | -ay | au | ba | -as | -bc | aq | be | -ap | bd | ar | -bb | -at | az | av | -ax |
| bw | -bq | -cc | bk | ci | -bf | ch | bl | -cb | -br | bv | bx | -bp | -cd | bj | cj | -bg | cg | bm | -ca | -bs | bu | by | -bo | -ce | bi | ck | -bh | cf | bn | -bz | -bt |
| al | -aj | -an | ah | -ao | -ai | am | ak | -ak | -am | ai | ao | -ah | an | aj | -al | -al | aj | an | -ah | ao | ai | -am | -ak | ak | am | -ai | -ao | ah | -an | -aj | al |
| bx | -bn | -ch | bg | -ce | -bq | bu | ca | -bk | -ck | bj | -cb | -bt | br | cd | -bh | ci | bm | -by | -bw | bo | cg | -bf | cf | bp | -bv | -bz | bl | cj | -bi | cc | bs |
| ay | -as | -be | ar | -az | -ax | at | bd | -aq | ba | aw | -au | -bc | ap | -bb | -av | av | bb | -ap | bc | au | -aw | -ba | aq | -bd | -at | ax | az | -ar | be | as | -ay |
| by | -bk | cj | bn | -bv | -cb | bh | -cg | -bq | bs | ce | -bf | cd | bt | -bp | -ch | bi | -ca | -bw | bm | ck | -bl | bx | bz | -bj | ci | bo | -bu | -cc | bg | -cf | -br |
| af | -ad | ag | ae | -ae | -ag | ad | -af | -af | ad | -ag | -ae | ae | ag | -ad | af | af | -ad | ag | ae | -ae | -ag | ad | -af | -af | ad | -ag | -ae | ae | ag | -ad | af |
| bz | -bh | ce | bu | -bm | cj | bp | -br | -ch | bk | -bw | -cc | bf | -cb | -bx | bj | -cg | -bs | bo | ck | -bn | bt | cf | -bi | by | ca | -bg | cd | bv | -bl | ci | bq |
| az | -ap | ba | ay | -aq | bb | ax | -ar | bc | aw | -as | bd | av | -at | be | au | -au | -be | at | -av | -bd | as | -aw | -bc | ar | -ax | -bb | aq | -ay | -ba | ap | -az |
| ca | -bf | bz | cb | -bg | by | cc | -bh | bx | cd | -bi | bw | ce | -bj | bv | cf | -bk | bu | cg | -bl | bt | ch | -bm | bs | ci | -bn | br | cj | -bo | bq | ck | -bp |
| am | -ah | al | an | -ai | ak | ao | -aj | aj | -ao | -ak | ai | -an | -al | ah | -am | -am | ah | -al | -an | ai | -ak | -ao | aj | -aj | ao | ak | -ai | an | al | -ah | am |
| cb | -bi | bu | ci | -bp | bn | -cg | -bw | bg | -bz | -cd | bk | -bs | -ck | br | -bl | ce | by | -bf | bx | cf | -bm | bq | -cj | -bt | bj | -cc | -ca | bh | -bv | -ch | bo |
| ba | -ar | av | -be | -aw | aq | -az | -bb | as | -au | bd | ax | -ap | ay | bc | -at | at | -bc | -ay | ap | -ax | -bd | au | -as | bb | az | -aq | aw | be | -av | ar | -ba |
| cc | -bl | bp | -cg | -by | bh | -bt | ck | bu | -bg | bx | ch | -bq | bk | -cb | -cd | bm | -bo | cf | bz | -bi | bs | -cj | -bv | bf | -bw | -ci | br | -bj | ca | ce | -bn |
| ac | -ab | ab | -ac | -ac | ab | -ab | ac | ac | -ab | ab | -ac | -ac | ab | -ab | ac | ac | -ab | ab | -ac | -ac | ab | -ab | ac | ac | -ab | ab | -ac | -ac | ab | -ab | ac |
| cd | -bo | bk | -bz | -ch | bs | -bg | bv | -ck | -bw | bh | -br | cg | ca | -bl | bn | -cc | -ce | bp | -bj | by | ci | -bt | bf | -bu | cj | bx | -bi | bq | -cf | -cb | bm |
| bb | -au | aq | -ax | be | ay | -ar | at | -ba | -bc | av | -ap | aw | -bd | -az | as | -as | az | bd | -aw | ap | -av | bc | ba | -at | ar | -ay | -be | ax | -aq | au | -bb |
| ce | -br | bf | -bs | cf | cd | -bq | bg | -bt | cg | cc | -bp | bh | -bu | ch | cb | -bo | bi | -bv | ci | ca | -bn | bj | -bw | cj | bz | -bm | bk | -bx | ck | by | -bl |
| an | -ak | ah | -aj | am | ao | -al | ai | -ai | al | -ao | -am | aj | -ah | ak | -an | -an | ak | -ah | aj | -am | -ao | al | -ai | ai | -al | ao | am | -aj | ah | -ak | an |
| cf | -bu | bj | -bl | bw | -ch | -cd | bs | -bh | bn | -by | cj | cb | -bq | bf | -bp | ca | ck | -bz | bo | -bg | br | -cc | -ci | bx | -bm | bi | -bt | ce | cg | -bv | bk |
| bc | -ax | as | -aq | av | -ba | -be | az | -au | ap | -at | ay | -bd | -bb | aw | -ar | ar | -aw | bb | bd | -ay | at | -ap | au | -az | be | ba | -av | aq | -as | ax | -bc |
| cg | -bx | bo | -bf | bn | -bw | cf | ch | -by | bp | -bg | bm | -bv | ce | ci | -bz | bq | -bh | bl | -bu | cd | cj | -ca | br | -bi | bk | -bt | cc | ck | -cb | bs | -bj |
| ag | -af | ae | -ad | ad | -ae | af | -ag | -ag | af | -ae | ad | -ad | ae | -af | ag | ag | -af | ae | -ad | ad | -ae | af | -ag | -ag | af | -ae | ad | -ad | ae | -af | ag |
| ch | -ca | bt | -bm | bf | -bl | bs | -bz | cg | ci | -cb | bu | -bn | bg | -bk | br | -by | cf | cj | -cc | bv | -bo | bh | -bj | bq | -bx | ce | ck | -cd | bw | -bp | bi |
| bd | -ba | ax | -au | ar | -ap | as | -av | ay | -bb | be | bc | -az | aw | -at | aq | -aq | at | -aw | az | -bc | -be | bb | -ay | av | -as | ap | -ar | au | -ax | ba | -bd |
| ci | -cd | by | -bt | bo | -bj | bf | -bk | bp | -bu | bz | -ce | cj | ch | -cc | bx | -bs | bn | -bi | bg | -bl | bq | -bv | ca | -cf | ck | cg | -cb | bw | -br | bm | -bh |
| ao | -an | am | -al | ak | -aj | ai | -ah | ah | -ai | aj | -ak | al | -am | an | -ao | -ao | an | -am | al | -ak | aj | -ai | ah | -ah | ai | -aj | ak | -al | am | -an | ao |
| cj | -cg | cd | -ca | bx | -bu | br | -bo | bl | -bi | bf | -bh | bk | -bn | bq | -bt | bw | -bz | cc | -cf | ci | ck | -ch | ce | -cb | by | -bv | bs | -bp | bm | -bj | bg |
| be | -bd | bc | -bb | ba | -az | ay | -ax | aw | -av | au | -at | as | -ar | aq | -ap | ap | -aq | ar | -as | at | -au | av | -aw | ax | -ay | az | -ba | bb | -bc | bd | -be |
| ck | -cj | ci | -ch | cg | -cf | ce | -cd | cc | -cb | ca | -bz | by | -bx | bw | -bv | bu | -bt | bs | -br | bq | -bp | bo | -bn | bm | -bl | bk | -bj | bi | -bh | bg | -bf |

The bottom right quadrant of the DCT2 64x64 matrix is of the form:

| aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | Aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa | aa | -aa | -aa | aa |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| -bu | -bw | bs | by | -bq | -ca | bo | cc | -bm | -ce | bk | cg | -bi | -ci | bg | ck | -bf | cj | bh | -ch | -bj | cf | bl | -cd | -bn | cb | bp | -bz | -br | bx | bt | -bv |
| -ax | av | az | -at | -bb | ar | bd | -ap | be | aq | -bc | -as | ba | au | -ay | -aw | aw | ay | -au | -ba | as | bc | -aq | -be | ap | -bd | -ar | bb | at | -az | -av | ax |
| bt | bz | -bn | -cf | bh | -ck | -bi | ce | bo | -by | -bu | bs | ca | -bm | -cg | bg | -cj | -bj | cd | Bp | -bx | -bv | br | cb | -bl | -ch | bf | -ci | -bk | cc | bq | -bw |
| al | -aj | -an | ah | -ao | -ai | am | ak | -ak | -am | ai | ao | -ah | an | aj | -al | -al | aj | an | -ah | ao | ai | -am | -ak | ak | am | -ai | -ao | ah | -an | -aj | al |
| -bs | -cc | bi | -cj | -bl | bz | bv | -bp | -cf | bf | -cg | -bo | bw | by | -bm | -ci | bh | -cd | -br | bt | cb | -bj | ck | bk | -ca | -bu | bq | ce | -bg | ch | bn | -bx |
| -ay | as | be | -ar | az | ax | -at | -bd | aq | -ba | -aw | au | bc | -ap | bb | av | -av | -bb | ap | -bc | -au | aw | ba | -aq | bd | at | -ax | -az | ar | -be | -as | ay |
| br | cf | -bg | cc | bu | -bo | -ci | bj | -bz | -bx | bl | -ck | -bm | bw | ca | -bi | ch | bp | -bt | -cd | bf | -ce | -bs | bq | cg | -bh | cb | bv | -bn | -cj | bk | -by |
| af | -ad | ag | ae | -ae | -ag | ad | -af | -af | ad | -ag | -ae | ae | ag | -ad | af | af | -ad | ag | ae | -ae | -ag | ad | -af | -af | ad | -ag | -ae | ae | ag | -ad | af |
| -bq | -ci | bl | -bv | -cd | bg | -ca | -by | bi | -cf | -bt | bn | -ck | -bo | bs | cg | -bj | bx | cb | -bf | cc | bw | -bk | ch | br | -bp | -cj | bm | -bu | -ce | bh | -bz |
| -az | ap | -ba | -ay | aq | -bb | -ax | ar | -bc | -aw | as | -bd | -av | at | -be | -au | au | be | -at | av | bd | -as | aw | bc | -ar | ax | bb | -aq | ay | ba | -ap | az |
| bp | -ck | -bq | bo | -cj | -br | bn | -ci | -bs | bm | -ch | -bt | bl | -cg | -bu | bk | -cf | -bv | bj | -ce | -bw | bi | -cd | -bx | bh | -cc | -by | bg | -cb | -bz | bf | -ca |
| am | -ah | al | an | -ai | ak | ao | -aj | aj | -ao | -ak | ai | -an | -al | ah | -am | -am | ah | -al | -an | ai | -ak | -ao | aj | -aj | ao | ak | -ai | an | al | -ah | am |
| -bo | ch | bv | -bh | ca | cc | -bj | bt | cj | -bq | bm | -cf | -bx | bf | -by | -ce | bl | -br | ck | bs | -bk | cd | bz | -bg | bw | cg | -bn | bp | -ci | -bu | bi | -cb |
| -ba | ar | -av | be | aw | -aq | az | bb | -as | au | -bd | -ax | ap | -ay | -bc | at | -at | bc | ay | -ap | ax | bd | -au | as | -bb | -az | aq | -aw | -be | av | -ar | ba |
| bn | -ce | -ca | bj | -br | ci | bw | -bf | bv | cj | -bs | bi | -bz | -cf | bo | -bm | cd | cb | -bk | bq | -ch | -bx | bg | -bu | -ck | bt | -bh | by | cg | -bp | bl | -cc |
| ac | -ab | ab | -ac | -ac | ab | -ab | ac | ac | -ab | ab | -ac | -ac | ab | -ab | ac | ac | -ab | ab | -ac | -ac | ab | -ab | ac | ac | -ab | ab | -ac | -ac | ab | -ab | ac |
| -bm | cb | cf | -bq | bi | -bx | -cj | bu | -bf | bt | -ci | -by | bj | -bp | ce | cc | -bn | bl | -ca | -cg | br | -bh | bw | ck | -bv | bg | -bs | ch | bz | -bk | bo | -cd |
| -bb | au | -aq | ax | -be | -ay | ar | -at | ba | bc | -av | ap | -aw | bd | az | -as | as | -az | -bd | aw | -ap | av | -bc | -ba | at | -ar | ay | be | -ax | aq | -au | bb |
| bl | -by | -ck | bx | -bk | bm | -bz | -cj | bw | -bj | bn | -ca | -ci | bv | -bi | bo | -cb | -ch | bu | -bh | bp | -cc | -cg | bt | -bg | bq | -cd | -cf | bs | -bf | br | -ce |
| an | -ak | ah | -aj | am | ao | -al | ai | -ai | al | -ao | -am | aj | -ah | ak | -an | -an | ak | -ah | aj | -am | -ao | al | -ai | ai | -al | ao | am | -aj | ah | -ak | an |
| -bk | bv | -cg | -ce | bt | -bi | bm | -bx | ci | cc | -br | bg | -bo | bz | -ck | -ca | bp | -bf | bq | -cb | -cj | by | -bn | bh | -bs | cd | ch | -bw | bl | -bj | bu | -cf |
| -bc | ax | -as | aq | -av | ba | be | -az | au | -ap | at | -ay | bd | bb | -aw | ar | -ar | aw | -bb | -bd | ay | -at | ap | -au | az | -be | -ba | av | -aq | as | -ax | bc |
| bj | -bs | cb | -ck | -cc | bt | -bk | bi | -br | ca | -cj | -cd | bu | -bl | bh | -bq | bz | -ci | -ce | bv | -bm | bg | -bp | by | -ch | -cf | bw | -bn | bf | -bo | bx | -cg |
| ag | -af | ae | -ad | ad | -ae | af | -ag | -ag | af | -ae | ad | -ad | ae | -af | ag | ag | -af | ae | -ad | ad | -ae | af | -ag | -ag | af | -ae | ad | -ad | ae | -af | ag |
| -bi | bp | -bw | cd | -ck | -ce | bx | -bq | bj | -bh | bo | -bv | cc | -cj | -cf | by | -br | bk | -bg | bn | -bu | cb | -ci | -cg | bz | -bs | bl | -bf | bm | -bt | ca | -ch |
| -bd | ba | -ax | au | -ar | ap | -as | av | -ay | bb | -be | -bc | az | -aw | at | -aq | aq | -at | aw | -az | bc | be | -bb | ay | -av | as | -ap | ar | -au | ax | -ba | bd |
| bh | -bm | br | -bw | cb | -cg | -ck | cf | -ca | bv | -bq | bl | -bg | bi | -bn | bs | -bx | cc | -ch | -cj | ce | -bz | bu | -bp | bk | -bf | bj | -bo | bt | -by | cd | -ci |
| ao | -an | am | -al | ak | -aj | ai | -ah | ah | -ai | aj | -ak | al | -am | an | -ao | -ao | an | -am | al | -ak | aj | -ai | ah | -ah | ai | -aj | ak | -al | am | -an | ao |
| -bg | bj | -bm | bp | -bs | bv | -by | cb | -ce | ch | -ck | -ci | cf | -cc | bz | -bw | bt | -bq | bn | -bk | bh | -bf | bi | -bl | bo | -br | bu | -bx | ca | -cd | cg | -cj |
| -be | bd | -bc | bb | -ba | az | -ay | ax | -aw | av | -au | at | -as | ar | -aq | ap | -ap | aq | -ar | as | -at | au | -av | aw | -ax | ay | -az | ba | -bb | bc | -bd | be |
| bf | -bg | bh | -bi | bj | -bk | bl | -bm | bn | -bo | bp | -bq | br | -bs | bt | -bu | bv | -bw | bx | -by | B | -ca | cb | -cc | cd | -ce | cf | -cg | ch | -ci | cj | -ck |

The values of the DCT2 transform matrix above are defined below.  Each value may have two names, one for matrices up to 32x32, and an alternative for 64x64 matrices.  6 bit VVC standard matrix values are included for reference).

| Name | Alternate | 14 bit | 6 bit |  | Name | Alternate | 14 bit | 6 bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a | aa | 16384 | 64 |  |  | bf | 23360 | 91 |
| b | ab | 21266 | 83 |  |  | bg | 23053 | 90 |
| c | ac | 9224 | 36 |  |  | bh | 23048 | 90 |
| d | ad | 22813 | 89 |  |  | bi | 23023 | 90 |
| e | ae | 19244 | 75 |  |  | bj | 22610 | 88 |
| f | af | 12769 | 50 |  |  | bk | 22339 | 87 |
| g | ag | 4563 | 18 |  |  | bl | 21936 | 86 |
| h | ah | 23129 | 90 |  |  | bm | 21502 | 84 |
| i | ai | 22063 | 87 |  |  | bn | 21266 | 83 |
| j | aj | 20450 | 80 |  |  | bo | 20730 | 81 |
| k | ak | 17972 | 70 |  |  | bp | 20251 | 79 |
| l | al | 14642 | 57 |  |  | bq | 19726 | 77 |
| m | am | 11109 | 43 |  |  | br | 18731 | 73 |
| n | an | 6446 | 25 |  |  | bs | 18201 | 71 |
| o | ao | 2316 | 9 |  |  | bt | 17638 | 69 |
| p | ap | 23106 | 90 |  |  | bu | 16604 | 65 |
| q | aq | 22852 | 90 |  |  | bv | 15881 | 62 |
| r | ar | 22445 | 88 |  |  | bw | 15084 | 59 |
| s | as | 21848 | 85 |  |  | bx | 14322 | 56 |
| t | at | 20995 | 82 |  |  | by | 13340 | 52 |
| u | au | 19810 | 78 |  |  | bz | 12238 | 48 |
| v | av | 18601 | 73 |  |  | ca | 11330 | 44 |
| w | aw | 17143 | 67 |  |  | cb | 10493 | 41 |
| x | ax | 15718 | 61 |  |  | cc | 9428 | 37 |
| y | ay | 13853 | 54 |  |  | cd | 8426 | 33 |
| z | az | 11749 | 46 |  |  | ce | 7100 | 28 |
| A | ba | 9846 | 38 |  |  | cf | 6151 | 24 |
| B | bb | 7908 | 31 |  |  | cg | 5101 | 20 |
| C | bc | 5573 | 22 |  |  | ch | 3848 | 15 |
| D | bd | 3281 | 13 |  |  | ci | 2734 | 11 |
| E | be | 946 | 4 |  |  | cj | 1754 | 7 |
|  |  |  |  |  |  | ck | 574 | 2 |

2x2 DCT2

The matrix M2 is defined as the first two coefficients of every 32th row in the combined matrix M64.

| a | a |
| --- | --- |
| a | -a |

4x4 DCT2

The matrix M4 is defined as the first four coefficients of every 16th row in the combined matrix M64.

| a | a | a | a |
| --- | --- | --- | --- |
| b | c | -c | -b |
| a | -a | -a | a |
| c | -b | b | -c |

8x8 DCT2

The matrix M8 is defined as the first 8 coefficients of every 8th row in the combined matrix M64.

| a | a | a | a | a | a | a | a |
| --- | --- | --- | --- | --- | --- | --- | --- |
| d | e | f | g | -g | -f | -e | -d |
| b | c | -c | -b | -b | -c | c | b |
| e | -g | -d | -f | f | d | g | -e |
| a | -a | -a | a | a | -a | -a | a |
| f | -d | g | e | -e | -g | d | -f |
| c | -b | b | -c | -c | b | -b | c |
| g | -f | e | -d | d | -e | f | -g |

16x16 DCT2

The matrix M16 is defined as the first 16 coefficients of every 4th row in the combined matrix M64.

| a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| h | i | j | k | l | m | n | o | -o | -n | -m | -l | -k | -j | -i | -h |
| d | e | f | g | -g | -f | -e | -d | -d | -e | -f | -g | g | f | e | d |
| i | l | o | -m | -j | -h | -k | -n | n | k | h | j | m | -o | -l | -i |
| b | c | -c | -b | -b | -c | c | b | b | c | -c | -b | -b | -c | c | b |
| j | o | -k | -i | -n | l | h | m | -m | -h | -l | n | i | k | -o | -j |
| e | -g | -d | -f | f | d | g | -e | -e | g | d | f | -f | -d | -g | e |
| k | -m | -i | o | h | n | -j | -l | l | j | -n | -h | -o | i | m | -k |
| a | -a | -a | a | a | -a | -a | a | a | -a | -a | a | a | -a | -a | a |
| l | -j | -n | h | -o | -i | m | k | -k | -m | i | o | -h | n | j | -l |
| f | -d | g | e | -e | -g | d | -f | -f | d | -g | -e | e | g | -d | f |
| m | -h | l | n | -i | k | o | -j | j | -o | -k | i | -n | -l | h | -m |
| c | -b | b | -c | -c | b | -b | c | c | -b | b | -c | -c | b | -b | c |
| n | -k | h | -j | m | o | -l | i | -i | l | -o | -m | j | -h | k | -n |
| g | -f | e | -d | d | -e | f | -g | -g | f | -e | d | -d | e | -f | g |
| o | -n | m | -l | k | -j | i | -h | h | -i | j | -k | l | -m | n | -o |

32x32DCT2

The matrix M32 is defined as the first 32 coefficients of every 2nd row in the combined matrix M64.

| a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a | a |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| p | q | r | s | t | u | v | w | x | y | z | A | B | C | D | E | -E | -D | -C | -B | -A | -z | -y | -x | -w | -v | -u | -t | -s | -r | -q | -p |
| h | i | j | k | l | m | n | o | -o | -n | -m | -l | -k | -j | -i | -h | -h | -i | -j | -k | -l | -m | -n | -o | o | n | m | l | k | j | i | h |
| q | t | w | z | C | -E | -B | -y | -v | -s | -p | -r | -u | -x | -A | -D | D | A | x | u | r | p | s | v | y | B | E | -C | -z | -w | -t | -q |
| d | e | f | g | -g | -f | -e | -d | -d | -e | -f | -g | g | f | e | d | d | e | f | g | -g | -f | -e | -d | -d | -e | -f | -g | g | f | e | d |
| r | w | B | -D | -y | -t | -p | -u | -z | -E | A | v | q | s | x | C | -C | -x | -s | -q | -v | -A | E | z | u | p | t | y | D | -B | -w | -r |
| i | l | o | -m | -j | -h | -k | -n | n | k | h | j | m | -o | -l | -i | -i | -l | -o | m | j | h | k | n | -n | -k | -h | -j | -m | o | l | i |
| s | z | -D | -w | -p | -v | -C | A | t | r | y | -E | -x | -q | -u | -B | B | u | q | x | E | -y | -r | -t | -A | C | v | p | w | D | -z | -s |
| b | c | -c | -b | -b | -c | c | b | b | c | -c | -b | -b | -c | c | b | b | c | -c | -b | -b | -c | c | b | b | c | -c | -b | -b | -c | c | b |
| t | C | -y | -p | -x | D | u | s | B | -z | -q | -w | E | v | r | A | -A | -r | -v | -E | w | q | z | -B | -s | -u | -D | x | p | y | -C | -t |
| j | o | -k | -i | -n | l | h | m | -m | -h | -l | n | i | k | -o | -j | -j | -o | k | i | n | -l | -h | -m | m | h | l | -n | -i | -k | o | j |
| u | -E | -t | -v | D | s | w | -C | -r | -x | B | q | y | -A | -p | -z | z | p | A | -y | -q | -B | x | r | C | -w | -s | -D | v | t | E | -u |
| e | -g | -d | -f | f | d | g | -e | -e | g | d | f | -f | -d | -g | e | e | -g | -d | -f | f | d | g | -e | -e | g | d | f | -f | -d | -g | e |
| v | -B | -p | -C | u | w | -A | -q | -D | t | x | -z | -r | -E | s | y | -y | -s | E | r | z | -x | -t | D | q | A | -w | -u | C | p | B | -v |
| k | -m | -i | o | h | n | -j | -l | l | j | -n | -h | -o | i | m | -k | -k | m | i | -o | -h | -n | j | l | -l | -j | n | h | o | -i | -m | k |
| w | -y | -u | A | s | -C | -q | E | p | D | -r | -B | t | z | -v | -x | x | v | -z | -t | B | r | -D | -p | -E | q | C | -s | -A | u | y | -w |
| a | -a | -a | a | a | -a | -a | a | a | -a | -a | a | a | -a | -a | a | a | -a | -a | a | a | -a | -a | a | a | -a | -a | a | a | -a | -a | a |
| x | -v | -z | t | B | -r | -D | p | -E | -q | C | s | -A | -u | y | w | -w | -y | u | A | -s | -C | q | E | -p | D | r | -B | -t | z | v | -x |
| l | -j | -n | h | -o | -i | m | k | -k | -m | i | o | -h | n | j | -l | -l | j | n | -h | o | i | -m | -k | k | m | -i | -o | h | -n | -j | l |
| y | -s | -E | r | -z | -x | t | D | -q | A | w | -u | -C | p | -B | -v | v | B | -p | C | u | -w | -A | q | -D | -t | x | z | -r | E | s | -y |
| f | -d | g | e | -e | -g | d | -f | -f | d | -g | -e | e | g | -d | f | f | -d | g | e | -e | -g | d | -f | -f | d | -g | -e | e | g | -d | f |
| z | -p | A | y | -q | B | x | -r | C | w | -s | D | v | -t | E | u | -u | -E | t | -v | -D | s | -w | -C | r | -x | -B | q | -y | -A | p | -z |
| m | -h | l | n | -i | k | o | -j | j | -o | -k | i | -n | -l | h | -m | -m | h | -l | -n | i | -k | -o | j | -j | o | k | -i | n | l | -h | m |
| A | -r | v | -E | -w | q | -z | -B | s | -u | D | x | -p | y | C | -t | t | -C | -y | p | -x | -D | u | -s | B | z | -q | w | E | -v | r | -A |
| c | -b | b | -c | -c | b | -b | c | c | -b | b | -c | -c | b | -b | c | c | -b | b | -c | -c | b | -b | c | c | -b | b | -c | -c | b | -b | c |
| B | -u | q | -x | E | y | -r | t | -A | -C | v | -p | w | -D | -z | s | -s | z | D | -w | p | -v | C | A | -t | r | -y | -E | x | -q | u | -B |
| n | -k | h | -j | m | o | -l | i | -i | l | -o | -m | j | -h | k | -n | -n | k | -h | j | -m | -o | l | -i | i | -l | o | m | -j | h | -k | n |
| C | -x | s | -q | v | -A | -E | z | -u | p | -t | y | -D | -B | w | -r | r | -w | B | D | -y | t | -p | u | -z | E | A | -v | q | -s | x | -C |
| g | -f | e | -d | d | -e | f | -g | -g | f | -e | d | -d | e | -f | g | g | -f | e | -d | d | -e | f | -g | -g | f | -e | d | -d | e | -f | g |
| D | -A | x | -u | r | -p | s | -v | y | -B | E | C | -z | w | -t | q | -q | t | -w | z | -C | -E | B | -y | v | -s | p | -r | u | -x | A | -D |
| o | -n | m | -l | k | -j | i | -h | h | -i | j | -k | l | -m | n | -o | -o | n | -m | l | -k | j | -i | h | -h | i | -j | k | -l | m | -n | o |
| E | -D | C | -B | A | -z | y | -x | w | -v | u | -t | s | -r | q | -p | p | -q | r | -s | t | -u | v | -w | x | -y | z | -A | B | -C | D | -E |

DCT2 64x64

The 64x64 DCT2 uses the full matrix M64.

DCT8

The table below contains values for the DCT8 matrices that follow.  6 bit VVC standard matrix values are included for reference.

| Label | 4x4 |  | 8x8 |  | 16x16 |  | 32x32 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14 bit | 6 bit | 14 bit | 6 bit | 14 bit | 6 bit | 14 bit | 6 bit |
| a | 21505 | 84 | 22018 | 86 | 22569 | 88 | 23065 | 90 |
| b | 18893 | 74 | 21790 | 85 | 22542 | 88 | 23136 | 90 |
| c | 14081 | 55 | 19958 | 78 | 22202 | 87 | 22715 | 89 |
| d | 7425 | 29 | 18154 | 71 | 21664 | 85 | 22533 | 88 |
| e |  |  | 15363 | 60 | 20754 | 81 | 22544 | 87 |
| f |  |  | 11754 | 46 | 19738 | 77 | 22053 | 86 |
| g |  |  | 8148 | 32 | 18787 | 73 | 21901 | 85 |
| h |  |  | 4350 | 17 | 17369 | 68 | 21463 | 84 |
| i |  |  |  |  | 15781 | 62 | 21131 | 82 |
| j |  |  |  |  | 14044 | 55 | 20385 | 80 |
| k |  |  |  |  | 12209 | 48 | 20019 | 78 |
| l |  |  |  |  | 10360 | 40 | 19708 | 77 |
| m |  |  |  |  | 8498 | 33 | 19007 | 74 |
| n |  |  |  |  | 6421 | 25 | 18415 | 72 |
| o |  |  |  |  | 4295 | 17 | 17448 | 68 |
| p |  |  |  |  | 1967 | 8 | 16894 | 66 |
| q |  |  |  |  |  |  | 16143 | 63 |
| r |  |  |  |  |  |  | 15230 | 60 |
| s |  |  |  |  |  |  | 14312 | 56 |
| t |  |  |  |  |  |  | 13616 | 53 |
| u |  |  |  |  |  |  | 12679 | 50 |
| v |  |  |  |  |  |  | 11526 | 46 |
| w |  |  |  |  |  |  | 10770 | 42 |
| x |  |  |  |  |  |  | 9720 | 38 |
| y |  |  |  |  |  |  | 8606 | 34 |
| z |  |  |  |  |  |  | 7734 | 30 |
| A |  |  |  |  |  |  | 6623 | 26 |
| B |  |  |  |  |  |  | 5414 | 21 |
| C |  |  |  |  |  |  | 4478 | 17 |
| D |  |  |  |  |  |  | 3225 | 13 |
| E |  |  |  |  |  |  | 2291 | 9 |
| F |  |  |  |  |  |  | 1043 | 4 |

4x4 DCT8

The 4x4 DCT8 transform matrix is of the form below and uses values from the table above.

| a | b | c | d |
| --- | --- | --- | --- |
| b | 0 | -b | -b |
| c | -b | -d | a |
| d | -b | a | -c |

8x8 DCT8

The 8x8 DCT8 transform matrix is of the form below and uses values from the table above.

| A | b | c | d | e | f | g | H |
| --- | --- | --- | --- | --- | --- | --- | --- |
| B | e | h | -g | -d | -a | -c | -f |
| C | h | -e | -a | -f | g | b | D |
| D | -g | -a | -h | c | e | -f | -b |
| E | -d | -f | c | g | -b | -h | A |
| F | -a | g | e | -b | h | d | -c |
| G | -c | b | -f | -h | d | -a | e |
| H | -f | d | -b | a | -c | e | -g |

16x16 DCT8

The 16x16 DCT8 transform matrix is of the form below and uses values from the table above.

| A | B | c | d | e | f | g | h | i | j | k | l | m | n | o | p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B | E | h | k | n | 0 | -n | -k | -h | -e | -b | -b | -e | -h | -k | -n |
| C | H | m | -p | -k | -f | -a | -e | -j | -o | n | i | d | b | g | l |
| d | K | -p | -i | -b | -f | -m | n | g | a | h | o | -l | -e | -c | -j |
| e | N | -k | -b | -h | 0 | h | b | k | -n | -e | -e | -n | k | b | h |
| f | 0 | -f | -f | 0 | f | f | 0 | -f | -f | 0 | f | f | 0 | -f | -f |
| g | -n | -a | -m | h | f | -o | -b | -l | i | e | -p | -c | -k | j | d |
| h | -k | -e | n | b | 0 | -b | -n | e | k | -h | -h | k | e | -n | -b |
| i | -h | -j | g | k | -f | -l | e | m | -d | -n | c | o | -b | -p | a |
| j | -e | -o | a | -n | -f | i | k | -d | -p | b | -m | -g | h | l | -c |
| k | -b | n | h | -e | 0 | e | -h | -n | b | -k | -k | b | -n | -h | e |
| l | -b | i | o | -e | f | -p | -h | c | -m | -k | a | -j | -n | d | -g |
| m | -e | d | -l | -n | f | -c | k | o | -g | b | -j | -p | h | -a | i |
| n | -h | b | -e | k | 0 | -k | e | -b | h | -n | -n | h | -b | e | -k |
| o | -k | g | -c | b | -f | j | -n | -p | l | -h | d | -a | e | -i | m |
| p | -n | l | -j | h | -f | d | -b | a | -c | e | -g | i | -k | m | -o |

32x32 DCT8

The 32x32 DCT8 transform matrix is of the form below and uses values from the table above.

| a | b | c | d | e | f | g | h | i | j | k | l | m | n | o | p | q | r | s | t | u | v | w | x | y | z | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| b | e | h | k | n | q | t | w | z | C | F | -E | -B | -y | -v | -s | -p | -m | -j | -g | -d | -a | -c | -f | -i | -l | -o | -r | -u | -x | -A | -D |
| c | h | m | r | w | B | 0 | -B | -w | -r | -m | -h | -c | -c | -h | -m | -r | -w | -B | 0 | B | w | r | m | h | c | c | h | m | r | w | B |
| d | k | r | y | F | -A | -t | -m | -f | -b | -i | -p | -w | -D | C | v | o | h | a | g | n | u | B | -E | -x | -q | -j | -c | -e | -l | -s | -z |
| e | n | w | F | -y | -p | -g | -c | -l | -u | -D | A | r | i | a | j | s | B | -C | -t | -k | -b | -h | -q | -z | E | v | m | d | f | o | x |
| f | q | B | -A | -p | -e | -g | -r | -C | z | o | d | h | s | D | -y | -n | -c | -i | -t | -E | x | m | b | j | u | F | -w | -l | -a | -k | -v |
| g | t | 0 | -t | -g | -g | -t | 0 | t | g | g | t | 0 | -t | -g | -g | -t | 0 | t | g | g | t | 0 | -t | -g | -g | -t | 0 | t | g | g | t |
| h | w | -B | -m | -c | -r | 0 | r | c | m | B | -w | -h | -h | -w | B | m | c | r | 0 | -r | -c | -m | -B | w | h | h | w | -B | -m | -c | -r |
| i | z | -w | -f | -l | -C | t | c | o | F | -q | -a | -r | E | n | d | u | -B | -k | -g | -x | y | h | j | A | -v | -e | -m | -D | s | b | p |
| j | C | -r | -b | -u | z | g | m | F | -o | -e | -x | w | d | p | -E | -l | -h | -A | t | a | s | -B | -i | -k | -D | q | c | v | -y | -f | -n |
| k | F | -m | -i | -D | o | g | B | -q | -e | -z | s | c | x | -u | -a | -v | w | b | t | -y | -d | -r | A | f | p | -C | -h | -n | E | j | l |
| l | -E | -h | -p | A | d | t | -w | -a | -x | s | e | B | -o | -i | -F | k | m | -D | -g | -q | z | c | u | -v | -b | -y | r | f | C | -n | -j |
| m | -B | -c | -w | r | h | 0 | -h | -r | w | c | B | -m | -m | B | c | w | -r | -h | 0 | h | r | -w | -c | -B | m | m | -B | -c | -w | r | h |
| n | -y | -c | -D | i | s | -t | -h | E | d | x | -o | -m | z | b | C | -j | -r | u | g | -F | -e | -w | p | l | -A | -a | -B | k | q | -v | -f |
| o | -v | -h | C | a | D | -g | -w | n | p | -u | -i | B | b | E | -f | -x | m | q | -t | -j | A | c | F | -e | -y | l | r | -s | -k | z | d |
| p | -s | -m | v | j | -y | -g | B | d | -E | -a | -F | c | C | -f | -z | i | w | -l | -t | o | q | -r | -n | u | k | -x | -h | A | e | -D | -b |
| q | -p | -r | o | s | -n | -t | m | u | -l | -v | k | w | -j | -x | i | y | -h | -z | g | A | -f | -B | e | C | -d | -D | c | E | -b | -F | a |
| r | -m | -w | h | B | -c | 0 | c | -B | -h | w | m | -r | -r | m | w | -h | -B | c | 0 | -c | B | h | -w | -m | r | r | -m | -w | h | B | -c |
| s | -j | -B | a | -C | -i | t | r | -k | -A | b | -D | -h | u | q | -l | -z | c | -E | -g | v | p | -m | -y | d | -F | -f | w | o | -n | -x | e |
| t | -g | 0 | g | -t | -t | g | 0 | -g | t | t | -g | 0 | g | -t | -t | g | 0 | -g | t | t | -g | 0 | g | -t | -t | g | 0 | -g | t | t | -g |
| u | -d | B | n | -k | -E | g | -r | -x | a | -y | -q | h | -F | -j | o | A | -c | v | t | -e | C | m | -l | -D | f | -s | -w | b | -z | -p | i |
| v | -a | w | u | -b | x | t | -c | y | s | -d | z | r | -e | A | q | -f | B | p | -g | C | o | -h | D | n | -i | E | m | -j | F | l | -k |
| w | -c | r | B | -h | m | 0 | -m | h | -B | -r | c | -w | -w | c | -r | -B | h | -m | 0 | m | -h | B | r | -c | w | w | -c | r | B | -h | m |
| x | -f | m | -E | -q | b | -t | -B | j | -i | A | u | -c | p | F | -n | e | -w | -y | g | -l | D | r | -a | s | C | -k | h | -z | -v | d | -o |
| y | -i | h | -x | -z | j | -g | w | A | -k | f | -v | -B | l | -e | u | C | -m | d | -t | -D | n | -c | s | E | -o | b | -r | -F | p | -a | q |
| z | -l | c | -q | E | u | -g | h | -v | -D | p | -b | m | -A | -y | k | -d | r | -F | -t | f | -i | w | C | -o | a | -n | B | x | -j | e | -s |
| A | -o | c | -j | v | F | -t | h | -e | q | -C | -y | m | -a | l | -x | -D | r | -f | g | -s | E | w | -k | b | -n | z | B | -p | d | -i | u |
| B | -r | h | -c | m | -w | 0 | w | -m | c | -h | r | -B | -B | r | -h | c | -m | w | 0 | -w | m | -c | h | -r | B | B | -r | h | -c | m | -w |
| C | -u | m | -e | d | -l | t | -B | -D | v | -n | f | -c | k | -s | A | E | -w | o | -g | b | -j | r | -z | -F | x | -p | h | -a | i | -q | y |
| D | -x | r | -l | f | -a | g | -m | s | -y | E | C | -w | q | -k | e | -b | h | -n | t | -z | F | B | -v | p | -j | d | -c | i | -o | u | -A |
| E | -A | w | -s | o | -k | g | -c | b | -f | j | -n | r | -v | z | -D | -F | B | -x | t | -p | l | -h | d | -a | e | -i | m | -q | u | -y | C |
| F | -D | B | -z | x | -v | t | -r | p | -n | l | -j | h | -f | d | -b | a | -c | e | -g | i | -k | m | -o | q | -s | u | -w | y | -A | C | -E |

DST7

The table below contains values for the DST7 matrices that follow.  6 bit VVC standard matrix values are included for reference.

| Label | 4x4 |  | 8x8 |  | 16x16 |  | 32x32 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 14 bit | 6 bit | 14 bit | 6 bit | 14 bit | 6 bit | 14 bit | 6 bit |
| a | 7425 | 29 | 4350 | 17 | 1967 | 8 | 1043 | 4 |
| b | 14081 | 55 | 8148 | 32 | 4295 | 17 | 2291 | 9 |
| c | 18893 | 74 | 11754 | 46 | 6421 | 25 | 3225 | 13 |
| d | 21505 | 84 | 15363 | 60 | 8498 | 33 | 4478 | 17 |
| e |  |  | 18154 | 71 | 10360 | 40 | 5414 | 21 |
| f |  |  | 19958 | 78 | 12209 | 48 | 6623 | 26 |
| g |  |  | 21790 | 85 | 14044 | 55 | 7734 | 30 |
| h |  |  | 22018 | 86 | 15781 | 62 | 8606 | 34 |
| i |  |  |  |  | 17369 | 68 | 9720 | 38 |
| j |  |  |  |  | 18787 | 73 | 10770 | 42 |
| k |  |  |  |  | 19738 | 77 | 11526 | 46 |
| l |  |  |  |  | 20754 | 81 | 12679 | 50 |
| m |  |  |  |  | 21664 | 85 | 13616 | 53 |
| n |  |  |  |  | 22202 | 87 | 14312 | 56 |
| o |  |  |  |  | 22542 | 88 | 15230 | 60 |
| p |  |  |  |  | 22569 | 88 | 16143 | 63 |
| q |  |  |  |  |  |  | 16894 | 66 |
| r |  |  |  |  |  |  | 17448 | 68 |
| s |  |  |  |  |  |  | 18415 | 72 |
| t |  |  |  |  |  |  | 19007 | 74 |
| u |  |  |  |  |  |  | 19708 | 77 |
| v |  |  |  |  |  |  | 20019 | 78 |
| w |  |  |  |  |  |  | 20385 | 80 |
| x |  |  |  |  |  |  | 21131 | 82 |
| y |  |  |  |  |  |  | 21463 | 84 |
| z |  |  |  |  |  |  | 21901 | 85 |
| A |  |  |  |  |  |  | 22053 | 86 |
| B |  |  |  |  |  |  | 22544 | 87 |
| C |  |  |  |  |  |  | 22533 | 88 |
| D |  |  |  |  |  |  | 22715 | 89 |
| E |  |  |  |  |  |  | 23136 | 90 |
| F |  |  |  |  |  |  | 23065 | 90 |

4x4 DST7

The 4x4 DST7 transform matrix is of the form below and uses values from the table above.

| a | b | c | d |
| --- | --- | --- | --- |
| c | c | 0 | -c |
| d | -a | -c | b |
| b | -d | c | -a |

8x8 DST7

The 8x8 DST7 transform matrix is of the form below and uses values from the table above.

| a | B | c | d | e | f | g | h |
| --- | --- | --- | --- | --- | --- | --- | --- |
| c | F | h | e | b | -a | -d | -g |
| e | G | b | -c | -h | -d | a | f |
| g | C | -d | -f | a | h | b | -e |
| h | -a | -g | b | f | -c | -e | d |
| f | -e | -a | g | -d | -b | h | -c |
| d | -h | e | -a | -c | g | -f | b |
| b | -d | f | -h | g | -e | c | -a |

16x16 DST7

The 16x16 DST7 transform matrix is of the form below and uses values from the table above.

| a | b | C | d | e | f | g | h | i | j | k | l | m | n | o | p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| c | f | I | l | o | o | l | i | f | c | 0 | -c | -f | -i | -l | -o |
| e | j | O | m | h | c | -b | -g | -l | -p | -k | -f | -a | d | i | n |
| g | n | L | e | -b | -i | -p | -j | -c | d | k | o | h | a | -f | -m |
| i | o | F | -c | -l | -l | -c | f | o | i | 0 | -i | -o | -f | c | l |
| k | k | 0 | -k | -k | 0 | k | k | 0 | -k | -k | 0 | k | k | 0 | -k |
| m | g | -f | -n | -a | l | h | -e | -o | -b | k | i | -d | -p | -c | j |
| o | c | -l | -f | i | i | -f | -l | c | o | 0 | -o | -c | l | f | -i |
| p | -a | -o | b | n | -c | -m | d | l | -e | -k | f | j | -g | -i | h |
| n | -e | -i | j | d | -o | a | m | -f | -h | k | c | -p | b | l | -g |
| l | -i | -c | o | -f | -f | o | -c | -i | l | 0 | -l | i | c | -o | f |
| j | -m | C | g | -p | f | d | -n | i | a | -k | l | -b | -h | o | -e |
| h | -p | I | -a | -g | o | -j | b | f | -n | k | -c | -e | m | -l | d |
| f | -l | O | -i | c | c | -i | o | -l | f | 0 | -f | l | -o | i | -c |
| d | -h | L | -p | m | -i | e | -a | -c | g | -k | o | -n | j | -f | b |
| b | -d | F | -h | j | -l | n | -p | o | -m | k | -i | g | -e | c | -a |

32x32 DST7

The 32x32 DST7 transform matrix is of the form below and uses values from the table above.

| a | b | c | d | e | f | g | h | i | j | K | l | m | n | o | p | q | r | s | t | u | v | w | x | y | z | A | B | C | D | E | F |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| c | f | i | l | o | r | u | x | A | D | F | C | z | w | t | q | n | k | h | e | b | -a | -d | -g | -j | -m | -p | -s | -v | -y | -B | -E |
| e | j | o | t | y | D | D | y | t | o | j | e | 0 | -e | -j | -o | -t | -y | -D | -D | -y | -t | -o | -j | -e | 0 | e | j | o | t | y | D |
| g | n | u | B | D | w | p | i | b | -e | -l | -s | -z | -F | -y | -r | -k | -d | c | j | q | x | E | A | t | m | f | -a | -h | -o | -v | -C |
| i | r | A | C | t | k | b | -g | -p | -y | -E | -v | -m | -d | e | n | w | F | x | o | f | -c | -l | -u | -D | -z | -q | -h | a | j | s | B |
| k | v | F | u | j | -a | -l | -w | -E | -t | -i | b | m | x | D | s | h | -c | -n | -y | -C | -r | -g | d | o | z | B | q | f | -e | -p | -A |
| m | z | z | m | 0 | -m | -z | -z | -m | 0 | m | z | z | m | 0 | -m | -z | -z | -m | 0 | m | z | z | m | 0 | -m | -z | -z | -m | 0 | m | z |
| o | D | t | e | -j | -y | -y | -j | e | t | D | o | 0 | -o | -D | -t | -e | j | y | y | j | -e | -t | -D | -o | 0 | o | D | t | e | -j | -y |
| q | E | n | -c | -t | -B | -k | f | w | y | h | -i | -z | -v | -e | l | C | s | b | -o | -F | -p | a | r | D | m | -d | -u | -A | -j | g | x |
| s | A | h | -k | -D | -p | c | v | x | e | -n | -F | -m | f | y | u | b | -q | -C | -j | i | B | r | -a | -t | -z | -g | l | E | o | -d | -w |
| u | w | b | -s | -y | -d | q | A | f | -o | -C | -h | m | E | j | -k | -F | -l | i | D | n | -g | -B | -p | e | z | r | -c | -x | -t | a | v |
| w | s | -d | -A | -o | h | E | k | -l | -D | -g | p | z | c | -t | -v | a | x | r | -e | -B | -n | i | F | j | -m | -C | -f | q | y | b | -u |
| y | o | -j | -D | -e | t | t | -e | -D | -j | o | y | 0 | -y | -o | j | D | e | -t | -t | e | D | j | -o | -y | 0 | y | o | -j | -D | -e | t |
| A | k | -p | -v | e | F | f | -u | -q | j | B | a | -z | -l | o | w | -d | -E | -g | t | r | -i | -C | -b | y | m | -n | -x | c | D | h | -s |
| C | g | -v | -n | o | u | -h | -B | a | D | f | -w | -m | p | t | -i | -A | b | E | e | -x | -l | q | s | -j | -z | c | F | d | -y | -k | r |
| E | c | -B | -f | y | i | -v | -l | s | o | -p | -r | m | u | -j | -x | g | A | -d | -D | a | F | b | -C | -e | z | h | -w | -k | t | n | -q |
| F | -a | -E | b | D | -c | -C | d | B | -e | -A | f | z | -g | -y | h | x | -i | -w | j | v | -k | -u | l | t | -m | -s | n | r | -o | -q | p |
| D | -e | -y | j | t | -o | -o | t | j | -y | -e | D | 0 | -D | e | y | -j | -t | o | o | -t | -j | y | e | -D | 0 | D | -e | -y | j | t | -o |
| B | -i | -s | r | j | -A | -a | C | -h | -t | q | k | -z | -b | D | -g | -u | p | l | -y | -c | E | -f | -v | o | m | -x | -d | F | -e | -w | n |
| z | -m | -m | z | 0 | -z | m | m | -z | 0 | z | -m | -m | z | 0 | -z | m | m | -z | 0 | z | -m | -m | z | 0 | -z | m | m | -z | 0 | z | -m |
| x | -q | -g | E | -j | -n | A | -c | -u | t | d | -B | m | k | -D | f | r | -w | -a | y | -p | -h | F | -i | -o | z | -b | -v | s | e | -C | l |
| v | -u | -a | w | -t | -b | x | -s | -c | y | -r | -d | z | -q | -e | A | -p | -f | B | -o | -g | C | -n | -h | D | -m | -i | E | -l | -j | F | -k |
| t | -y | e | o | -D | j | j | -D | o | e | -y | t | 0 | -t | y | -e | -o | D | -j | -j | D | -o | -e | y | -t | 0 | t | -y | e | o | -D | j |
| r | -C | k | g | -y | v | -d | -n | F | -o | -c | u | -z | h | j | -B | s | -a | -q | D | -l | -f | x | -w | e | m | -E | p | b | -t | A | -i |
| p | -F | q | -a | -o | E | -r | b | n | -D | s | -c | -m | C | -t | d | l | -B | u | -e | -k | A | -v | f | j | -z | w | -g | -i | y | -x | h |
| n | -B | w | -i | -e | s | -F | r | -d | -j | x | -A | m | a | -o | C | -v | h | f | -t | E | -q | c | k | -y | z | -l | -b | p | -D | u | -g |
| l | -x | C | -q | e | g | -s | E | -v | j | b | -n | z | -A | o | -c | -i | u | -F | t | -h | -d | p | -B | y | -m | a | k | -w | D | -r | f |
| j | -t | D | -y | o | -e | -e | o | -y | D | -t | j | 0 | -j | t | -D | y | -o | e | e | -o | y | -D | t | -j | 0 | j | -t | D | -y | o | -e |
| h | -p | x | -F | y | -q | i | -a | -g | o | -w | E | -z | r | -j | b | f | -n | v | -D | A | -s | k | -c | -e | m | -u | C | -B | t | -l | d |
| f | -l | r | -x | D | -C | w | -q | k | -e | -a | g | -m | s | -y | E | -B | v | -p | j | -d | -b | h | -n | t | -z | F | -A | u | -o | i | -c |
| d | -h | l | -p | t | -x | B | -F | C | -y | u | -q | m | -i | e | -a | -c | g | -k | o | -s | w | -A | E | -D | z | -v | r | -n | j | -f | b |
| b | -d | f | -h | j | -l | n | -p | r | -t | v | -x | z | -B | D | -F | E | -C | A | -y | w | -u | s | -q | o | -m | k | -i | g | -e | c | -a |