| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0353-v2 |
| --- | --- |

| Title: | AHG14: On interaction between ACT and BDPCM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In HEVC, RDPCM, which is a similar coding tool to BDPCM in VVC, can be applied to both luma and chroma components when Adaptive color transform is used. On the other hand, in current VVC, interaction between ACT and Intra BDPCM is inconsistency; When ACT is used, Intra BDPCM for chroma is normatively disallowed while Intra BDPCM for luma is allowed.

This contribution proposes to enable a combination of ACT and Chroma BDPCM, which makes the interaction between ACT and Luma/Chroma BDPCM consistency.

Simulation results show that:

For 444/lossless condition (DualTreeOn),

- Average bitrate differences for RGB are 0.0% for AI, -0.71% for RA and -0.90% for LB

- Runtimes (Enc, Dec) for RGB are (99%, 97%) for AI, (100%, 97%) for RA and (101%, 98%) for LB

For 444/lossless condition (DualTreeOff),

- Average bitrate differences for RGB are -2.19% for AI, -1.23% for RA and -1.19% for LB

- Runtimes (Enc, Dec) for RGB are (96%, 92%) for AI, (99%, 98%) for RA and (100%, 96%) for LB

For 444/SCC/common QP condition (QP=22, 27, 32, 37, DualTreeOff),

- Average BD-rate differences (R, G, B) are (0.07%, 0.10%, 0.09%) for AI, (0.01%, 0.03%, 0.04%) for RA and (x%, x%, x%) for LB

- Runtimes (Enc, Dec) for RGB are (96%, 97%) for AI, (99%, 100%) for RA and (x%, x%) for LB

For 444/SCC/low QP condition (QP=2, 7, 12, 17, DualTreeOff),

- Average BD-rate differences (R, G, B) are (-0.22%, -0.18%, -0.20%) for AI, (x%, x%, x%) for RA and (-0.13%, -0.14%, -0.17%) for LB

- Runtimes (Enc, Dec) for RGB are (95%, 99%) for AI, (x%, x%) for RA and (99%, 98%) for LB

In v2, software is included.

Introduction

In HEVC [1], RDPCM, which is a similar coding tool to BDPCM in VVC [2], can be applied to both luma and chroma components when Adaptive color transform is used. On the other hand, in current VVC, interaction between ACT and Intra BDPCM for luma and chroma is inconsistency; When ACT is used, Intra BDPCM for chroma is normatively disallowed while Intra BDPCM for luma is allowed.

Related syntaxes in VVC are below:

Table 1-1: Syntax table related to BDPCM for luma/chroma

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_act_enabled_flag && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag // ACT for Intra | ae(v) |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) |  |
| else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_luma_flag | ae(v) |
| if( intra_bdpcm_luma_flag ) |  |
| intra_bdpcm_luma_dir_flag | ae(v) |
| else { |  |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( pred_mode_plt_flag && treeType = = DUAL_TREE_CHROMA ) |  |
| palette_coding( x0, y0, cbWidth / SubWidthC, cbHeight / SubHeightC, treeType ) |  |
| else if( !pred_mode_plt_flag ) { |  |
| if( !cu_act_enabled_flag ) { |  |
| if( cbWidth / SubWidthC <= MaxTsSize && cbHeight / SubHeightC <= MaxTsSize && sps_bdpcm_enabled_flag ) |  |
| intra_bdpcm_chroma_flag | ae(v) |
| if( intra_bdpcm_chroma_flag ) |  |
| intra_bdpcm_chroma_dir_flag | ae(v) |
| else { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| … |  |
| if( sps_act_enabled_flag && (CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag // ACT for non-Intra | ae(v) |
| … |  |

As can be seen, if cu_act_enabled_flag is equal to 1, syntaxes related to BDPCM for chroma (intra_bdpcm_chroma_flag, intra_bdpcm_chroma_dir_flag) and chroma intra prediction mode (cclm_mode_flag, etc…) are skipped. The chroma intra prediction mode is treated as Direct Mode, i.e. IntraPredModeC is set equal to lumaIntraPredMode as below:

Table 1-2: Derivation process for chroma intra prediction mode when ACT is used

8.4.3. Derivation process for chroma intra prediction mode … The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived as follows: - If cu_act_enabled_flag[ xCb ][ yCb ] is equal to 1, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is set equal to lumaIntraPredMode. - Otherwise, if BdpcmFlag[ xCb ][ yCb ][ 1 ] is equal to 1, IntraPredModeC[ xCb ][ yCb ] is set equal to BdpcmDir[ xCb ][ yCb ][ 1 ] ? INTRA_ANGULAR50 : INTRA_ANGULAR18. - Otherwise ( cu_act_enabled_flag[ xCb ][ yCb ] is equal to 0 and BdpcmFlag[ xCb ][ yCb ][ 1 ] is equal to 0 ), the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using cclm_mode_flag, cclm_mode_idx, intra_chroma_pred_mode and lumaIntraPredMode as specified in Table 20. …

Table 1-3 summarizes allowed combination of ACT/BDPCM/intra prediction mode when ACT is used in VVC. As can be seen, case 1/7/11 (highlighted in yellow) are allowed combination in VVC. However, conceptually, case 3/6/9/14 (highlighted in green) are considered as possible combinations since chroma intra prediction mode is identical to luma intra prediction mode in such a case.

Table 1-3: Allowed combination of ACT/BDPCM/intra prediction mode when ACT is used

| Case | ACT (cu_act_enabled_flag) | Luma BDPCM (BdpcmMode) | Luma Intra Pred | Chroma BDPCM (BdpmMode) | Chroma Intra Pred | Allowed combination? |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 (off) | Any | 0 (off) | DM | Y |
| 2 | 1 | 0 | Any | 0 | Not DM | N |
| 3 | 1 | 0 | HOR | 1 (HOR) | HOR | N |
| 4 | 1 | 0 | HOR | 2 (VER) | VER | N |
| 5 | 1 | 0 | VER | 1 | HOR | N |
| 6 | 1 | 0 | VER | 2 | VER | N |
| 7 | 1 | 1 (HOR) | HOR | 0 | DM (HOR) | Y |
| 8 | 1 | 1 | HOR | 0 | Not DM | N |
| 9 | 1 | 1 | HOR | 1 | HOR | N |
| 10 | 1 | 1 | HOR | 2 | VER | N |
| 11 | 1 | 2 (VER) | VER | 0 | DM | Y |
| 12 | 1 | 2 | VER | 0 | Not DM | N |
| 13 | 1 | 2 | VER | 1 | HOR | N |
| 14 | 1 | 2 | VER | 2 | VER | N |

, where

- BdpcmMode equal to 0 indicates (intra_bdpcm_x_flag, intra_bdpcm_x_dir_flag) equal to (0,0).

- BdpcmMode equal to 1 indicates (intra_bdpcm_x_flag, intra_bdpcm_x_dir_flag) equal to (1,0).

- BdpcmMode equal to 2 indicates (intra_bdpcm_x_flag, intra_bdpcm_x_dir_flag) equal to (1,1).

- DM stands for Direct Mode

Proposal

To make interaction between ACT and Luma/Chroma BDPCM consistency, it is proposed to enable a combination of ACT and Chroma BDPCM identical to case 3/6/9/14 shown in Table 1-3.

Summary of proposed changes are as below:

- Align signaling location of cu_act_enabled_flag for Intra CU with that of cu_act_enabled_flag for Non-Intra CU

- Signal cu_act_enabled_flag for Intra CU if following conditions are satisfied:

- CuPredMode is equal to MODE_INTRA

- IntraSubpartitionSplitType is equal to ISP_NO_SPLIT

- sps_act_enabled_flag is equal to 1

- chorma intra prediction mode is identical to luma intra prediction

- No change to signaling condition of cu_act_enabled_flag for Non-Intra CU

- To reduce encoder runtime for selecting Chroma BDPCM in ACT mode, Luma BDPCM mode is reused as Chroma BDPCM mode

  - Set intra_bdpcm_chroma_flag equal to intra_bdpcm_luma_flag

  - Set intra_bdpcm_chroma_dir_flag equal to intra_bdpcm_luma_dir_flag

Highlighted syntax changes are as below:

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_act_enabled_flag && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| … |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) |  |
| else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_luma_flag | ae(v) |
| if( intra_bdpcm_luma_flag ) |  |
| intra_bdpcm_luma_dir_flag | ae(v) |
| else { |  |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 ) { |  |
| if( pred_mode_plt_flag && treeType = = DUAL_TREE_CHROMA ) |  |
| palette_coding( x0, y0, cbWidth / SubWidthC, cbHeight / SubHeightC, treeType ) |  |
| else if( !pred_mode_plt_flag ) { |  |
| if( !cu_act_enabled_flag ) { |  |
| if( cbWidth / SubWidthC <= MaxTsSize && cbHeight / SubHeightC <= MaxTsSize && sps_bdpcm_enabled_flag ) |  |
| intra_bdpcm_chroma_flag | ae(v) |
| if( intra_bdpcm_chroma_flag ) |  |
| intra_bdpcm_chroma_dir_flag | ae(v) |
| else { |  |
| if( CclmEnabled ) |  |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } else if( treeType != DUAL_TREE_CHROMA ) { /* MODE_INTER or MODE_IBC */ |  |
| … |  |
| if( sps_act_enabled_flag && (CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦¦ ( CuPredMode[ chType ][ x0 ][ y0 ] == MODE_INTRA && IntraSubpartitionSplitType == ISP_NO_SPLIT && ( (intra_chroma_pred_mode == 4) ¦¦ (IntraPredModeY[ x0 ][ y0 ] == 18 && intra_bdpcm_chroma_flag == 1 && intra_bdpcm_chroma_dir_flag == 0) ¦¦ (IntraPredModeY[ x0 ][ y0 ] == 50 && intra_bdpcm_chroma_flag == 1 && intra_bdpcm_chroma_dir_flag == 1) ) ) && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| … |  |

8.4.3. Derivation process for chroma intra prediction mode … The chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived as follows: - If cu_act_enabled_flag[ xCb ][ yCb ] is equal to 1, the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is set equal to lumaIntraPredMode. - Otherwise, if BdpcmFlag[ xCb ][ yCb ][ 1 ] is equal to 1, IntraPredModeC[ xCb ][ yCb ] is set equal to BdpcmDir[ xCb ][ yCb ][ 1 ] ? INTRA_ANGULAR50 : INTRA_ANGULAR18. - Otherwise ( cu_act_enabled_flag[ xCb ][ yCb ] is equal to 0 and BdpcmFlag[ xCb ][ yCb ][ 1 ] is equal to 0 ), the chroma intra prediction mode IntraPredModeC[ xCb ][ yCb ] is derived using cclm_mode_flag, cclm_mode_idx, intra_chroma_pred_mode and lumaIntraPredMode as specified in Table 20. …

Changes relative to JVET-Q2001vE are found in JVET-R0353_DraftText.docx

Simulation Results

The proposed method is implemented on the VTM-8.0 and tested under following test conditions:

- 444/lossless coding/DualTreeOn [3]

- 444/lossless coding/DualTreeOff following [3]

- 444/SCC/common QP condition (DualTreeOff) [4]

- 444/SCC/low QP condition (QP=2,7,12,17, DualTreeOff) with first 100 frames following [4]

Table 3-1: Coding Performance under 444/lossless condition/DualTreeOn

| RGB | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 10.9 | 10.9 | 0.00% |
| TGM 720p | 5.0 | 5.0 | 0.00% |
| A | 3.0 | 3.0 | 0.00% |
| M | 3.0 | 3.0 | 0.00% |
| CC | 0.9 | 0.9 | 0.00% |
| Overall | 5.2 | 5.2 | 0.00% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 97% |  |  |
| YUV | All Intra |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 10.7 | 10.7 | 0.00% |
| TGM 720p | 6.4 | 6.4 | 0.00% |
| A | 3.5 | 3.5 | 0.00% |
| M | 3.9 | 3.9 | 0.00% |
| CC | 1.0 | 1.0 | 0.00% |
| Overall | 5.8 | 5.8 | 0.00% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

| RGB | Random Access |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 173.2 | 173.2 | -0.10% |
| TGM 720p | 51.5 | 52.2 | -1.03% |
| A | 8.6 | 8.6 | -0.41% |
| M | 19.1 | 19.5 | -1.79% |
| CC | 0.9 | 0.9 | -0.12% |
| Overall | 61.5 | 61.7 | -0.71% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |
| YUV | Random Access |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 174.7 | 174.7 | 0.00% |
| TGM 720p | 64.5 | 64.5 | 0.00% |
| A | 7.9 | 7.9 | 0.00% |
| M | 26.3 | 26.3 | 0.00% |
| CC | 1.1 | 1.1 | 0.00% |
| Overall | 66.3 | 66.3 | 0.00% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 99% |  |  |

| RGB | Low delay B |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 251.2 | 251.4 | -0.11% |
| TGM 720p | 90.1 | 93.0 | -1.74% |
| A | 7.1 | 7.2 | -0.42% |
| M | 21.0 | 21.4 | -1.84% |
| CC | 0.9 | 0.9 | -0.09% |
| Overall | 90.7 | 91.6 | -0.90% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 98% |  |  |
| YUV | Low delay B |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 252.9 | 252.9 | 0.00% |
| TGM 720p | 115.8 | 115.8 | 0.00% |
| A | 7.9 | 7.9 | 0.00% |
| M | 28.9 | 28.9 | 0.00% |
| CC | 1.1 | 1.1 | 0.00% |
| Overall | 99.2 | 99.2 | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |

Table 3-2: Coding Performance under 444/lossless condition/DualTreeOff

| RGB | All Intra |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 14.0 | 14.0 | -0.78% |
| TGM 720p | 5.9 | 6.1 | -3.63% |
| A | 3.2 | 3.3 | -1.83% |
| M | 3.3 | 3.5 | -3.52% |
| CC | 0.9 | 0.9 | -0.67% |
| Overall | 6.3 | 6.4 | -2.19% |
| Enc Time[%] | 96% |  |  |
| Dec Time[%] | 92% |  |  |
| YUV | All Intra |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 14.4 | 14.4 | 0.00% |
| TGM 720p | 7.5 | 7.5 | 0.00% |
| A | 3.7 | 3.7 | 0.00% |
| M | 4.4 | 4.4 | 0.00% |
| CC | 1.0 | 1.0 | 0.00% |
| Overall | 7.1 | 7.1 | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 98% |  |  |

| RGB | Random Access |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 215.0 | 217.0 | -0.56% |
| TGM 720p | 57.1 | 59.3 | -2.29% |
| A | 8.7 | 8.7 | -0.51% |
| M | 19.4 | 19.8 | -2.18% |
| CC | 0.9 | 0.9 | -0.15% |
| Overall | 73.4 | 74.5 | -1.23% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 98% |  |  |
| YUV | Random Access |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 225.6 | 225.6 | 0.00% |
| TGM 720p | 73.6 | 73.6 | 0.00% |
| A | 8.0 | 8.0 | 0.00% |
| M | 26.8 | 26.8 | 0.00% |
| CC | 1.1 | 1.1 | 0.00% |
| Overall | 81.4 | 81.4 | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 96% |  |  |

| RGB | Low delay B |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 300.6 | 302.9 | -0.46% |
| TGM 720p | 94.5 | 99.1 | -2.35% |
| A | 7.2 | 7.2 | -0.46% |
| M | 21.2 | 21.7 | -2.06% |
| CC | 0.9 | 0.9 | -0.10% |
| Overall | 104.2 | 106.0 | -1.19% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 96% |  |  |
| YUV | Low delay B |  |  |
|  | ratio |  | bit-rate savings |
|  | VTM-8.0 | Tested |  |
| TGM 1080p | 312.4 | 312.4 | 0.00% |
| TGM 720p | 123.7 | 123.7 | 0.00% |
| A | 7.9 | 7.9 | 0.00% |
| M | 29.2 | 29.2 | 0.00% |
| CC | 1.1 | 1.1 | 0.00% |
| Overall | 116.1 | 116.1 | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |

Table 3-3: Coding performance under 444/SCC/common QP condition

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.15% | 0.12% | 0.12% | 94% | 97% |
| TGM 720p | -0.01% | 0.08% | -0.02% | 96% | 97% |
| Animation | 0.05% | 0.06% | 0.10% | 97% | 96% |
| Mixed content | 0.10% | 0.14% | 0.18% | 97% | 97% |
| Overall | 0.07% | 0.10% | 0.09% | 96% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.01% | 0.06% | 0.04% | 98% | 101% |
| TGM 720p | 0.04% | 0.06% | 0.05% | 99% | 100% |
| Animation | 0.07% | -0.02% | 0.04% | 100% | 99% |
| Mixed content | -0.07% | -0.01% | 0.01% | 101% | 101% |
| Overall | 0.01% | 0.03% | 0.04% | 99% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.06% | -0.04% | -0.07% | 101% | 99% |
| TGM 720p | 0.00% | 0.05% | 0.00% | 100% | 99% |
| Animation |  |  |  |  |  |
| Mixed content | -0.21% | -0.26% | -0.15% | 98% | 97% |
| Overall |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 98% | 95% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 97% |
|  | Random access Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 98% | 98% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Animation | 0.00% | 0.00% | 0.00% | 98% | 102% |
| Mixed content | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 96% | 93% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 98% | 96% |
| Animation | 0.00% | 0.00% | 0.00% | 95% | 94% |
| Mixed content | 0.00% | 0.00% | 0.00% | 98% | 90% |
| Overall | 0.00% | 0.00% | 0.00% | 97% | 94% |

Table 3-4: Coding performance under 444/SCC/low QP condition

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.14% | -0.02% | -0.05% | 93% | 99% |
| TGM 720p | -0.36% | -0.30% | -0.32% | 94% | 98% |
| Animation | -0.13% | -0.24% | -0.29% | 98% | 101% |
| Mixed content | -0.22% | -0.19% | -0.17% | 96% | 98% |
| Overall | -0.22% | -0.18% | -0.20% | 95% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.08% | 0.01% | -0.05% | 99% | 100% |
| TGM 720p | -0.07% | -0.08% | -0.21% | 99% | 99% |
| Animation |  |  |  |  |  |
| Mixed content | -0.05% | -0.11% | -0.12% | 99% | 99% |
| Overall |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | RGB 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.10% | -0.06% | -0.04% | 100% | 99% |
| TGM 720p | -0.19% | -0.16% | -0.28% | 99% | 100% |
| Animation | -0.05% | -0.22% | -0.23% | 100% | 95% |
| Mixed content | -0.14% | -0.13% | -0.15% | 99% | 98% |
| Overall | -0.13% | -0.14% | -0.17% | 99% | 98% |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Animation | 0.00% | 0.00% | 0.00% | 98% | 97% |
| Mixed content | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Animation | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Mixed content | 0.00% | 0.00% | 0.00% | 99% | 93% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | YUV 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 101% | 99% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Animation | 0.00% | 0.00% | 0.00% | 98% | 92% |
| Mixed content | 0.00% | 0.00% | 0.00% | 99% | 93% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 96% |

As can be seen, the proposed method provides additional gains for RGB444 without runtime increase under 444/lossless coding and 444/SCC/low QP condition. Especially, for AI, encoder runtime decreased by ~5% thanks to early termination already implemented in encoder search for ACT.

Conclusion

This contribution proposes to enable a combination of ACT and Chroma BDPCM, which makes the interaction between ACT and Luma/Chroma BDPCM consistency. Simulation results shows that the proposed method has coding performance improvements under 444/lossless coding and 444/SCC/low QP condition.

It is recommended to adopt the proposed method to the next VTM/VVC WD.

Reference

[1] ISO/IEC 23008-2 | ITU-T Rec. H.265, “High efficiency video coding”, ISO/IEC | ITU-T standard, 2019.

[2] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8)”, JVET-Q2001, Jan. 2020.

[3] T.-C. Ma, A. Nalci, T. Nguyen, “JVET common test conditions and software reference configurations for lossless, near lossless, and mixed lossy/lossless coding”, JVET-Q2014, Apr. 2020.

[4] Y.-H. Chao, Y.-C. Sun, J. Xu, X. Xu, “JVET common test conditions and software reference configurations for non-4:2:0 colour formats”, JVET-Q2013, Apr. 2020.

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).