| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0358-v1 |
| --- | --- |

| Title: | A combined solution for latency reduction in transformation process with TU-level signalling and removal of chroma LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Vadim Seregin, Wei-Jung Chien, Ted Hsieh, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Tzu-Der Chuang, Man-Shu Chiang, Zhi-Yi Lin, Chih-Wei Hsu, Ching-Yeh Chen,Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Xiaoyu Xiu, Tsung-Chuan Ma, Yi-Wen Chen, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA Chris Rosewarne, Jonathan Gan Building A, 5 Talavera Rd, Macquarie Park, NSW 2113, Australia | Email: | hegilmez@qti.qualcomm.com analci@qti.qualcomm.com vseregin@qti.qualcomm.com wchien@qti.qualcomm.com thsieh@qti.qualcomm.com martak@qti.qualcomm.com peter.chuang@mediatek.com man-shu.chiang@mediatek.com zhi-yi.lin@mediatek.com cw.hsu@mediatek.com chingyeh.chen@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com xiaoyuxiu@kwai.com tsung-chuanma@kwai.com yiwenchen@kwai.com jhuhong-jheng@kwai.com xianglinwang@kwai.com chris.rosewarne@canon.com.au jonathan.gan@canon.com.au |
| Source: | Qualcomm Inc., MediaTek Inc., Kwai Inc, and Canon Australia Pty Ltd. |  |  |

_____________________________

# Abstract

In the current version of VVC (draft 8), the transform-related signaling is performed after residual coding at CU-level, and it depends on the positions of coded coefficients. This dependency inherently introduces additional latency, particularly critical for the two-stage LFNST process, since the parsing of LFNST index and the (inverse) transformation process can only start after coefficients from all color components are decoded. The latency is even further increased when the LFNST process is applied for chroma blocks. In order to reduce the latency and computational complexity, this contribution proposes a combined solution based on the proposals JVET-R0234, JVET-R0235, JVET-R0236, JVET-R0056, JVET-R0331, and JVET-R0176. The proposed combination (i) signals the transform syntax elements at the TU-level after coding luma transform blocks and (ii) removes LFNST from chroma. The experimental results under common test conditions (CTC) show that the proposed combination leads to average BD-rates of “Y:0.12%, U:1.14%, V:1.37%” in AI, and “Y:0.07%, U: 0.27%, V: 0.37%” in RA configurations with about 6% encoder speed-up in AI configurations. This minor coding loss is mainly due to removing LFNST from chroma components, and proposed signaling changes have negligible impact. Additional experimental results with a slightly different encoder setting show that the BD-rate losses can be reduced to “Y:0.02%, U:0.82%, V:1.03%” in AI and “Y:0.04%, U:0.13%, V:0.34%” in RA configurations with similar average encoding runtimes as compared to the VTM-8.0 anchor.

# Introduction

In the VVC specification working draft (JVET-Q2001-vE), both MTS and LFNST indices (mts_idx and lfnst_idx) are signalled after residual coding of all color components at coding unit (CU) level as follows:

#### Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| … |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |
| } |  |

Note that the current transform signaling design requires all coefficients from all components to be decoded first, and then the lfnst_idx and mts_idx syntax elements are parsed. This can introduce excessive latency  in certain common decoder pipelines (e.g., in LFNST) as the inverse transformation process cannot start before decoding coefficients from all components especially when LFNST is applied for chroma components.

# Proposed Method

In order to reduce the latency in transformation process, this document simply to aims to:

- move signaling of the transform syntax elements, lfnst_idx and mts_idx, to TU-level after coefficient coding of luma TBs so that they are signalled after all necessary luma coefficients are parsed. Accordingly, if ISP is used, the lfnst_idx is signaled rightafter luma coefficients from all TUs are parsed (i.e., lfnst_index is signaled with the last TU obtained via ISP).

- remove LFNST for chroma components completely, as LFNST for chroma is only enabled in dual-tree/separate-tree partition mode.

Note that removing LFNST for chroma in separate-tree partitioning mode further reduce complexity and provides a harmonized design with the existing VVC design.

Specifically, the proposed changes can be reflected on top of the VVC specification working draft (JVET-Q2001-vE) as follows, where (i) changes highlighted in green show the changes related to moving syntax elements from CU-level to TU-level, and (ii) changes highlighted in yellow indicate the additional changes needed (e.g., ones related to handle the case where ISP is used):

#### Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ) { |  |
| if( treeType != DUAL_TREE_CHROMA && ( ( !( cbWidth = = 4 && cbHeight = = 4 ) && modeType != MODE_TYPE_INTRA ) ¦ ¦ ( sps_ibc_enabled_flag && cbWidth <= 64 && cbHeight <= 64 ) ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| … |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsDcOnly = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| ISPTuFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| lfnstNotTsFlag = ( treeType = = DUAL_TREE_CHROMA ¦ ¦ transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 ) && ( treeType = = DUAL_TREE_LUMA ¦ ¦ ( transform_skip_flag[ x0 ][ y0 ][ 1 ] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0 ) ) |  |

| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| --- | --- |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |
| } |  |

#### Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| if( IntraSubPartitionsSplitType != ISP_NO_SPLIT && treeType = = SINGLE_TREE && subTuIndex = = NumIntraSubPartitions − 1 ) { |  |
| xC = CbPosX[ chType ][ x0 ][ y0 ] |  |
| yC = CbPosY[ chType ][ x0 ][ y0 ] |  |
| wC = CbWidth[ chType ][ x0 ][ y0 ] / SubWidthC |  |
| hC = CbHeight[ chType ][ x0 ][ y0 ] / SubHeightC |  |
| } else { |  |
| xC = x0 |  |
| yC = y0 |  |
| wC = tbWidth / SubWidthC |  |
| hC = tbHeight / SubHeightC |  |
| } |  |
| … |  |
| if( ( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_CHROMA ) && ChromaArrayType != 0 && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex = = NumIntraSubPartitions − 1 ) ) ) { |  |
| tu_cb_coded_flag[ xC ][ yC ] | ae(v) |
| tu_cr_coded_flag[ xC ][ yC ] | ae(v) |
| } |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT && !( cu_sbt_flag && ( ( subTuIndex = = 0 && cu_sbt_pos_flag ) ¦ ¦ ( subTuIndex = = 1 && !cu_sbt_pos_flag ) ) ) && ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ ( chromaAvailable && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) ¦ ¦ CbWidth[ chType ][ x0 ][ y0 ] > MaxTbSizeY ¦ ¦ CbHeight[ chType ][ x0 ][ y0 ] > MaxTbSizeY ) ) ¦ ¦ ( IntraSubPartitionsSplitType != ISP_NO_SPLIT && ( subTuIndex < NumIntraSubPartitions − 1 ¦ ¦ !InferTuCbfLuma ) ) ) |  |
| tu_y_coded_flag[ x0 ][ y0 ] | ae(v) |
| if(IntraSubPartitionsSplitType != ISP_NO_SPLIT ) |  |
| InferTuCbfLuma = InferTuCbfLuma && !tu_y_coded_flag[ x0 ][ y0 ] |  |
| } |  |
| if (IntraSubPartitionsSplitType != ISP_NO_SPLIT && subTuIndex = = NumIntraSubPartitions − 1) |  |
| ISPTuFlag = 0 |  |
| … |  |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cb_coded_flag[ xC ][ yC ] ¦ ¦ tu_cr_coded_flag[ xC ][ yC ] ) ) ¦ ¦ ( tu_cb_coded_flag[ xC ][ yC ] && tu_cr_coded_flag[ xC ][ yC ] ) ) && chromaAvailable ) |  |
| tu_joint_cbcr_residual_flag[ xC ][ yC ] | ae(v) |
| if( tu_y_coded_flag[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| lfnstWidth = tbWidth ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = tbHeight ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if(treeType != DUAL_TREE_CHROMA ) { |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 lfnstNotTsFlag = = 1 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max(CbWidth[ chType ][ x0 ][ y0 ]cbWidth, CbHeight[ chType ][ x0 ][ y0 ]cbHeight ) <= MaxTbSizeY) && (IntraSubPartitionsSplitType == ISP_NO_SPLIT ¦ ¦ ISPTuFlag = = 0) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max(CbWidth[ chType ][ x0 ][ y0 ]cbWidth, CbHeight[ chType ][ x0 ][ y0 ]cbHeight) <= 32 && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && MtsDcOnly = = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |
| if( tu_cb_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cr_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cb_coded_flag[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ¦ ¦ slice_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

# Experimental Results

The proposed method is implemented on top of VTM-8.0 and evaluated under common test conditions (CTC).

Table 1 shows the results of proposed combination without any additional encoder changes.

Table 2 presents the results of proposed normative changes with the following encoder modifications:

- The maximum “TT” partition for chroma I-slices is set to 64 (i.e., MaxTTChromaISlice=64).

- The g_aucIntraModeNumFast_UseMPM_2D table is modified as:

const uint8_t g_aucIntraModeNumFast_UseMPM_2D[7 - MIN_CU_LOG2 + 1][7 - MIN_CU_LOG2 + 1] =

{

{5, 5, 4, 3, 2, 2},  //   4x4,   4x8,   4x16,   4x32,   4x64,   4x128,

{5, 4, 4, 3, 3, 2},  //   8x4,   8x8,   8x16,   8x32,   8x64,   8x128,

{4, 4, 4, 3, 3, 2},  //  16x4,  16x8,  16x16,  16x32,  16x64,  16x128,

{3, 3, 3, 3, 3, 2},  //  32x4,  32x8,  32x16,  32x32,  32x64,  32x128,

{2, 3, 3, 3, 3, 2},  //  64x4,  64x8,  64x16,  64x32,  64x64,  64x128,

{2, 2, 2, 2, 2, 3},  // 128x4, 128x8, 128x16, 128x32, 128x64, 128x128,

};

Table 1.  Results of the proposed method over VTM-8.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 1.56% | 1.97% | 95% | 100% |
| Class A2 | -0.04% | 1.58% | 1.60% | 91% | 99% |
| Class B | 0.12% | 1.05% | 1.47% | 96% | 102% |
| Class C | 0.25% | 0.66% | 0.92% | 92% | 99% |
| Class E | 0.22% | 1.08% | 0.98% | 94% | 99% |
| Overall | 0.12% | 1.14% | 1.37% | 94% | 100% |
| Class D | 0.26% | 0.86% | 0.98% | 94% | 103% |
| Class F | 0.23% | 0.54% | 0.49% | 97% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.48% | 0.52% | 94% | 94% |
| Class A2 | 0.02% | 0.34% | 0.61% | 94% | 95% |
| Class B | 0.05% | 0.61% | 0.81% | 98% | 96% |
| Class C | 0.16% | -0.34% | -0.45% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.28% | 0.37% | 96% | 95% |
| Class D | 0.18% | 0.61% | 0.66% | 100% | 101% |
| Class F | 0.23% | -0.16% | 0.17% | 100% | 100% |
|  | Low Delay B |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 95% | 82% |
| Class E | 0.00% | 0.00% | 0.00% | 95% | 81% |
| Overall | 0.00% | 0.00% | 0.00% | 97% | 89% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 101% | 100% |

Table 2.  Results of the proposed method with additional encoder only changes over VTM-8.0 under CTC.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 1.38% | 1.82% | 101% | 98% |
| Class A2 | -0.13% | 1.10% | 1.19% | 99% | 96% |
| Class B | 0.03% | 0.70% | 1.01% | 104% | 99% |
| Class C | 0.12% | 0.54% | 0.74% | 101% | 93% |
| Class E | 0.04% | 0.52% | 0.38% | 101% | 93% |
| Overall | 0.02% | 0.81% | 1.01% | 101% | 96% |
| Class D | 0.10% | 0.68% | 0.81% | 102% | 100% |
| Class F | 0.06% | 0.24% | 0.27% | 101% | 96% |
|  | Random Access |  |  |  |  |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.38% | 0.48% | 100% | 100% |
| Class A2 | -0.03% | 0.34% | 0.49% | 96% | 95% |
| Class B | 0.01% | 0.31% | 0.55% | 101% | 100% |
| Class C | 0.11% | -0.38% | -0.52% | 98% | 92% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.15% | 0.24% | 99% | 97% |
| Class D | 0.17% | 0.47% | 0.37% | 102% | 101% |
| Class F | 0.12% | -0.45% | 0.00% | 102% | 102% |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.00% | 0.04% | 100% | 102% |
| Class C | -0.07% | -0.44% | -0.15% | 97% | 94% |
| Class E | -0.15% | -0.19% | 0.00% | 97% | 91% |
| Overall | -0.07% | -0.19% | -0.04% | 99% | 97% |
| Class D | -0.01% | -0.70% | -0.22% | 100% | 101% |
| Class F | -0.01% | 0.27% | -0.03% | 101% | 98% |

# Conclusions

This proposes to move MTS and LFNST index syntax elements to TU-level, and signaling is done right after necessary coefficient information from luma components is parsed. The LFNST process for chroma components are also disabled. The proposed method reduces the latency and computational complexity, which is especially critical for the two-stage LFSNT process. Since the proposed changes provide a good trade-off between complexity and coding efficiency, it is recommended to adopt the proposed method in VVC.

The file “JVET-Q2001-vE_JVET-R0358-spec.docx” attached to this submission includes the proposed changes over the VVC specification working draft (JVET-Q2001-vE).

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 8),” JVET-Q2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, M. Coban, V. Seregin, M. Karczewicz, “Chroma LFNST Simplification and Signaling,” JVET-Q0686, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, A.K. Ramasubramonian, M. Coban, V. Seregin, M. Karczewicz, T. Hashimoto, T. Chujoh, E. Sasaki, T. Ikai, “Combination of JVET-Q0106, JVET-0686 and JVET-Q0133 for LFNST Signaling, Latency Reduction and Scaling Process,” JVET-Q0784, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 Jan. 2020.

- H.E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, M. Karczewicz, “Removal of redundant LFNST index signalling,” JVET-R0234, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 18th Meeting, 15–24 April 2020.

- H.E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz, “Removal of LFNST for chroma components,” JVET-R0235, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 18th Meeting, 15–24 April 2020.

- H.E. Egilmez, A. Nalci, V. Seregin, W.-J. Chien, T. Hsieh, M. Karczewicz, “Latency reduction in transformation process with TU-level signalling,” JVET-R0236, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 18th Meeting, 15–24 April 2020.

- T.-D. Chuang, M.-S. Chiang, Z.-Y. Lin, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “LFNST complexity reduction,” JVET-R0056, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 18th Meeting, 15–24 April 2020.

- X. Xiu, T.-C. Ma, Y.-W. Chen, H.-J. Jhu, X. Wang, “Cleanup of LFNST signalling in single tree”, JVET-R0331, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 18th Meeting, 15–24 April 2020.

- C. Rosewarne, J. Gan, “On Chroma LFNST”, JVET-R0176, ISO/IEC JTC 1/SC 29/WG 11 Joint Video  Exploration Team (JVET) 18th Meeting, 15–24 April 2020.

# Patent rights declarations

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).